SA WG2 Meeting #124	S2-178359

Nov 27 – Dec 1, 2017, Reno, NV, USA	(revision of S2-178165)

Source:	China Mobile, Deutsche Telekom, Verizon

Title:	23.502: An alternative solution on discovering the PCF requested by AF

Document for:	Approval

Agenda Item:	6.5.6

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution:  This contribution proposes an alternative solution procedure on discovering the PCF serving for a PDU session when AF requests with the UE's IP address.

- Introduction

Last meeting, S2-178165 is postponed based on the last meeting discussion. This proposal will re-submit the procedure of S2-178165 in consistent with the idea of discussion paper S2-177477.

Besides, this proposal adds a new service provided by SMF to support the above procedure.

- Conclusion and Proposal

It is proposed to adopt the changes in TS 23.502.

* * * First Change * * *

4.3.6	Application Function influence on traffic routing

4.3.6.1	General

This clause describes the procedures between an Application Function and the SMF to maintain an efficient user plane path for Application Functions that require it.

As described in TS 23.501 [2] clause 5.6.7, an Application Function may send requests to influence SMF routeing decisions for User Plane traffic of PDU Sessions.  The AF requests may influence UPF (re)selection and allow routeing of user traffic to a local access (identified by a DNAI) to a Data Network . The AF may also provide in its request subscriptions to SMF events.

4.3.6.2	Processing AF requests to influence traffic routeing

Figure 4.3.6.2-1: Processing AF requests to influence traffic routeing

NOTE 1:	The 5GC functions used in this scenario are assumed to all belong to the same PLMN (HPLMN in non-roaming case or VPLMN in case of a PDU Session in LBO mode).

NOTE 2:	AF requests received from an AF located in the HPLMN for local breakout and home routed roaming scenarios are not supported.

1.	The AF creates an AF Traffic steering request. The content of this request is defined in 23.501 [2] clause 5.6.7. The request contains also an AF Transaction Id. In case it subscribes to events related with PDU Sessions the AF indicates also where it desires to receive the corresponding notifications (AF notification reporting information).

2.	Depending on the deployment, the AF may send its request via the NEF or supports following functionalities assigned to the NEF in this procedure: Information mapping and transfer of the content of the AF request to the PCF.

2a:	If the request targets an IP UE address (i.e. a specific ongoing session), the AF may send its request directly to the PCF or via the NEF.

2b:	When the request is not for a specific ongoing session, the AF sends its request via NEF.

When required the NEF authenticates the AF and authorizes the AF request.

The NEF may ensure the necessary mapping from the information provided by the AF into information needed by the 5GC, e.g. mapping from AF-Service-Identifier into:

-	a target DNN and slicing information (S-NSSAI).

-	a list of DNAI(s) and routing profile ID(s).

NOTE 2:	The NEF can only derive a list of DNAI(s) from the AF-Service-Identifier when the DNAI(s) being used by the applications are statically defined. When the DNAI(s) where applications are instantiated may vary dynamically then the AF should explicitly provide DNAI(s) and N6 routing information in its request.

The NEF stores AF notification reporting information received from the AF and maps it to NEF notification reporting information intended for PCF(s).

3.	The AF request has to be sent to the relevant PCF(s).

3a	When the AF request targets an individual UE identified by its IP address, one single PCF needs to be involved i.e. the PCF that serves the corresponding PDU Session. The selection of the PCF(s) that will receive the AF request is done by the NEF when a NEF is involved or by the AF itself otherwise. The selection of the PCF is described in TS 23.501 [2], clause 6.3.

Editors' Note: it is FFS whether the AF Request may correspond to a list IP addresses / IPv6 Prefixed of on-going PDU Sessions. If that were the case the request would have to be forked to multiple PCF.

3b	When the AF request targets multiple UE (i.e. a group of UE or all UE having a PDU Session associated with the target DNN and S-NSSAI) it may be needed to involve multiple PCF(s). The way for the NEF to provide the content of AF request to all potentially impacted PCF(s) is described in TS 23.501 [2], clause 6.3

4.	The PCF (case 3a) / NEF (case 3b) generates an AF Traffic steering response.

5.	When the AF request is considered as semi-permanent (i.e. does not target an individual IP address / Prefix allocated to an UE) the PCF stores the AF request for usage by future PDU Sessions.

6.	The PCF determines if existing PDU Sessions are impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service.

7.	When a PCC rule received from the PCF requires it, the SMF may take appropriate actions to reconfigure the User plane of the PDU Session such as:

-	Inserting (and configuring) a new UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.2

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies)

-	Updating the UPF in the target DNAI with new traffic steering rules

-	Notify the application when a trigger notification has been met. This is further detailed in clause 4.3.6.3.

4.3.6.3	Notification of User Plane Management Events

The SMF may send a notification to the AF if the AF had subscribed to user plane management event notifications as described in clause 4.3.6.2 and in TS 23.501 [2] clause 5.6.7. The following are the examples of such events:

-	A PDU Session anchor identified in the AF subscription request has been established or released.

-	A DNAI has changed.

-	The SMF has received a request for AF notification and the on-going PDU Session meets the conditions to notify the AF.

The SMF uses notification reporting information received from PCF to issue the notification either via an NEF or directly to the AF.

The following flow depicts the sequence of events:

Figure 4.3.6.3-1: Notification of user plane management event

1.	A condition for an AF notification has been met as described above.

2. In case of early notification requested by the AF, the SMF notifies the intended AF of the target DNAI of the PDU Session.

3.	The SMF enforces the change of DNAI or addition, relocation, or removal of a UPF.

4.	In case of late notification requested by the AF, the SMF notifies the AF of the selected target DNAI of the PDU Session.

4.3.6.4	NEF Transferring an AF request to the relevant PCF

Figure 4.3.6.3-2: Transferring an AF request to the relevant PCF

NOTE :	The procedure is related to TS 23.501 [2] clause 6.3.7.3.

1.	NEF receives an AF request with an IP address, and finds out the address of the SMF having the IP address in its IP address pool for UE IP address assignment.

2.	NEF sends to SMF a query request for the address of the relevant PCF with the IP address.

3.	SMF finds out the address of the relevant PCF serving for the PDU session(s) of UE with the IP address.

4.	SMF responds the address of the relevant PCF to NEF.

5.	NEF transfers the AF request to the relevant PCF.

5.2.8	SMF services

5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF |
|  | Update | Request/Response | V-SMF, H-SMF |
|  | Release | Request/Response | H-SMF, V-SMF |
|  | Create SM context | Request/Response | AMF |
|  | Update SM context | Request/Response | AMF |
|  | Release SM context | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF |
| Nsmf_EventExposure | Subscribe for one UE, group of UE(s), any UE | Subscribe/Notify | PCF, NEF, AMF |
|  | UnSubscribe for one UE, group of UE(s), any UE |  | PCF, NEF, AMF |
|  | Notify |  | PCF, NEF, AMF |
| Nsmf_ContextQuery | PCF Query | Request/Response | NEF |

5.2.8.4	Nsmf_ContextQuery

5.2.8.4.1	General

Service description: This service allows a network function to query the context information of PDU session in SMF. The following are the key functionalities of this NF service.

-	Allow consumer NFs to request the query of context information of PDU session in SMF; and

-	Providing the requested context information on a PDU session in SMF to consumer NFs.

5.2.8.4.2	Nsmf_ContextQuery_PCF Query service operation

Service operation name: Nsmf_ContextQuery_PCF Query

Description: Query the address of PCF serving for a PDU session in SMF.

This service operation is invoked by NEF for querying the address of PCF to SMF.

Input, Required: UE IP address.

Input, Optional: None.

Output, Required:PCF address.

Output, Optional: None.

See step clause 4.3.6.4 for details on the usage of this service operation.

* * * End of Change * * *