SA WG2 Meeting #129-bis	S2-1812390

26 - 30 November 2018, West Palm Beach, USA

Source:	NTT DOCOMO

Title:	Solution #4 update

Document for:	Approval

Agenda Item:	6.15.3

Work Item / Release:	FS_Vertical LAN / Rel-16

Abstract of the contribution: This contribution proposes to update Solution #4 in TR 23.734.

1.	Discussion

Some of solution candidates discuss the secondary authentication/authorization aspect as a part of solution, while FS_eNS discusses that aspect more extensively. We might need to pay attention to their conclusion as well.

For this particular solution #4, seemingly it does not matter whether AMF and "the non-public network's UDM" reside in the non-public network or in the public PLMN.

2.	Proposal

It is proposed to include the below changes in TR 23.734.

## ---Start of the 1st Change---

### 6.4.1	Description

To address the key issue #2 ("Network Identification for non-public networks"), the solution is based on the following principles:

-	A non-public network is uniquely identified by the combination of a PLMN ID and a non-public network ID.

-	For non-public networks isolated from PLMN, a dedicated PLMN ID is reserved unique globally or within a country(e.g. the PLMN ID consists of an MCC and a dedicated MNC value which a local regulation of the MCC may assign with the non-public network isolated from PLMN).

Editor's note:	Whether it can be assumed that PLMN ID will be assigned for non-public network isolated from PLMN)by regulatory for each country is FFS.

In a given region, per policy of the local regulation in the MCC, the non-public network IDs may be centrally managed by the local regulation or assigned in un-managed manner by individual network operators.

-	A non-public network is distinguishable from public network by the non-public network ID which is assigned in addition to PLMN ID. It is distinguishable by the PLMN ID whether the non-public network is isolated from PLMN or not.

To address the key issue #1 ("Network discovery & selection and access for non-public networks "), the solution is based on the following principles:

-	NG-RAN nodes supporting access to non-public networks broadcast in SIB one or more tuples of PLMN ID and NPN ID to indicate to UEs the non-public networks they support.

-	In automatic network selection mode when a UE discovers available networks, the UE would select the highest priority available network out of the preferred network selection list, which indicates the preference on UE's network selection among public networks and non-public networks, which RAT type can be considered together.  How to define or use the preferred network selection list will be decided by CT1 WG.

Editor's note: It is FFS whether or how to handle the case that PLMN which manages the NPN is different from the PLMN which manages the UE  subscription/credential.

-	The UEs only which subscribe to a non-public network are authorized to register the non-public network, so that such UEs are configured with their UE identity (e.g. SUPI, or NAI) and credentials for the subscribing non-public network.

-	Depending on whether the non-public network is accessible to the public PLMN that manages the UE credentials (i.e. AMF in the non-public network can access UDM/AUSF in the the public PLMN. Or AMF resides in the public PLMN), UE identity and its corresponding credential for the non-public network are determined as follows:

-	For the UEs in the non-public network which is accessible to the public PLMN that manages the UE credentials:

-	UE ID (i.e. SUPI) which is assigned by a public PLMN is used also in the non-public network, where the corresponding credential for the public PLMN is used also for the non-public network as well as the public PLMN. And UE authentication can be performed by the public PLMN's AUSF/UDM.

-	UE subscription data for the non-public network is stored in the non-public network's UDM (This UDM can reside in the public PLMN). So UE authorization for non-public network is performed by the non-public network's UDM.

NOTE:	This secondary authorization aspect can be enhanced following the conclusion of FS_eNS.

-	For UEs in the non-public network which is not accessible to the publicPLMN that manages the UE credentials or not managed by any public PLMN(e.g. in the non-public network isolated from PLMN):

-	UE ID (i.e. NAI) is assigned in the non-public network, where the realm part of the NAI should include non-public network identification (e.g. NPN-dedicated PLMN ID+NPN ID).

-	UE subscription data for the non-public network is stored in the non-public network's UDM only. So UE authentication and authorization is performed by the AUSF/UDM in the non-public network.

-	UE accessing the non-public network includes the non-public network Identification(e.g. PLMN ID+NPN ID) in the RRC message to indicate that the UE intends to access the non-public network.

-	UE accessing the non-public network need indicate to the AMF that the UE intends to access the non-public network (e.g. by including the non-public network Identification in the registration request message ).

-	On receiving the registration request message, the AMF may perform for UE authentication in the non-public network:

-	UE authentication procedure in the public PLMN based on the SUPI or SUCI if the public PLMN is accessible via the non-public network (i.e. if UE is authenticated by the public PLMN, it is considered as authenticated in the non-public network also); and

-	UE authentication procedure in the non-public network exclusively from PLMN if the non-public network is not accessible to the public PLMN that manages the UE credentials or not managed by any public PLMN.

## ---Start of the 2nd Change---

### 6.4.2	Procedures

Editor's note:	This clause describes services and related procedures for the solution.

UE Registration procedure to the non-public network which is accessible to the public PLMN that manages the UE credentials:

Figure 6.4.2-1: UE Registration procedure to non-public network which is accessible to the public PLMN

NOTE:	The boundary between "Non-public NW" and "public PLMN" in the above is depicted as an example.

0.	A new SIB provided by 5G-RAN indicates non-public network(s) which it supports (i.e. combination of NPN ID and PLMN ID is broadcast in the SIB). UE discovers and select the non-public network which the UE has subscription to.

1.	UE to RAN: AN message (AN parameters, Registration Request (Registration type, SUCI or 5G-GUTI, etc), where the AN parameters include the selected NPN ID.

2.	The RAN selects an AMF which is in the non-public network as described in TS 23.501 [3], clause 6.3.5.

3.	RAN to AMF: N2 message (N2 parameters, Registration Request), where the N2 parameters include the selected NPN ID.

4.	[conditional] the new AMF retrieves the UE context from the old AMF which the UE indicates by 5G-GUTI included in the Registration Request.

5.	The AMF may decide to initiate UE authentication for the non-public network by invoking an AUSF. In that case, the AMF selects an AUSF in the hPLMN if the UE Identity in the Registration Request message from the UE is SUPI or SUCI or 5G-GUTI.

6.	If authentication is required, the AMF requests it from the AUSF, which is in the hPLMN. NAS/AS Security contexts are shared between UE and non-public network.

7.	The AMF selects a UDM to retrive the UE subscription data for the non-public network and register UE context, based on the SUPI and NPN ID.

8.	The AMF registers with the UDM, which is selected for the non-public network in step 7, using Nudm_UECM_Registration and subscribes to be notified when the UDM deregisters this AMF, and retrieves the Access and Mobility Subscription data, SMF Selection Subscription data and UE context in SMF data using Nudm_SDM_Get.

9.	Based on the subscription data for the non-public network, for the authorized UE the AMF sends a Registration Accept message to the UE indicating that the Registration Request has been accepted.

10.	The UE completes the Registration procedure.

## ---End of the Changes---