SA WG2 Meeting #129	S2-1810805

Dongguan, China, 15-19 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0656 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network controlled NSSAI for SR-related Access Stratum connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NEC, Telecom Italia, Huawei, InterDigital, AT&T, Verizon, Sprint, Orange, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-10-9 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in tdoc S2-1810390 we need to conclude on the topic of what NSSAI value to include in the RRC connection establishment when this is triggered by a SR (whithout this, the Specification is incomplete). This CR proposes a way forward in alignment with S2-1810390 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aa PLMN can instruct inform the UE on which behaviour to adopt when triggering a Service Request procedure, in a Registration Accept Message by including Access Stratum Connection Establishment NSSAI Inclusion Mode information in a Registration Accept Message that instructs the UE to: 1.- Include in Access Sstratum Connection Establishment a NSSAI = Aallowed NSSAI for the UE, or 2. Include in Access Stratum Connection Establishment only the S-NSSAIs of the Network Slices that trigger the RRC Connection Establishment, or 3. Include no NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined UE behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | If SA3 deems that it is a security issue to send S-NSSAI in cleartext in AS signaling and to send encrypted S-NSSAI in the NAS signaling, then it is an option to remove S-NSSAI from NAS signaling, instead RAN can send S-NSSAI in the N2 signaling towards AMF. |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.15.x	Operator-controlled inclusion of NSSAI in Access Stratum Connection Establishment

The operator controls the NSSAI the UE shall include in the Access Stratum Connection Establishment driven by a Service Request or a Registration Request not caused by Mobility (i.e. not caused by the UE entering a Tracking Area not included in the current Registration Area).

During the Registration procedure, the AMF shall provide the UE in the Registration Accept message, an Access Stratum Connection Establishment NSSAI Inclusion Mode parameter indicating whether the UE shall include in the Access Stratum Connection Establishment (e.g. a RRC connection Establishment defined in TS 38.331)

- NSSAI set to the Allowed NSSAI for the UE for both Registration Request not due to mobility and Service request triggered Connection Establishment

b)	For the case of  Access Stratum Connection Establishment caused by a Service Request: a NSSAI including the S-NSSAI(s) of the Network Slice(s) that trigger the Connection Establishment and, for the Registration Request not due to mobility, a NSSAI set to the Allowed NSSAI ; or

c)	for both Registration Request not due to mobility and Service Request triggered Connection Establishment no NSSAI.

The UE shall include a NSSAI in the Access Stratum based on the received Access Stratum Connection Establishment NSSAI Inclusion Mode.

The AMF shall by default indicate to the UE to operate in mode c) unless the operator requests different indication to be provided to the UE.

NOTE: 	The indication to the UE is based on the policies implemented by the operator

**** END OF CHANGES ****