# SA WG2 Meeting #S2-129	S2-1811351

# 15 - 19 October, 2018, Dong Guan, China

Source:	CATT

Title:	Interim Conclusions on KI#7

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	FS_eLCS / Rel-16

Abstract of the contribution: This paper analyzes solutions for KI#7and proposes interim conclusions.

Discussion:

- Currently we/SA2 still waiting response from SA1 and RAN2 on LS OUT (S2-185792 LS OUT on Location Service exposure to NG-RAN).

  - RAN2 replied in S2-1810028/ R2-1813425.” So far, RAN2 has not identified specific use cases for RAN knowing UE’s location.  RAN2 will study this issue as part of the Rel-16 SI.”

- For solutions discuss exposure within 5G CN, to AF outside of CN, to NG-RAN, table 1 capture initial evaluations, and comparison, based on following aspects:

  - Upon the receipt of location request from external AF, which NF that the NEF will forward the location request, and:

  - How this NF is select by NEF?

  - The interface aspects of the NEF,

  - How to support internal LCS client, e.g. location consumer inside 5G CN ?

  - Location service supported e.g. Deferred triggered, periodic, one time locations

  - Other aspects, and impacted NFs;

- Based on current TR23.731V060, it is possible to evaluate and reach some interim conclusions during SA2#129:

  - For solutions address LCS exposure to AF outside of HPLMN, following aspect should be discussed :

    - For exposure location service to AF outside of HPLMN, NEF should provide new APIs to the AF; and related services and service operation need be provided on Nnef interface;

    - The NEF should forward receive LCS request from AF to GMLC, to progress location service; procedure defined in 6.9. or 6.23 (GMLC based) could be used as basis;

    - LMF should be used as serving NFs to handle LDR type and/or periodical request, and procedure defined in 6.9.3.3 could be used as basis;

  - For other aspect, deferred to next meeting:

    - Exposure location service to NF located in 5G-CN;

    - Exposure location service to NF located in 5NG-RAN;

|  | Solution 9 | Solution 10 | Solution 14 | Solution 23 | Solution 11 |
| --- | --- | --- | --- | --- | --- |
| Which NF that NEF forwards the received LCS request from external | GMLC | LMF | GMLC | - HGMLC or - AMF | Does not apply |
| How such NF is selected? | using the NRF service to find first GMLC, and in roaming case, HGMCL finds VGMLC by contacting UDM; | Query UDM for serving LMF, if non, use NRF service to get one; | Not Known, Le interface is used between NEF and GMLC; | - Not mentioned? - Query UDM to get serving AMF | Does not apply |
| Inter faces aspects of NEF | North bound: Not mentioned; | NEF need support the corresponding service interface | an SBI to NEF for external client (?) | SBI interface to NF, and new APIs to AFs | Does not apply |
| Internal exposure to NF within 5G-CN | NF send request to GMLC; GMLC provide service based interface for location request | Solution itself does not differentiate external and internal exposure | ? not clear ? an SBI interface (to the NEF for external client or directly to a GMLC in case of authorized clients); | via NEF, then NEF forward LCS request to GMLC; | Does not apply |
| Internal exposure to NG-RAN | Not mentioned | Not mentioned | Not mentioned | Not mentioned | For target UE in connect mode, the NG-RAN send Location request to AMF via N2 |
| Location Request type supported | Deferred Location supported, related serving NF activate and de-act the trigger, detect the event of the trigger. Serving NF not specified in CN, UE could be one. | Includes LDR and LIR; Request, update, and cancel LDR request procedure defined. LMF will handle such location request. | Rel-15 defined procedure supported. How to support “Deferred Location request” not clearly specified. | Including a request for triggered or periodic location. | 5GC NI LR used; |
| Privacy checking | GMLC perform | serving LMF performs | HGMLC interacts with UDM to perform | - HGMLC interact with UDM - NEF interact with UDM | Not mentioned. |
| Other aspects | Target UE located in EPC considered. | UE provides the expected location privacy provisioning to the network(LMF) proposed. | LMF selection criterias defined | Enhanced NG-RAN Location Reporting Procedure proposed | Only UE in connect mode, such solution works. |
| Other key issues addressed | Key Issue 3: "Support of low latency LCS" | Key issue 4 "Reduce overhead for repetitive non-successful privacy verification". | Key Issue 1 (Enhancement to LCS Architecture) |  | Key Issue 1 "Enhancements to LCS architecture" |
| Impacted NFs | NEF,GMLC,UDM | NEF,UDM,LMF | AMF,LMF,GMLC,UDM,CHF | AMF,UDM,NEF,NG-RAN | NG-RAN,AMF,UDM, |

Proposal

Proposal:

It is proposed to agree the following changes to TR 23.731.

*** Start of 1st change ***all new text in 1st change

8.x	Interim agreement for Key Issue #7

For Key Issue 7: Location service exposure, following interim agreements are reached:

-	Interim agreement 1: For exposure location service to AF outside of HPLMN, NEF should provide new APIs to the AF; and related services and service operation need be provided on Nnef interface;

*** End of 1st change ***

*** Start of 2nd change ***all new text in 2nd change

7.C	Evaluation of Solutions for Key Issue #7

# Table 7.C.-X: Analysis of solutions for Key Issue #7

|  | Solution 9 | Solution 10 | Solution 14 | Solution 23 | Solution 11 |
| --- | --- | --- | --- | --- | --- |
| Which NF that NEF forwards the received LCS request from external | GMLC | LMF | GMLC | - HGMLC or - AMF | Does not apply |
| How such NF is selected? | using the NRF service to find first GMLC, and in roaming case, HGMCL finds VGMLC by contacting UDM; | Query UDM for serving LMF, if non, use NRF service to get one; | Not Known, Le interface is used between NEF and GMLC; | - Not mentioned? - Query UDM to get serving AMF | Does not apply |
| Inter faces aspects of NEF | North bound: Not mentioned; | NEF need support the corresponding service interface | an SBI to NEF for external client (?) | SBI interface to NF, and new APIs to AFs | Does not apply |
| Internal exposure to NF within 5G-CN | NF send request to GMLC; GMLC provide service based interface for location request | Solution itself does not differentiate external and internal exposure | ? not clear ? an SBI interface (to the NEF for external client or directly to a GMLC in case of authorized clients); | via NEF, then NEF forward LCS request to GMLC; | Does not apply |
| Internal exposure to NG-RAN | Not mentioned | Not mentioned | Not mentioned | Not mentioned | For target UE in connect mode, the NG-RAN send Location request to AMF via N2 |
| Location Request type supported | Deferred Location supported, related serving NF activate and de-act the trigger, detect the event of the trigger. Serving NF not specified in CN, UE could be one. | Includes LDR and LIR; Request, update, and cancel LDR request procedure defined. LMF will handle such location request. | Rel-15 defined procedure supported. How to support “Deferred Location request” not clearly specified. | Including a request for triggered or periodic location. | 5GC NI LR used; |
| Privacy checking | GMLC perform | serving LMF performs | HGMLC interacts with UDM to perform | - HGMLC interact with UDM - NEF interact with UDM | Not mentioned. |
| Other aspects | Target UE located in EPC considered. | UE provides the expected location privacy provisioning to the network(LMF) proposed. | LMF selection criterias defined | Enhanced NG-RAN Location Reporting Procedure proposed | Only UE in connect mode, such solution works. |
| Other key issues addressed | Key Issue 3: "Support of low latency LCS" | Key issue 4 "Reduce overhead for repetitive non-successful privacy verification". | Key Issue 1 (Enhancement to LCS Architecture) |  | Key Issue 1 "Enhancements to LCS architecture" |
| Impacted NFs | NEF,GMLC,UDM | NEF,UDM,LMF | AMF,LMF,GMLC,UDM,CHF | AMF,UDM,NEF,NG-RAN | NG-RAN,AMF,UDM, |

*** End of 2nd change