3GPP TSG-SA WG2 Meeting #131	S2-1902004

# 25 February – 1 March, 2019, Santa Cruz - Tenerife, Spain	 		      revision of S2-1900849

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0785 | rev | 3 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating 5.8.2.11 for N4 Rules to support ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is necessary to extend N4 rules including PDRs to support ATSSS. In TR 23.793, it is concluded that the N4 rules are used by the UPF for downlink traffic steering and the N4 is extended as described in solution#14. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Multi-Access Rules based on TR 23.793 solution#14 with the following exceptions: - Instead of Access Forwarding Action Information, this CR adds Per-Access Forwarding Action Information (PAFAI) provided per access type. - This CR adds ‘Priority’ field into PAFAI to support both ‘Priority-based” and ‘Active-Standby” steering modes. - In this revision, ‘Steering method’ is added into clause 5.8.2.11.x to indicate the applicable traffic steering functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UPF cannot support steering access traffic for downlink. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.1, 5.8.2.11.2, 5.8.2.11.3, and 5.8.2.11.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * Start of changes* * * *

5.8.2.11	Parameters for N4 session management

5.8.2.11.1	General

These parameters are used by SMF to control the functionality of the UPF as well as to inform SMF about events occurring at the UPF.

The N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3] will use the relevant parameters in the same way for all N4 reference points: the N4 Session Establishment procedure as well as the N4 Session Modification procedure provide the control parameters to the UPF, the N4 Session Release procedure removes all control parameters related to an N4 session, and the N4 Session Level Reporting procedure informs the SMF about events related to the PDU Session that are detected by the UPF.

The parameters over N4 reference point provided from SMF to UPF comprises an N4 Session ID and may also contain:

-	Packet Detection Rules (PDR) that contain information to classify traffic (PDU(s)) arriving at the UPF;

-	Forwarding Action Rules (FAR) that contain information on whether forwarding, dropping or buffering is to be applied to a traffic identified by PDR(s);

-	Multi-Access Rules (MAR) that contain information on how to handle ATSSS for a Multi-Access PDU Session;

-	Usage Reporting Rules (URR) contains information that defines how traffic identified by PDR(s) shall be accounted as well as how a certain measurement shall be reported;

-	QoS Enforcement Rules (QER), that contain information related to QoS enforcement of traffic identified by PDR(s);

-	Trace Requirements.

The N4 Session ID is assigned by the SMF and uniquely identifies an N4 session.

If the UPF indicated support of Trace, the SMF may activate a trace session during a N4 Session Establishment or a N4 Session Modification procedure. In that case it provides Trace Requirements to the UPF. The SMF may deactivate an on-going trace session using a N4 Session Modification procedure. There shall be at most one trace session activated per N4 Session at a time.

5.8.2.11.2	N4 Session Context

N4 Session Context is identified by an N4 Session ID. An N4 Session Context is generated by SMF and UPF respectively to store the parameters related to an N4 session, including N4 session ID, all PDRs, URRs, QERs and FARs or MARs used for this N4 session.

5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR |  |
| Rule ID |  | Unique identifier to identify this rule |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN" | Combination of UE IP address (together with Network instance, |
| detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length | if necessary), CN tunnel info, packet filter set, application ID, |
| information | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet | Ethernet PDU Session Information and QFI are used for traffic detection. |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID |  |
|  | Packet Filter Set | Details see clause 5.7.6, TS 23.501. | Details like all the combination possibilities on N3, N9 interfaces |
|  | Application ID |  | are left for stage 3 decision. |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI |  |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA-PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. NOTE 2: Either a FAR ID or a MAR ID is included, not both |  |  |  |

5.8.2.11.4	QoS Enforcement Rule

The following table describes the QoS Enforcement Rule (QER) that defines how a packet shall be treated in terms of bit rate limitation and packet marking for QoS purposes. All Packet Detection Rules that refer to the same QER share the same QoS resources, e.g. MFBR.

Table 5.8.2.11.4-1: Attributes within QoS Enforcement Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this QER |  |
| Rule ID | Unique identifier to identify this information. |  |
| QoS Enforcement Rule correlation ID (NOTE 1) | An identity allowing the UP function to correlate multiple Sessions for the same UE and APN. | Is used to correlate QoS Enforcement Rules for APN-AMBR enforcement. |
| Gate status UL/DL | Instructs the UP function to let the flow pass or to block the flow. | Values are: open, close, close after measurement report (for termination action "discard"). |
| Maximum bitrate | The uplink/downlink maximum bitrate to be enforced for the packets. | This field may e.g. contain any one of: - APN-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of all PDN Connections to an APN) (NOTE 1). - Session-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of the PDU Session) - QoS Flow MBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - SDF MBR (for a QER that is referenced by the uplink/downlink Packet Detection Rule of a SDF) - Bearer MBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Guaranteed bitrate | The uplink/downlink guaranteed bitrate authorized for the packets. | This field contains: - QoS Flow GBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - Bearer GBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Averaging window | The time duration over which the Maximum and Guaranteed bitrate shall be calculated. | This is for counting the packets received during the time duration. |
| Down-link flow level marking | Flow level packet marking in the downlink. | For UPF, this is for controlling the setting of the RQI in the encapsulation header as described in clause 5.7.5.3. |
| Packet rate (NOTE 1) | Number of packets per time interval to be enforced. | This field contains any one of: - downlink packet rate for Serving PLMN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections using CIoT EPS Optimizations as described in TS 23.401 [26]). - uplink/downlink packet rate for APN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections to the same APN using CIoT EPS Optimizations as described in TS 23.401 [26]). |
| NOTE 1: This parameter is only used for interworking with EPC. |  |  |

5.8.2.11.5	Usage Reporting Rule

The following table describes the Usage Reporting Rule (URR) that defines how a packet shall be accounted as well as when and how to report the measurements.

Table 5.8.2.11.5-1: Attributes within Usage Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this URR |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting usage. |
| Reporting triggers | One or multiple of the events can be activated for the generation and reporting of the usage report. | Applicable events include: - Start/stop of traffic detection with/without application instance identifier and deduced SDF filter reporting; Deletion of last PDR for a URR; Periodic measurement threshold reached; Volume/Time/Event measurement threshold reached; Immediate report requested; Measurement of incoming UL traffic; Measurement of discarded DL traffic; MAC address reporting in the UL traffic. |
| Periodic measurement threshold | Defines the point in time for sending a periodic report for this URR (e.g. timeofday). | This allows generation of periodic usage report for e.g. offline charging. It can also be used for realizing the Monitoring time of the usage monitoring feature. It can also be used for realizing the Quota-Idle-Timeout, i.e. to enable the CP function to check whether any traffic has passed during this time. |
| Volume measurement threshold | Value in terms of uplink and/or downlink and/or total byte-count when the measurement report is to be generated. |  |
| Time measurement threshold | Value in terms of the time duration (e.g. in seconds) when the measurement report is to be generated. |  |
| Event measurement threshold | Number of events (identified according to a locally configured policy) after which the measurement report is to be generated. |  |
| Inactivity detection time | Defines the period of time after which the time measurement shall stop, if no packets are received. | Timer corresponding to this duration is restarted at the end of each transmitted packet. |
| Event based reporting | Points to a locally configured policy which is identifies event(s) trigger for generating usage report. |  |
| Linked URR ID(s) | Points to one or more other URR ID. | This enables the generation of a combined Usage Report for this and other URRs by triggering their reporting. See clause 5.2.2.4, TS 29.244 [65]. |
| Measurement Method | Indicates the method for measuring the network resources usage, i.e. the data volume, duration, combined volume/duration, or event. |  |
| Measurement information | Indicates specific conditions to be applied for measurements | It is used to request: - measurement before QoS enforcement, and/or - to pause or set to active a measurement as for the Pause of charging described in clause 4.4.4 of TS 23.502 [3], and/or - to request reduced reporting for application start/stop events. |

5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. |  |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included. |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). |  |
| Destination interface (NOTE 3) | Contains the values "access side", "core side", "SMF" or "N6-LAN". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF or, the N6-LAN (i.e. the DN or the local DN). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP+QFI, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, DNAI). Any extension header stored for this packet shall be added. |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. |  |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The TSP ID action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 in TS 29.244 [65]. |  |  |

5.8.2.11.7	Usage Report generated by UPF

The UPF sends the usage report to inform the SMF about the measurement of an active URR or about the detection of application traffic of an active Packet Detection Rule. For each URR, the usage report may be generated repeatedly, i.e. as long as any one of the valid event triggers applies. A final usage report is sent for a URR when it is no longer active, i.e. either the URR is removed or all the references to this URR in any of the Packet Detection Rules belonging to the N4 session.

Following attributes can be included in the usage report:

Table 5.8.2.11.7-1: Attributes within Usage Report

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Uniquely identifies a session. | Identifies the N4 session associated to this Usage Report |
| Rule ID | Uniquely identifies the Packet Detection Rule or Usage Reporting Rule within a session which triggered the report. | Packet Detection Rule is only indicated when Reporting trigger is Detection of 1st DL packet for a QoS Flow or Start/stop of traffic detection. Usage Reporting Rule is indicated for all other Reporting triggers. |
| Reporting trigger | Identifies the trigger for the usage report. | Applicable values are: Detection of 1st DL packet for a QoS Flow; Start/stop of traffic detection with/without application instance identifier and deduced SDF filter reporting; Deletion of last PDR for a URR; Periodic measurement threshold reached; Volume/Time/Event measurement threshold reached; Immediate report requested; Measurement of incoming UL traffic; Measurement of discarded DL traffic; MAC address reporting in the UL traffic. |
| Start time | Provides the timestamp, in terms of absolute time, when the collection of the information provided within Usage-Information is started. | Not sent when Reporting trigger is Start/stop of traffic detection. |
| End time | Provides the timestamp, in terms of absolute time, when the information provided within Usage-Information is generated. | Not sent when Reporting trigger is Start/stop of traffic detection. |
| Measurement information | Defines the measured volume/time/events for this URR. | Details refer to TS 29.244 [65]. |

5.8.2.11.X	Multi-Access Rule

The following table describes the Multi-Access Rule (MAR) that includes the association to the two FARs for both 3GPP access and N3GPP access in case of supporting ATSSS.

Table 5.8.2.11.8-1: Attributes within Multi-Access Rule

| Attribute |  |  | Description |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N4 Session ID |  |  | Identifies the N4 session associated to this PDR. |  |  |  |  |
| Rule ID |  |  | Unique identifier to identify this rule. |  |  |  |  |
| Steering method |  | Indicates the applicable traffic steering functionality: Values “MPTCP functionality”, “ATSSS-LL functionality”, etc |  |
| Steering mode |  |  | Values “Active-Standby”, “Smallest Delay”, “Load Balancing” or “Priority-based”. |  |  |  |  |
| Per-Access Forwarding Action information (NOTE 1) | Forwarding Action Rule ID | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
|  | Weight | Identifies the weight for the FAR in case steering mode is "Load Balancing" | The weights for all FARs need to sum up to 100 |
|  | Priority | Values “Active or Standby” or “High or Low” for the FAR | “Active or Standby” for “Active-Standby” steering mode and “High or Low” for “Priority-based” steering mode |
|  | List of Usage Reporting Rule ID(s) | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. | This enables the SMF to request separate usage reports for different FARs (i.e. different accesses) |
| NOTE 1: The Per-Access Forwarding Action information is provided per access type (i.e. 3GPP access or Non-3GPP access) |  |  |  |

* * * End of Changes * * *