SA WG2 Meeting #131	S2-1902204

25 February – 1 March, Santa Cruz de Tenerife, Spain	(revision of S2-1900xxx)

Source:	Vodafone, Telecom Italia, AT&T, Telstra, Sprint, China Mobile, KDDI, Verizon, Lenovo, Motorola Mobility

Title:	FS_UDICOM - Conclusions

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes conclusion for FS_UDICOM

# 1 Proposal

As closure of UDICOM study approaches, conclusions are deemed necessary. This proposal is to include conclusions for the study.

******************First change***********************

# 8	Conclusions

### 8.1 For EPS and 5GS interworking required for mobility cases from EPS to 5GS and from 5GS to EPS, including the impacts on SMS handling

Solution #2 is considered to progress as normative optional feature, both stage 2 and stage 3 for deployments in which upgrading legacy systems to service based architectures is a feasible/acceptable option for the network operator. A number of open issues listed in clause 7 need to be addressed during normative phase.

CT4 is expected to take the responsibility of the normative specification work while SA2 will add relevant references in affected 5G specifications (TS 23.501, TS 23.502).

Solutions #3 and #6 are feasible alternatives in different scenarios where migration of subscription data or upgrades of legacy systems are not possible/feasible and conclusion is to document them as implementation/deployment options, addressing the open issues described in the relevant evaluation clauses. The format of  the documentation for these solutions is subject to decision in SA2/SA.

### 8.2 For IMS and 5GS interworking required for T-ADS, P-CSCF restoration and NPLI retrieval

Three different scenarios are addressed by the proposed solutions:

- HSS as single point of contact for IMS (diameter based)

- An UDM/HSS combo as single point of contact for IMS (diameter based)

- IMS HSS/SLF as single point of contact for IMS (diameter based)

To facilitate this, conclusion is to p

rogress normative work for new service(s) with new service Request-Response operation(s) produced by UDM: for T-ADS (i.e. IMS over PS support) and NPLI (i.e. time stamp and RAT type), and for P-CSCF restoration. The decision about the number of operations is left for normative work.

The operations can be consumed by HSS (as in solution #2) for scenarios in which HSS is the single point of contact for IMS (diameter based) interactions and adapted to SBA.

The operations can be consumed by an IMS HSS when in stand-alone mode and acting as a single point of contact for a diameter-based IMS. This case would correspond to solution #2 when the HSS referred acts as IMS HSS and is adapted to SBA.

NOTE 1: 	These new operations require coordination with the normative work derived from FS_eIMS5G [14].

NOTE 2: 	Modification of TS 23.228 to clarify the interactions between HSS, UDM and IMS will be coordinated with any normative work arising from FS_eIMS5G [14].

Solutions #3 and #6 are feasible alternatives and concluded to be documented as implementation/deployment option. The format for the documentation of these solutions is subject to decision at SA2/SA.

******************End of changes***********************