3GPP TSG-SA2 Meeting #135	S2-1909805

# Split, Croatia, 14th October 2019 - 18th October 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1855 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consumer/Producer binding for subscribe/notify |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-10-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT4 agreed (see C4-193727) on consumer/producer binding for subscribe/notify procedures: when subscribing, the consumer can include a binding indication to the NF instance or NF set; for NF set, the notifications will be sent to another NF instance in the NF set if the original NF instance is not available. The handling of the callback URI is further described in C4-193727. Moreover, with the LS C4-193792, CT4 requests confirmation from SA2 that it should be possible to provide a binding indication when subscribing to notifications and when sending subsequent notifications, and to update stage 2 accordingly if necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 4.17.12.4 to support a binding indication when subscribing to notifications and when sending subsequent notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch between stage 3 and stage 2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 4.17.12.4	Binding for subscription requests

Binding can be created as part of a subscription request. In this case, illustrated in Figure 4.17.12.4-1, the subscription request may include a binding indication referring to NF service instance, NF instance or NF Set of the NF service consumer as specified in Table 6.3.1.0-1 of TS 23.501 [2]. The NF service producer or SCP shall select the target for the related  notifications using the received binding indication as specified in Table 6.3.1.0-1 of TS 23.501 [2]. If the binding needs to be updated, the NF service consumer may initiate a new request to the NF service producer with an updated Notification Correlation ID and binding indication.

Figure 4.17.12.4-1: Binding to the consumer in a subscription request

Binding can be created as part of an implicit subscription. In this case, the response to the service request that creates an implicit subscription may include a binding indication as in 4.17.12.2. For subsequent operations on the created implicit subscription, bindings may be used as shown in Figure 4.17.12.4.-1.

If the binding needs to be updated, the NF service producer may provide an updated binding indication in the response to a subsequent service request.

Figure 4.17.12.4-2: Binding for implicit subscription

If binding is indicated as part of a service request that determines an implicit subscription, the binding applies also for the notifications from the Producer.

* * * * End of Changes * * * *