3GPP TSG-WG SA2 Meeting #136AH 	S2-2001257

# Incheon, South Korea, January 13 – 17, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1930 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DDD status event with UPF buffering procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the DDD status event subscription, the following EN in 4.15.3.2.5 need to be resolved: A procedure to subscribe to data delivery status notifications if buffering in the UPF is used is ffs. When the UP connection of the PDU Session is deactivated and the SMF decides to activate buffering in UPF for the session, the SMF shall inform the UPF to start buffering packets for this PDU Session. When the UPF receives downlink data, it triggers the network triggerred service request procedure 4.2.3.3. In the procedure, when the SMF is informed the UE is unreacheable by the AMF, the SMF determines Extended Buffering does not apply or apply accordingly. To support the DDD event notification in case of buffering in UPF applies, the following scenarios are needed to be considerred: - When the SMF receives the subscription from UDM, for a UE’s specific PDU session, the SMF has be informed the UE is unreachable and extended buffer is activated. - When the SMF receives the subscription from UDM, for a UE’s specific PDU session, the SMF has not be information the UE is unreachable. When DN is received by the SMF and the SMF is informed by the AMF, the SMF determines t to activate the extended buffer in the UPF. For the above two cases, when the SMF receives the DDD status event subscription, the SMF can further request the UPF forward the new traffic information of buffered data from now on, and when the UPF determines to discard the buffered information, it will aslo forward the traffic information of the discard data. Based on the notification, the SMF can know the DDD status. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify how to subcribe to data delivery status notifications if buffering in the UPF is used. Some mirror correction to the Information flow for downlink data delivery status with SMF buffering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If extended buffer in SMF is not supported, Data delivery status notifications can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.5, 4.15.3.2.a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15.3.2.5	Information flow for downlink data delivery status with SMF buffering

The procedure is used if the SMF requests the UPF to forward downlink data packets that are subject of extended buffering in the SMF. The procedure describes a mechanism for the Application Function to subscribe downlink data delivery status notifications. The downlink data delivery status notifications relates to high latency communication, see also clauses 4.24.2 and 4.2.3.3.

Cancelling the subscription is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID in the same order as indicated in figure 4.15.3.2.5-1 for the corresponding subscribe requests. Step 0 and the notification steps 7 to 9 are not applicable in cancellation case.

Figure 4.15.3.2.5-1: Information flow for downlink data delivery status with SMF buffering

0.	The SMF (in the non-roaming case the H-SMF, in the roaming case the V-SMF, in case of PDU session with I-SMF the I-SMF) configures the relevant UPF to forward downlink data packets towards the SMF as described in clause 5.8.3 in 23.501 [2]. The SMF decides to apply this behavior based on the "expected UE behaviour". Alternatively, step 0 is triggered by step 3,

1.	The AF sends Nnef_EventExposure_Subscribe Request to NEF requesting notification for event "Downlink data delivery status" with traffic descriptor (e.g. the source of the downlink IP or Ethernet traffic) for a UE or group of UEs. If the reporting event subscription is authorized by the NEF, the NEF records the association of the event trigger and the requester identity. The Downlink data delivery status events include:

-	First downlink Packetin extended buffering event:

-	This event is triggered when the first new downlink data packet is buffered with extended buffering matching the traffic descriptor.

-	in notifications about this Downlink data delivery status, the SMF provides the Extended Buffering time as determined in clause 4.2.3.3.

-	First downlink Packet discarded:

-	This event occurs when the first packet matching the traffic descriptor is discarded because the Extended Buffering time, as determined by the SMF, expires or the amount of downlink data to be buffered is exceeded.

-	First Downlink Packet transmitted:

-	This event occurs when the first packet matching the traffic descriptor is transmitted after previous buffering or discarding of corresponding packet(s) because the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE according to clause 4.2.3.3.

2.	The NEF sends the Nudm_EventExposure_Subscribe Request to UDM. Identifier of the UE or group of UEs, the traffic descriptor, monitoring event received from AF in step 1, and notification endpoint of the NEF are included in the message. If the reporting event subscription is authorized by the UDM, the UDM records the association of the event trigger and the requester identity. Otherwise, the UDM continues in step 5 indicating failure.

3.	The UDM sends the Nsmf_EventExposure_Subscribe Request message to each SMF where at least one UE identified in step 2 has a PDU session established; If the UDM is able to derive the applicable DNN and S-NSSAI from the traffic descriptor via configured information, the UDM may send Nsmf_EventExposure_Subscribe Request messages only to SMFs with PDU sessions with that DNN and S-NSSAI for such UEs and includes the Identifier of the UE or Internal-Group-Id, traffic descriptor, monitoring event and the notification endpoint of NEF received in step 2 are included in the message. If the UDM becomes aware that such a UE has a PDU session established with the DNN and S-NSSAI corresponding to the traffic descriptor at a later time than when receiving step 2, the UDM then executes step 3.

In the case of home-routed PDU session or PDU session with I-SMF, the UDM sends the Nsmf_EventExposure_Subscribe Request message to each H-SMF or SMF, and the H-SMF or SMF further sends Nsmf_EventExposure_Subscribe Request message to each related V-SMF or I-SMF. Steps 7-8 are performed by V-SMF or I-SMF.

4.	The SMF sends the Nsmf_EventExposure_Subscribe Response message to the UDM.

5.	The UDM send sends the Nsmf_EventExposure_Subscribe response message to the NEF.

6.	The NEF sends the Nsmf_EventExposure_Subscribe response to the AF.

7.	The SMF detects a change in Downlink Data Delivery Status event as described in clause 4.2.3. The SMF becomes aware that Downlink Packet(s) require extended buffering via a Namf_Communication_N1N2MessageTransfer service operation with the AMF. If the SMF decides to discard packets, the "Downlink Packet(s) discarded event" is detected. The SMF detects that previously buffered packets can be transmitted by the fact that the related PDU session becomes ACTIVE.

8.	The SMF sends the Nsmf_EventExposure_Notify with Downlink Delivery Status event message to NEF.

9.	The NEF sends Nnef_EventExposure_Notify with Downlink Delivery Status event message to AF.

# * * * * Second change * * * *

4.15.3.2.a	Information flow for downlink data delivery status with UPF buffering

The procedure is used if the SMF requests the UPF to buffer packets. The procedure describes a mechanism for the Application Function to subscribe to notifications about downlink data delivery notification. The data delivery status notifications relates to high latency communication, see also subclauses 4.24.2 and 4.2.3.3.

Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID.

Figure 4.15.3.2.a-1: Information flow for downlink data delivery status with UPF buffering

0.	The SMF (in the non-roaming case the H-SMF. in the roaming case the V-SMF, in case of PDU session with I-SMF the I-SMF) configures the relevant UPF to buffer packets as described in clause 5.8.3 in 23.501 [2].

1.	AF interacts with NEF to subscribe DDD status as described in clause 4.15.3.2.5.

2.	If the SMF is informed that the UE is unreachable via a Namf_Communication_N1N2MessageTransfer service operation as described in clause 4.2.3. When the DDDs status event with traffic descriptor is received in the SMF, if extended DL Data buffering in the UPF applies, in addition to notify the UPF buffer the DL data as defined in 4.2.3, the SMF requests the UPF to report the traffic information of the buffered downlink packets and discarded packets in the UPF. The SMF determines the DDD status by comparing the traffic information reported by the UPF with the Traffic Descriptor(s) received in the event subscription(s). The SMF detects that previously buffered packets can be transmitted by the fact that the related PDU session becomes ACTIVE.

3.	The SMF sends the Nsmf_EventExposure_Notify with Downlink Delivery Status event message to NEF.

4.	The NEF sends Nnef_EventExposure_Notify with Downlink Delivery Status event message to AF.

# * * * * End of changes * * * *