3GPP TSG-WG SA2 Meeting #136AH 	S2-2000865

# Incheon, South Korea, January 13 – 17, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0093 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS context in AMF, NEF, GMLC and LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was not yet defined the LCS context stored in LCS involved NF, i.e. AMF, LMF, GMLC and NEF. This will lead to the problem that for certain scenario, e.g. cancellation of the location request, which NF execute the logic (perform a function) and based on what information. If in standard the LCS context is defined for the involved LCS entity, it will be clear on the above issue, at least it will get into the debate one NF cannot do this because of lack of a certain parameter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add LCS context for AMF, GMLC, NEF and LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear for a certain scenario, e.g. cancellation of the location request, which NF execute the logic (perform a function) and based on what information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.3, 4.1.2.4, 5.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.2	GMLC

7.2.1	Information for an LCS Client

The GMLC holds information for external LCS clients which are permitted to request location information for UE subscribers. Table 7.2.1-1 shows the information which may be stored in the GMLC for an external LCS Client.

Table 7.2.1-1: GMLC Information for an External LCS Client

| LCS Client Information | Status | Description |
| --- | --- | --- |
| LCS Client Type | M | Identifies the type of LCS client from among the following: - Emergency Services - Value Added Services - PLMN Operator Services - Lawful Intercept Services |
| External identity | O | A list of one or more identifiers used to identify an external LCS client. The identity may be used for a 5GC-MT-LR and/or 5GC-MO-LR. The format of the identity is an international E.164 address, ITU-T Recommendation E.164 [23]. |
| Authentication data | M | Data employed to authenticate the identity of an external LCS client – details are outside the scope of the present document |
| Internal identity | O | Identifies the sub-type of a PLMN operator services LCS Client from among the following: - LCS client broadcasting location related information - O&M LCS client in the HPLMN - O&M LCS client in the VPLMN - LCS client recording anonymous location information - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE |
| Client name | O | An address string which is associated with the LCS client's external identity (i.e., E.164 address). |
| Client name type | O | Indication of the type of the LCS client name. The type of the LCS client name can be one of the following: - Logical name - MSISDN - E-mail address (RFC 2396 [25]) - URL (RFC 2396 [25]) - SIP URL (RFC 3261 [26]) - IMS public identity (1 23.228 [27]) - GPSI |
| Override capability | O | Indication of whether the LCS client possesses the POI capability (only applicable to lawful intercept and emergency services clients) |
| Authorized UE List | O | A list of SUPIs and/or groups of SUPI for which the LCS client may issue a request for a 5GC-MT-LR for immediate or deferred location. |
| Priority | O | The priority of the LCS client |
| QoS parameters | M | The default QoS requirements for the LCS client, comprising: - Accuracy - Response time - LCS QoS Class |
| Service Coverage | O | A list of E.164 country codes for geographic areas, ITU-T Recommendation E.164 [23] where the LCS client is permitted to request and receive UE location information. |
| Allowed LCS Request Types | M | Indicates which of the following are allowed: - Request of current immediate location - Request of current or last known immediate location - Request of deferred location for the UE available event - Request of deferred location for UE periodic events - Request of deferred location for the Area Event - Request of deferred location for the Motion Event |
| Local Co-ordinate System | O | Definition of the co-ordinate system(s) in which a location estimate shall be provided – details are outside the scope of the present document |
| Access Barring List(s) | O | List(s) of SUPIs or groups of SUPI for which a location request is barred |
| Service Identities | O | List of service identities allowed for the LCS client. |
| Maximum Target UE Number | O | The maximum number of the Target UEs in one LCS request. For a specific LCS Client, this parameter may have different values for different service identities. |

7.2.x	Information for a location request

The GMLC holds information for a location request. Table 7.2.x-1 shows the information which may be stored in the GMLC for an external LCS Client.

Table 7.2.x-1: GMLC Information for a location request

| Reference ID | O | A reference ID allocated to external LCS client. It does not apply to the case location request is from an AF. |
| --- | --- | --- |
| External LCS client ID | M | Identity of the LCS client. |

# * * * * Second change * * * *

7.Y	AMF

7.2.y	Information for a location request

The AMF holds information for a location request. Table 7.2.y-1 shows the information which may be stored in the AMF for an external LCS Client.

Table 7.2.x-1: AMF Information for a location request

| UE identity | M | Identity of UE the location request is targeting for. |
| --- | --- | --- |
| Location request message | M | Message received from NEF or GMLC |
| LMF ID | M | Identity of the LMF serving the location request |
| GMLC ID | M | Identity of the GMLC serving the location request |
| NEF ID | M | Identity of the NEF serving the location request |
| UE LCS privacy | M | UE LCS Privacy retrieved from UDM |

# * * * * Third change * * * *

7.Z	LMF

7.2.z	Information for a location request

The LMF holds information for a location request. Table 7.2.z-1 shows the information which may be stored in the LMF for an external LCS Client.

Table 7.2.z-1: LMF Information for a location request

| UE identity | M | Identity of UE the location request is targeting for. |
| --- | --- | --- |
| Location request message | M | Message received from NEF or GMLC |
| AMF ID | M | Identity of the LMF serving the location request |

# * * * * Fourth change * * * *

7.A	NEF

7.2.A	Information for a location request

The NEF holds information for a location request. Table 7.2.A-1 shows the information which may be stored in the NEF for an external LCS Client.

Table 7.2.a-1: NEF Information for a location request

| Reference ID | O | A reference ID allocated to application function. |
| --- | --- | --- |
| AF ID | M | Identity of the AF c |

# * * * * End of changes * * * *