SA WG2 Meeting #136	S2-1912334

Reno, NV, USA, November 18 – 22, 2019	(merge of S2-192334)

Source:	Huawei, HiSilicon, NTT Docomo, China Mobile, CATT, Verizon, Nokia, Ericsson

Title:	Key Issue: Customization of 5GS for NPN deployment scenarios

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution proposes to add a new Key Issue in customization of 5GS for NPN deployment scenarios.

# 1. Introduction

The eNPN SID includes the following objective:

4.  Study the possibility for customizations or optimizations of 5GS when used for NPN considering different deployment scenarios, e.g. when the NPN is deployed and managed with the support of PLMN, when the NPN is deployed for different coverage and device density.

5.  Study the need for additional exposure capabilities due to support for NPN.

In this contribution, it is proposed to study potential enhancements to 5GS considering two customized NPN deployment scenarios.

# 2. Discussion

The 5G non-public network is deployed on the organisation’s defined premises, such as a campus or a factory, due to the high QoS/security requirements, or to be isolated from other networks as a form of protection[1], etc.

In some deployment scenarios from 5G-ACIA[1], NPNs are deployed in conjunction with a public network which is refered as PNI NPN in TS23.501.

„A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2]. An NPN may be deployed as:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public network integrated NPN, i.e. a non-public network deployed with the support of a PLMN.“

The verticals may require the UPF deployed in enterprise domain in order to protect their data/traffic. Meanwhile, verticals would like to isolate their NPN subscription data and operational data from the public network (physically or logically) [1].

„The chosen deployment scenario impacts the following privacy and security aspects:

•	Data privacy through isolation: Data in the NPN and the public network need to be segregated (physically or logically) and processed separately, in order to fulfil the security and privacy requirements of both networks. Note that the OT data includes not only the controller communication data, but also operational data such as subscriptions, number of active devices, devices identities etc.

Control and management privacy through isolation: This service aspect relates to the degree of segregation/isolation of the control and management plane functions of both networks for privacy and security reasons.“

In Rel. 16, the subscription data is managed by UDM. The operational data are controled by SMF, etc. and stored at the UDR. 5GC NFs dealing with NPN operational data needs to be isolated from the other 5GC NFs dealing with PLMN operational data.

Observation 1: PLMN NFs and NPN NFs may be deployed in different infrastructure and trusted-domains (isolated)

In case of PNI NPN, CP NFs deployed in PLMN (e.g., PCF, AMF, SMF, etc.) control the NPN traffic flows. PNI NPN is implemented as a PLMN network slice. NFs in NPN network (e.g., default AMF) needs to interact with PLMN NF (e.g., UDM) for the authenrization of the slice access.

Observation 2: Interaction between PLMN 5GC NFs and NPN 5GC NFs is needed to control NPN traffic flows.

In Rel. 16, the 3rd party/NPN is able to influence on/monitor per UE service via AF interaction with 5GC. AF can be considered as one of NPN NFs.

Observation 3: Interaction between PLMN 5GC NFs and NPN AF is needed for service customization.

In addition, SA5 TR28.807 defines a use case about “NPN provisioning by a network slice of a PLMN” specifying: “The NSI, based on the NPN requirement from the enterprise includes either only the RAN part or only the CN part or both the RAN part and the CN part.

For the CN part, there may be some network functions/network function services locally deployed at the enterprise’s premise or in the factory, and some other network functions/network function services being deployed in the PLMN network” and

“The 3GPP management system determines to utilize new CN NF(s) or CN NF service(s) of the CN part that are deployed locally at the enterprise’s premise or in the factory.”

This requires the interaction between PLMN and NPN in SA2 scope, in particular, how 5GC can support the Network Slice and Service assurance related exposure and control mechanisms.

Observation 4: Interaction between PLMN 5GC NFs and NPN 5GC NFs/AF are needed for the Network Slice and Service assurance related exposure and control mechanisms.

Proposal 1: It is proposed to study how to support the interaction between NPN and 5GC NFs considering cross domain deployment.

Proposal 2: It is proposed to study how to support the Network Slice and Service assurance related exposure and control mechanisms between NPN and 5GC NFs.

#### References:

[1] 5G-ACIA White paper, 5G Non-Public Networks for Industrial Scenarios, July 31, 2019

[2] TS22.261 Service requirements for the 5G system

# 3. Proposal

It is proposed to add a Key Issue of support of the interaction between NPN and 5GC NFs in FS_eNPN TR.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 23.501 [x] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.501 [x] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PNI-NPN	Public network integrated NPN

# * * * * Second change * * * *

## 5.X	Key issue# X: Support interaction of NFs/NF services deployed between operator’s and enterprise’s network domains

### 5.X.1	Description

In PNI-NPN, the Key Issue is to study

how to support the interaction of NFs/NF services deployed between operator’s and enterprise’s network domains:

  - When the NFs/NF services are distributed in the deployment between the operator’s network domain and enterprise’s network domains (e.g. AF, SMF, UPF deployed in enterprise’s network domain), how to support the NFs/NF services discovery and interaction between different trusted network domains.

  - Identify the capabilities and information that can be exposed between the different network domains.

NOTE 1:	Security aspects are to be covered by SA3.

# * * * * End of change * * * *