SA WG2 Meeting #136	S2-1912747

Reno, USA, November 18 – 22, 2019

Source:	China Mobile, Sprint,

Title:	Key issue on limitation of data rate per Network Slice in UL and DL.

Document for:	Discussion and Agreement

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2, Rel-17

Abstract of the contribution:

Discussion

One of the attributes in the GST documented in NG.116 is the following:

Maximum downlink throughput

This attribute defines the maximum data rate supported by the network slice in downlink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| Optional |  |

Maximum uplink throughput

This attribute defines the maximum data rate supported by the network slice in uplink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional | X |
| Optional |  |

There is no way for a PLMN to support per slice data rate limitation as so far. it is possible to perform per UE data rate limitation (UE-AMBR, see TS 23.501[x]), but not per network slice.

The difference between the key issue 3 and the new key issue is that, in key issue 3:

Whether and how to limit the data rate of UE for a Network Slice, ensuring that the aggregate of the PDU sessions that use the slice are rate limited to the rate defined for the Network Slice in DL and UL,

It is clear that in key issue 3, the rate limitation of network slice in DL and UL is based on limitation of per UE-level UL and DL data rate.

But in this new key issue, we would like to investigate other possible solutions, besides per UE level UL and DL data rate control.

For example, in order to limit the UL and DL data rate per slice level, the number of UEs, the number of PDU sessions, and the UL and DL data rate of per UE can be adjusted.

This key issue addresses this problem space.

Proposal

It is proposed the following key issue is added to TS 23.700-40

Proposed Text

5.X	Key Issue #X: Dynamic adjustment to meet the limitation of data rate per network slice in UL and DL.

5.X.1	General description

One of the attributes in the GST documented in GSMA 5GJA NG.116[x] is the following:

Maximum downlink throughput

This attribute defines the maximum data rate supported by the network slice in downlink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

Table 5.X.1-1: Maximum downlink throughput Table

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |
| Attribute Presence |  |
| Mandatory |  |
| Conditional | X |
| Optional |  |

Maximum uplink throughput

This attribute defines the maximum data rate supported by the network slice in uplink. These parameters can be used to offer different network slice contract qualities level, e.g. Gold, silver and bronze which have different maximum throughput values.

Table 5.X.1-2: Maximum uplink throughput

| Parameters |  |
| --- | --- |
| Value | Integer |
| Measurement unit | kbps |
| Example | 100 Mbps 20 Gbps |
| Tags | Scalability attributes KP |
| Attribute Presence |  |
| Mandatory |  |
| Conditional | X |
| Optional |  |

While it is possible for a PLMN to support per slice data rate limitation e.g. by configuration, In order to adhere to the GST parameters, the network may need to make adjustment to network to control aggregate traffic in UL and DL across the slice. e.g. trigger fairness across UE data rates in the PLMN when this limit of data rate is reached

This key issue shall study:

- Whether and how to adjust per UE data rate limits

- Whether and how to adjust total number of UEs operating in the slice

Editor’s note: it is FFS whether this attribute will be used at the same time as the max number of UEs, max bit rate of per UE per slice.

- Identify other network parameters need to adjust