3GPP TSG SA WG2 Meeting #136	S2-1911032

# Reno, USA, 18-22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1798 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for handover between SNPN and PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Description of service (PDU Session) continuity when UE moves between SNPN and PLMN when at the same time accessing PLMN via SNPN and vice versa is not clear in the specifications. There are 4 different scenarios that should be better described: UE handover from PLMN to SNPN when the PDU Session is anchored in PLMN. UE handover from PLMN to SNPN when the PDU Session is anchored in SNPN. UE handover from SNPN to PLMN when the PDU Session is anchored in PLMN. UE handover from SNPN to PLMN when the PDU Session is anchored in SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Informative procedures in a new annex is proposed. The procedures are already supported using the existing procedures in 23.502 but there is a benefit on showing more explicitly how these mobility procedures can be used for the applicable mobility scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not enough guidance for implementations as missing procedures for PDU Session continuity between PLMN and SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Rev "-": S2-1909377 Rev "1": S2-1911032: changing "shall" to informative statements |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** First change (All text is new) **************

Annex X (informative):
Procedures for Non-Public Network deployment options

# X.1	Overview

This Annex clarifies handover procedures between PLMN and SNPN.

# X.2	Handover of PLMN anchored PDU Session

## X.2.1	Handover of PLMN anchored PDU Session from 3GPP access to N3IWF via SNPN

Clause X.2.1 specifies how to handover a PLMN PDU Session from PLMN 3GPP access to N3IWF via SNPN.

Figure X.2.1-1: Handover of PLMN anchored PDU Session from 3GPP access to N3IWF via SNPN

no

This procedure assumes UE is registered via 3GPP access in PLMN and have one or more PLMN anchored PDU sessions.

1.	If the UE is not registered through N3IWF in PLMN via SNPN, the UE initiates Registration procedure as defined in clause 4.12.2.

2.	The UE performs PDU Session Establishment procedure with the PDU Session ID of the PDU Session to be moved as specified in clause 4.9.2.2 step 2.

3	If the User Plane of the PDU Session is activated in 3GPP access, the PLMN SMF executes the release of resource in 3GPP by performing step 3 in clause 4.9.2.2.

If the User Plane of the PDU Session is deactivated in 3GPP access, this step is skipped.

The steps 2 and 3 are repeated for all PDU Sessions to be moved from PLMN 3GPP access to PLMN N3IWF via SNPN.

## X.2.2	Handover of PLMN anchored PDU Session from N3IWF via SNPN to 3GPP access

Clause X.2.2 specifies how to handover a PLMN PDU Session from N3IWF via SNPN to PLMN 3GPP access.

Figure X.2.2-1: Handover of PLMN anchored PDU Session from N3IWF via SNPN to 3GPP access

This procedure assumes UE is registered via N3IWF access in PLMN and have one or more PLMN anchored PDU sessions.

1.	If the UE is not registered via PLMN 3GPP access, the UE initiates Registration procedure as defined in clause 4.2.2.2.2.

2.	The UE performs a PDU Session Establishment procedure with the PDU Session ID of the PDU Session to be moved as specified in clause 4.9.2.1, step 2.

3.	If the User Plane of the PDU Session is activated in N3IWF access, the PLMN SMF executes the release of resources in N3IWF access by performing step 3 in clause 4.9.2.1.

If the User Plane of the PDU Session is deactivated in N3IWF access, this step is skipped.

The steps 2 and 3 are repeated for all PDU Sessions to be moved from PLMN N3IWF via SNPN to PLMN 3GPP access.

# X.3	Handover of SNPN anchored PDU Session

## X.3.1	Handover of SNPN anchored PDU Session from 3GPP access to N3IWF via PLMN

Clause X.3.1 specifies how to handover a SNPN PDU Session from SNPN 3GPP access to N3IWF in SNPN via PLMN.

Figure X.3.1-1: Handover of SNPN anchored PDU Session from 3GPP access to N3IWF via PLMN

This procedure assumes UE is registered via 3GPP access in SNPN and have one or more SNPN anchored PDU sessions.

1.	If the UE is not registered in N3IWF in SNPN via PLMN, the UE initiates Registration procedure as defined in clause 4.12.2.

2.	The UE performs PDU Session Establishment procedure with the PDU Session ID of the PDU Session to be moved as specified in clause 4.9.2.2 step 2.

3	If the User Plane of the PDU Session is activated in 3GPP access, the SNPN SMF executes the release of resource in 3GPP by performing step 3 in clause 4.9.2.2.

If the User Plane of the PDU Session is deactivated in 3GPP access, this step is skipped.

The steps 2 and 3 are repeated for all PDU Sessions to be moved from SNPN 3GPP access to SNPN N3IWF access.

## X.3.2	Handover of SNPN anchored PDU Session from N3IWF via PLMN to 3GPP access

Clause X.3.2 specifies how to handover a SNPN PDU Session from SNPN N3IWF access via PLMN to SNPN 3GPP access.

Figure X.3.2-1: Handover of SNPN anchored PDU Session from SNPN N3IWF via PLMN to 3GPP access

This procedure assumes UE is registered via N3IWF access in SNPN and have one or more SNPN anchored PDU sessions.

1.	If the UE is not registered via SNPN 3GPP access, the UE initiates Registration procedure as defined in clause 4.2.2.2.2.

2.	The UE performs a PDU Session Establishment procedure with the PDU Session ID of the PDU Session to be moved as specified in clause 4.9.2.1, step 2.

3.	If the User Plane of the PDU Session is activated in non-3GPP access, the SNPN SMF executes the release of resources in N3IWF access by performing step 3 in clause 4.9.2.1.

If the User Plane of the PDU Session is deactivated in N3IWF access, this step is skipped.

The steps 2 and 3 are repeated for all PDU Sessions to be moved from SNPN N3IWF access via PLMN to SNPN 3GPP access.

*************** End of changes ***************