SA WG2 Meeting #137E	S2-2002083

Feb 24 - 28, 2020, Elbonia

Title:	[DRAFT] LS on UE radio capability format

Response to:

Release:	Release 16

Work Item:	RACS

Source:	OPPO

To:	RAN3

Cc:

Contact Person:

#### Name:	Jianhua Liu

E-mail Address:	liujianhua@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In the context of RACS, SA2 has found that the UE radio capability (e.g. the field RAT-type) is coded/decoded differently in TS 36.331 and TS 38.331. When RACS is used and CN sends to NG-RAN or E-UTRAN (“RAN node”) the radio capability corresponding to a UE Radio Capability ID, the RAN node does not know which format (i.e. TS 36.331 or TS 38.331) is in use in order to decode the UE radio capability.

SA2 discussed this issue and have the following questions:

1. If the UEs radio capabilities are retrieved over E-UTRAN in TS 36.331 format can the eNB that supports RACS function, locally create the TS 38.331 encoding for the same set of radio capabilities and upload it CN/UCMF? Is there any concern of the size impact on the CN-RAN interface message if both formats (of the same UE radio capability) need to be uploaded?

2. In SA2 there is also a proposal:

A. AMF/MME when requests for assignment of RAC ID, UCMF always stores the UE radio capability and the coding format (for example TS 38.331 or TS 36.331) in which the UE radio capability is stored.

B. When target RAN node requests for resolution of RAC ID, target RAN node will always get UE radio capability and coding format in which the UE radio capability is coded. The target RAN node will convert the UE radio capability in target coding format, if required. Can the target RAN node perform the required transcoding?

SA2 also wants to inform RAN groups that no decision has yet been made out of below proposed solutions:

a. whether both the TS 36.331 and TS 38.331 formats needs to be uploaded to CN as described in point 1; or

b. Only coding format will be provided to target RAN and target RAN is expected to do a transcoding as described in point 2; or

c. There is no impact to RAN WGs and the required format of UE radio capability is received by RAN nodes as per today’s standard, in this case CN handles to do required transcoding.

2. Actions:

To 3GPP RAN3:

ACTION:	SA2 kindly would like RAN3 to provide answers to the above questions and also provide any feedback necessary.

3. Date of Next TSG-WG2 Meetings:

TSG SA WG2 Meeting 138		20-24 April 2020			        Dubrovnik, Croatia

TSG SA WG2 Meeting 139		25-29 May 2020			        TBD