3GPP SA WG2 Meeting #S2-138E	S2-2002765

20 – 24 April, 2020, Electronic meeting          (merge of S2-2002765 and S2-2002878)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0152 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Any UE for Abnormal Behavior Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce Any UE subscription or request into Abnomal Behavior Analytics ID The data collection for Any UE may have major impact on load and signalling. Therefore care shall be taken with regards to load and major signaling caused when requesting Any UE. To optimize the data collection, different Filters may be used to lower the load and signalling. This contribution introduces a two-step method to lower load and signalling. If data from different types of NFs will be subscribed to, while assuring that data from the same UEs are retrieved, NWDAF may decide to optimize the data collection. If the Analytics Filter relevant to AMF (e.g. area of interest) is used in the subscription/request, the NWDAF first subscribes to data from AMF within the area of interest as in step 2. From the response from AMF it builds a list of SUPIs. Then it subscribes to data from SMF using this retrieved list of SUPIs as in step 3. If the Analytics Filter relevant to SMF (e.g. DNN and S-NSSAI combination) is used in the subscription/request, the NWDAF first subscribes to data from SMF within the DNN and S-NSSAI combination as in step 3. From the response from SMF it builds a list of SUPIs. Then it subscribes to data from AMF using this retrieved list of SUPIs as in step 2. When to perform this optimimization is NWDAF internal decision, as well as how NWDAF treats UEs that either start being served or end being served by either AMF or SMF during subscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Any UE as input parameter when requesting data from AMF and SMF. Also introduced an optimization to lower load and signalling utilizing Filter in the subscription or request to NWDAF. In the Moderated controversial issues email discussion only Area of interest has been brought up as a Filter. That optimization utilizes the Any UE subscription described for Observed Service Experience related Analytics to retrieve a list of SUPIs. Which is used as input for the subscription to SMF. But in this contribution also the same mechanism is used when DNN in the input to NWDAF, and the list of SUPIs need to come from SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any UE will not be included as a possible parameter in subscription or request for Abnormal Behavior analytics |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.1, 6.7.5.2, 6.7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR…. CR.. |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 6.7.5	Abnormal behaviour related network data analytics

#### 6.7.5.1	General

This clause defines how to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF.

NOTE 1:	The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

The consumer of this analytics could be a 5GC NF. The 5GC NF subscribes analytics on abnormal behaviour from a NWDAF based on the UE subscription, network configuration or application layer request.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription.

The consumer of this analytics shall indicate in the request:

-	Analytics ID set to "Abnormal behaviour";

-	The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information optionally including:

-	expected UE behaviour;

-	expected analytics type or list of Exception IDs with associated thresholds, where the expected analytics type can be mobility related, communication related or both;

-	Area of interest;

-	maximum number of objects;

-	maximum number of SUPIs;

-	Application ID;

-	DNN;

-	S-NSSAI.

NOTE 2:	The expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer, and the list of exception IDs indicates what specific analytics are expected by the consumer. Either the expected analytics type or the list of Exception IDs needs to be indicated, but they are not presented simultaneously. When the expected analytics type is indicated, the NWDAF performs corresponding abnormal behaviour analytics which are supported by the NWDAF. The relation between the expected analytics type and Exception IDs is defined in Table 6.7.5.1-1.

NOTE 3: 	If the target of Analytics Reporting is “any UE”, the target UEs for analytics reporting is retrieved based on a combination of the expected analytics type (or the list of exception IDs) and Analytics Filter Information as described in clause 6.7.5.4.

NOTE 4: 	If the target of Analytics Reporting is “any UE”, care shall be taken with regards to load and major signalling. Means to alleviate are via mandating some event filters (e.g. area of interest for AMF) and setting maximum number of SUPIs and/ or Sampling ratio as part of Event Reporting Information (clause 4.15.1, TS23.502 [3]).

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

Table 6.7.5.1-1: Relation between expected analytics type and Exception IDs

| Expected analytics type | Exception IDs matching the expected analytics type |
| --- | --- |
| mobility related | Unexpected UE location, Ping-ponging across neighbouring cells. |
| communication related | Unexpected long-live/large rate flows, Unexpected wakeup, Suspicion of DDoS attack, Wrong destination address, Too frequent Service Access/Abnormal traffic volume, Unexpected radio link failures. |

When the NWDAF detects those UEs that deviate from the expected UE behaviour, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.7.5.3.

#### 6.7.5.2	Input Data

The Exceptions information from AF is as specified in Table 6.7.5.2-1.

On request of the service consumer, the NWDAF shall collect and analyse UE behavioural information and/or expected UE behavioural parameters from the 5GC NFs (SMF, AMF, AF), or OAM.

NOTE 1:	Care shall be taken with regards to load and major signaling caused when requesting Any UE.

The UE behavioural information collected from 5GC NFs is as specified in clauses 6.7.2.2 and 6.7.3.2.

The expected UE behavioural parameters provided to the NWDAF are defined in clause 4.15.6.3, TS 23.502 [3].

Table 6.7.5.2-1: Exceptions information from AF

| Information | Description |
| --- | --- |
| IP address 5-tuple | To identify a data flow of a UE via the AF (such as the Firewall or a Threat Intelligence Sharing platform) |
| Exceptions (1..max) (NOTE) |  |
| >Exception ID | Indicating the Exception ID (such as Unexpected long-live/large rate flows and Suspicion of DDoS attack as defined in Table 6.7.5.3-2) of the data flow. |
| >Exception Level | Measured level, compared to the threshold |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| NOTE: The Exceptions information and the UE behavioural information as defined in clauses 6.7.2.2 and 6.7.3.2 could help NWDAF to train an Abnormal classifier, which could be used to classify a UE behaviour data into Normal behaviour or Exception. |  |

#### 6.7.5.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1 and Table 6.7.5.3-2. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer with the exception ID associated with the exception if the exception ID is within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer. The NWDAF shall provide the Exception Level and determine which of the other information elements to provide, depending on the observed exception.

Abnormal behaviour statistics information is defined in Table 6.7.5.3-1.

Table 6.7.5.3-1: Abnormal behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk |

Abnormal behaviour predictions information is defined in Table 6.7.5.3-2.

Table 6.7.5.3-2: Abnormal behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk |
| > Confidence | Confidence of this prediction |

The UE characteristics may provide a set of features common to all UEs affected with the exception.

The number of exceptions and the length of the SUPI list shall respectively be lower than the parameters maximum number of objects and Maximum number of SUPIs provided as input parameter.

If PCF subscribes notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.7.5.3-2 gives examples of additional measurement, AM/SM policies and corresponding actions for solving each risk.

Table 6.7.5.3-3: Examples of policies and actions for risk solving

| Exception ID and description | Additional measurement | AM/SM policy | Actions of NFs |
| --- | --- | --- | --- |
| Unexpected UE location | Unexpected UE location (TA or cells which the UE stays) | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Ping-ponging across neighbouring cells | Numbers, frequency, time and location information, assumption about the possible circumstances of the ping-ponging | NWDAF notifies the AMF or AF (Service Provider) | If the amount of ping-ponging across neighbouring cells is above the thresholds set by the service provider, the service provider may adjust and improve the antenna tilts of the neighbouring cells or the overlapping coverage conditions in the affected location. If the ping-ponging are per UE, then: 1. the AMF may adjust the UE (e.g. a stationary UE) registration area. 2. the AMF and/or the AF may allow the use of Coverage Enhancement for the affected UE. |
| Unexpected long-live/large rate flows | Unexpected flow template (IP address 5 tuple) | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Time of unexpected wake-up | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Victim's address (target IP address list) | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Wrong destination address (target IP address list) | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Too frequent Service Access/Abnormal traffic volume | Volume, frequency, time, assumptions about the possible circumstances | NWDAF notifies AF (Service Provider) |  |
| Unexpected radio link failures | Numbers, frequency, time and location, assumptions about the possible circumstances | Not applicable | If the unexpected radio link failures are per UE location bases, the AMF may allow the use of CE (Coverage Enhancement) in the affected location. Also, the Operator may improve the coverage conditions in the affected location. If the unexpected radio link failures are per UE bases, then the AMF and/or the AF may allow the use of CE for the affected UE. |

#### 6.7.5.4	Procedure

Figure 6.7.5.4-1: Procedure for NWDAF assisted misused or hijacked UEs identification

1a.	A consumer NF subscribes to/requests NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsInfo_Request (Analytics ID set to "Abnormal behaviour", Target of Analytics Reporting = Internal-Group-Identifier, Any UE or SUPI, Analytics Filter Information = Area of Interest, DNN, S-NSSAI).

A consumer NF may subscribe to/request abnormal behaviour notification/response from NWDAF for a group of UEs, Any UE or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic.

1b.	Consumer AF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting = External-group identifier or External UE ID or Any UE, Analytics Filter Information= DNN, S-NSSAI).

For untrusted AFs, the AF sends the subscription via a NEF, where the AF invokes NEF service Nnef_AnalyticsExposure_Subscribe or Nnef_AnalyticsExposure_Fetch (Analytics ID, Target of Analytics Reporting = External-group-identifier or External UE ID or Any UE, Analytics Filter Information= Application ID).

An AF may also subscribe to/request abnormal behaviour notification/response from NWDAF for a group of UEs or a specific UE or any UE, where the subscription/request message may contain expected UE behaviour parameters identified on the application layer. If an External-Group-Identifier is provided by the AF, the NEF interrogates UDM to map the External-Group-Identifier to the Internal-Group-Identifier and obtain SUPI list corresponding to the Internal-Group-Identifier. The NEF may also map AF-Identifier (e.g. Application ID) into DNN and S-NSSAI combination based on local configuration.

1c. [OPTIONAL] If data from different types of NFs will be subscribed to, while assuring that data from the same UEs are retrieved, NWDAF may decide to optimize the data collection.

NWDAF decides the order in which steps 2 and 3 are executed

If the Analytics Filter relevant to AMF (e.g. area of interest) is used in the subscription/request, the NWDAF first subscribes to data from AMF within the area of interest as in step 2. From the response from AMF, it builds a list of SUPIs. Then it subscribes to data from SMF using this retrieved list of SUPIs as in step 3.

If the Analytics Filter relevant to SMF (e.g. DNN and S-NSSAI combination) is used in the subscription/request, the NWDAF first subscribes to data from SMF within the DNN and S-NSSAI combination as in step 3. From the response from SMF, it builds a list of SUPIs. Then it subscribes to/requests data from AMF using this retrieved list of SUPIs as in step 2.

When to perform this optimimization is NWDAF internal decision, as well as how NWDAF treats UEs that either start being served or end being served by either AMF or SMF during subscription.

2.	NWDAF to AMF (Conditional): Namf_EventExposure_Subscribe (Event ID(s), Event Filter Information =Area of Interest, Internal-Group-Identifier, Any UE, SUPI or list of SUPIs).

The NWDAF sends subscription requests to the related AMF to collect UE behavioural information if it has not subscribed such data.

NOTE 1:	The NWDAF determines the AMF serving the UE as described in clause 6.2.2.1.

The AMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the AMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the AMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

[Conditional] The NWDAF may collect UE mobility information from OAM as part of step 2, following the procedure captured in clause 6.2.3.2.

3.	NWDAF to SMF (Conditional): Nsmf_EventExposure_Subscribe (Event ID(s), Event Filter Information =(DNN, S-NSSAI), Internal-Group-Identifier, Any UE, SUPI or list of SUPIs).

The NWDAF sends subscription requests to the related SMF if it has not subscribed such data.

NOTE 2:	The NWDAF determines the SMF serving the UE as described in clause 6.2.2.1.

The SMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the SMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the SMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

4.	The NWDAF performs data analytics for misused or hijacked UEs identification. Based on the analytics and operator's policies the NWDAF determines whether to send a notification to 5GC NFs or the AF.

5a.	NWDAF to consumer NF (AMF or PCF or SMF depending on the subscription) (Conditional): Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Response (Analytics ID, (Exception ID, Internal-Group-Identifier, Any UE or SUPI, Exception level)) (which is used depending on the service used in step 1a).

If the NWDAF determines to send a notification/response to the consumer 5GC NFs, the NWDAF invokes Nnwdaf_EventSubscription_Notify or Nnwdaf_AnalyticsInfo_Response services. Based on the notification/response, the 5G NFs adopt configured actions to resolve/mitigate/avoid the risks as described in the Table 6.7.5.3-1.

5b/5c.	NWDAF to AF (Conditional): Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Response (Analytics ID, (Exception ID, Exception Level, External UE ID)) (which is used depending on the service used in step 1b).

If the NWDAF determines to send a notification/response to the consumer AF, the NWDAF needs to include external UE ID of the identified UE into the notification/response message.

NOTE 3:	Based on the notification, the AF can adopt corresponding actions, e.g. adjusting recommended TCP Window Size, adjusting recommended Service Start and End.

NOTE 4:	The call flow only shows a subscribe-notify model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

*** END OF CHANGES ***