# 3GPP TSG SA2 Meeting #138E	S2-2003176r10

# Elbonia, April 20 – 23, 2020	(revision of SP-190xxxx)

Source:	Nokia, Nokia Shanghai Bell, Ericsson, Samsung, Qualcomm

Title:	New WID: System enhancement for Redundant PDU Session

Document for:	Approval

Agenda Item:	9.1

Abstract:		New WID to introduce system enhancement for redundant PDU Session

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	System enhacement for Redundant PDU Session

## Acronym:	SE_RPS

## Unique identifier:	XXX

Potential target Release: Release 17.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820019 | Enhancement of URLLC support in the 5G Core network (5G_URLLC) | Rel-16 work item for URLLC |

## 3	Justification

In Rel-16, related to SA2 5G_URLLC and RAN NR_IIoT, RAN3 in their LS (R3-194794) state:

- RAN3 has discussed Solution #1 of KI 1, especially about how the RSN is to be used by NG-RAN node, and if PDU Session pair information (i.e. the two PDU sessions are indicated as the redundant pair) is needed.

- There is consensus that RSN maywork. However, if PDU session pair information is not provided (within RSN or otherwise), this may imply a change in the way the MN selects the SN for DC: according to current principles, this selection shall ultimately be based on radio and RAN load conditions (which is available to RAN and not to core network). The same considerations apply when one gNB-CU and two gNB-DUs are used.

- RAN3 sees benefit if 5GC can provide the PDU session pair information to gain more flexibility in NG-RAN node, but RAN3 is not sure if the 5GC would be able to provide this information to NG-RAN node without adding too much complexity.

SA2 discussed various network-based solutions but could not reach consensus to move forward with any of the solution due to complexities (i.e. due to configuration involved in the network or other reasons). SA2 also discussed that it was too late in the release cycle to agree on a solution with UE impact. Thus, SA2 responded to RAN3 as follows in S2-1912649:

- SA2 has discussed this issue extensively without consensus due to solutions requiring increased complexity at the same time did not agree to a complete solution which may require impact to the UE. Since Release 16 stage 2 is frozen, SA2 concluded that for Rel-16 SA2 will not provide any solution to gain more flexibility and rely upon RSN as currently specified.  SA2 may consider this issue in future release and provide a complete solution, if there is a need to do so.

## 4	Objective

If the UE has the knowledge about PDU Session pair information for the redundant PDU Sessions, it is proposed that the UE provides PDU Session pair information  to the SMF(s) so that the SMF(s) can provide this information to the NG-RAN that the NG-RAN can eventually use the information for SN selection, also gNB CU/DU Selection. This would allow the two PDU Sessions to be independently established without any constraints on the selected SMF(s).

If the UE releases one of the redundant PDU Sessions and establish the third PDU Session, the PDU Session pair information  can be used for coordination with the newly established PDU Session.

Backward compatibility with Rel-16 should be considered (e.g. Rel-17 UE(s) in Rel-16 network).

In addition, it is to be verified if any additional work is needed to define how the UE is aware  of PDU Session pair information for the redundant PDU Sessions.

This will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as “Remarks”.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 23.501 | Updates to clause 5.33.2.1 | TSG SA#90 |
| 23.502 | Updates to PDU Session related procedure | TSG SA#90 |

## 6	Work item Rapporteur(s)

Devaki Chandramouli, devaki.chandraouli@nokia.com

## 7	Work item leadership

SA2.

## 8	Aspects that involve other WGs

Charging aspects will be studied by SA5.

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Ericsson

Qualcomm

Samsung

Huawei

LG Electronics Matrixx