SA WG2 Meeting #S2-138E	S2-2003268

20 - 24 April, 2020, Electronic, Elbonia	(revison of S2-2002926)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0160 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further corrections to Observed Service Experience analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For slice service experience and slice instance service experiences in Service Experience analytics: - Spatial validity, valid period, SUPI list and Ratio are also needed as part of the statistics. - Confidence (i.e. probability assertion), spatial validity, valid period, SUPI list and Ratio are also needed as part of the predictions information. - With the introduction of slice instance service experience, the slice service experience can be defined as service experience across Network Slice instances over the Analytics target period (average, variance). And the slice service experience is provided if multiple Network Slice instances are deployed. |  |  |  |  |  |  |  |  |
| FO |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add spatial validity and valid period to the statistics information for slice service experience and slice instance service experience. Add SUPI list and Ratio to the statistics information for slice instance service experience. 2. Add spatial validity, valid period and probability assertion to the predictions information for slice service experience and slice instance service experience. Add SUPI list and Ratio to the predictions information for slice instance service experience. 3. Clarify that the slice service experience is service experience across Network Slice instances and is provided only when multiple Network Slice instances are deployed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete statistics/predictions information for slice service experience and slice instance service experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** Start of Change ********************

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice service experience | Service experience across Network Slice instances over the Analytics target period (average, variance). |
| Slice instance service experiences (1…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Application service experiences (1..max) | List of observed service experience information for each Application. |
| >> Application ID | Identification of the application. |
| >> Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >> Service Experience | Service Experience over the Analytics target period (average, variance). |
| >> SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice service experience | Service experience across Network Slice instances |
| > Slice service experience | Service experience across Network Slice instances over the Analytics target period (average, variance). |
| > Probability assertion | Confidence of this prediction. |
| Slice instance service experiences (1…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Probability assertion | Confidence of this prediction. |
| > Application service experiences (1..max) | List of predicted service experience information for each Application. |
| >> Application ID | Identification of the application. |
| >> Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >> Service Experience | Service Experience over the Analytics target period (average, variance). |
| >> SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| >> Probability assertion | Confidence of this prediction. |

NOTE 1:	The spatial validity and validity period are provided for slice service experiences, slice instance service experiences or application service experiences if needed, in Service Experience statistics and predictions information.

NOTE 2:	If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Slice instance service experience entry is provided. In that case, the NSI ID is not provided and the Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 3:	The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and with the same value(s) in the service experience information for the slice instance which the application belongs to.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided  in Analytics Reporting Information by the NWDAF Service Consumer.

******************** End of Change ********************