SA WG2 Meeting #139E	S2-2003733r01

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #7, New Sol: AMF assistance to NG-RAN during PDU Session establishment procedure

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution for 5GC assisted cell selection to access network slice. The solution assumes that Allowed S-NSSAI consists of slices which cannot operate on the same frequency band. The solution also assumes that NSSF maintains the mapping between the S-NSSAI and the allowed frequency bands to operate.

# 1. Introduction

As highlighted in this key issue, an operator may only support certain network slices on certain operating frequency bands. One registration area may consist of multiple NG-RAN cells belonging to different operating bands, and thus it is possible that support for a particular S-NSSAI in a registration area is non-homogenous.

In this paper, we propose AMF shall provide assistance to NG-RAN about the S-NSSAI and the operating frequency band(s) in which the slice is allowed to operate as part of the PDU session establishment procedure. NG-RAN may use this assistance to steer the UE to the appropriate operating frequency band to access the slice.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| x | AMF assistance to NG-RAN during PDU Session establishment procedure | 7 |

NEXT CHANGE (All text is new)

## 6.X	Solution #x: AMF assistance to NG-RAN during PDU Session establishment procedure

### 6.X.1	Introduction

This solution is for Key Issue #7, "Support of 5GC assisted cell selection to access network slice". The solution is based on the following architectural assumptions:

-	The UE may simultaneously register to slices that are not all accessible on the same operating band(s). In other words, the UE may have S-NSSAIs in the Allowed NSSAI that are not all available in a common operating band.

-	The UE may simultaneously establish and access PDU Sessions on slices that are not all supported on the same operating band.

-	NSSF maintains the mapping between S-NSSAI and allowed frequency bands to operate.

-	AMF caches the information of the mapping between S-NSSAI and allowed frequency bands to operate.

### 6.X.2	High Level Description

NG-RAN is aware of the operating bands for each S-NSSAI based on the slice creation.

NG-RAN informs the UE to use a specific frequency band when sending and receiving user plane packets for the particular S-NSSAI.

Editor’s Note: How the NG-RAN informs the UE and during which procedure is FFS and should be defined in RAN WGs.

Editor’s Note: How the NG-RAN steers the UE is FFS (e.g. handover, configuring dual connectivity to simultaneously access two operating bands, etc.) should be defined in RAN WGs.

### 6.X.3	Procedures

The procedure in Figure 6.X.3-1 shows a signalling flow in which the AMF provides the "Allowed list of operating bands" to the NG-RAN as part of PDU Session Resource Setup Request.

Figure 6.X.3.1 – Providing "Allowed list of operating bands" to NG-RAN as part of PDU Session Resource Setup Request

0.	AMF has a copy of the mapping table (from NSSF) of S-NSSAI(s) and the allowed list of frequency bands to operate.

1.	UE sends PDU Establishment Request for the S-NSSAI.

NOTE:	Based on the S-NSSAI included by the UE in RRCConnectionSetup, NG-RAN may steer the UE to access the new operating band which supports the S-NSSAI user is interested in accessing.

Editor’s Note: How the NG-RAN steers the UE (e.g. handover) should be defined in RAN WGs.

2.	PDU Session establishment procedure continues as defined in TS 23.502 [6], clause 4.3.2.2.1, steps 2-11.

3.	AMF provides NG-RAN the following parameters required for the PDU Session via PDU Session Resource Setup Request: PDU Session ID, CN Tunnel Info, QoS profile with QFI, S-NSSAI, NAS PDU, UL NG-U UP TNL Information.

Editor’s Note: the PDU Session Establishment procedure is processed by AMF and SMF without direct knowledge of the UE radio capabilities in terms of supporting the frequency bands associated to the S-NSSAI, therefore the procedure handling if the UE is not capable of supporting the corresponding radio spectrum is FFS.

4.	NG-RAN responds with PDU Session Resource Setup Response that it is able to support the requested configuration.

5.	NG-RAN sends RRC Reconfiguration message to inform the UE to appropriately steer traffic for the S-NSSAI via the Allowed list of frequency band.

Editor’s Note: How the NG-RAN steers the UE is FFS (e.g. handover, configuring dual connectivity to simultaneously access two operating bands, etc.) should be defined in RAN WGs.

Editor’s Note: the handling of any existing PDU sessions for other S-NSSAIs needs to be defined.

6.	AMF forwards the PDU Session Establishment Accept to UE.

### 6.X.4	Impacts on existing entities and interfaces

-	NSSF shall be able to provide the supported operating frequency band(s) for each allowed S-NSSAI(s) to the AMF.

- 	AMF shall be able to provide supported operating frequency band(s) for S-NSSAI(s) to the NG-RAN as part of  PDU Session Resource Setup Request.

- 	NG-RAN shall be able to steer the UE to appropriate operating frequency band which supports the S-NSSAI included in the RRC connection procedure.

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *