3GPP TSG-SA/WG2 Meeting #140	S2-2004835r01

# Elbonia, Oct, 2020

Source:	ZTE

Title:	KI#3, New Sol: RAN node information exposure in AMF

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes a new solution for KI#3

1	Introduction

2	Proposal

It is proposed to agree the following changes in TR 23.748.

************* Start Changes *************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 5 |
| #1: Provisioning URSP configuration to the UE to establish PDU Sessions for edge applications | X |  |  |  |
| #2: Local DNS based edge server address discovery | X |  |  |  |
| #3: DNS AF | X |  |  |  |
| #4: Providing the DNS authoritative server with IP addressing information about where the UE is located | X |  |  |  |
| #5: Server Discovery using DNS, IP Routing and URSP | X |  |  |  |
| #6: Discovery of EAS based on DNS | X |  |  |  |
| #7: SMF/I-SMF selection based on DNAI | X |  |  |  |
| #X: RAN information exposure in AMF |  |  | X |  |

************* Next Change*************

6.x	Solution #n: RAN Node Information Exposure in AMF

6.x.1	Solution description

6.x.1.1	General

This solution addresses the Key Issue #3 Network Information Provisioning to Local Applications with low latency.

In some Edge Computing deployment the application may need to request the RAN information from RAN node directly with low latency. The main issue is how to identify the RAN node that the UE is current accessing and how to identify the UE within the RAN node. This solution proposes that for UE in Connected mode the AMF notifies the information of the RAN node that the UE is current accessing to the application. The RAN node information may include the available TNL address in the RAN node and the RAN UE NGAP ID. The application may use this information to request UE related RAN information from RAN node. As the RAN UE NGAP ID is unique within the RAN node the application can uses the RAN UE NGAP ID to uniquely identify the UE in the RAN node.

NOTE: 	What UE related information can be provided by RAN node and the protocol can be used between the RAN node and the application are outside of this solution.

6.x.2	Procedures

The procedure describes how the application can request information from RAN node..

Figure 6.x.2-1 Call flow for RAN Node information exposure

Step 1. The Application invokes Nnef_EventExposure_Subscrbe service (Event ID(s), NEF address, GPSI, DNN, S-NSSAI) in NEF. This message includes Event ID which indicates to subscribe the RAN node information from the AMF.

Step 2. The NEF discover the UDM and invokes Nudm_EventExposure_Subscribe service.

Step 3. Based on the current Registered AMF information, the UDM invokes the Namf_EventExposure_Subscribe service(Event ID, SUPI, NEF address, S-NSSAI, DNN ) in AMF. The Event ID indicates to subscribe the RAN node information from the AMF.

Step 4. The AMF stores the subscription information in the UE context. The AMF sends Namf_EventExposure_Subscribe Respose to the UDM.

Step 5. The UDM sends Nudm_EventExposure_Subscribe Response to the NEF.

Step 6. The NEF sends Nnef_EventExposure_Subscribe Response to Applications.

Step 7. When there is any update on the RAN node information, e.g. the UE enters the CONNECTED mode or handovers to another RAN node, the AMF sends Namf_EventExposure_Notification to the NEF including the RAN node information. The RAN node information may includes RAN node TNLA address(es) and RAN UE NGAP ID.

Step 8. The NEF sends Nnef_EventExposure_Notification to the Application.

After this step the NEF may use the RAN node information to request the UE related RAN information from RAN node.

6.x.3	Impacts on Existing Nodes and Functionality

UDM

1.	Support new event to expose the RAN node information.

NEF

1.	Support new event to expose the RAN node information.

AMF

1.	Support new event to exposure the RAN node information. The RAN node information includes RAN node TNLA addresses and current RAN UE NGAP ID of the UE in Connected mode.

*************** End Changes ***************