SA WG2 Meeting #140E	S2-2005851r01

19 August - 2 September 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #2, New Solution: Multiple AF Scenarios

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This document proposes a new solution to KI#2, and is specific to multiple AF scenarios, e.g. due to local EAS relocation.

# 1		Discussion

Edge Computing deployments include Edge Application Servers located at the Edge Data Network, as also shown in “Architectural Assumptions” section figure 4.1-1 of TR 23.748 v0.4.0. Often these edge network has associated control plane Application Function, thus implying multiple AF instances i.e. a local AF and also AF located in the core.

Some of the solutions in TR 23.748 v0.4.0 already assume such deployments with multiple AFs, for example, solution# 28 and solution#13. However, local AF scenarios needs some additional considerations.

This pCR proposes a new solution to address multiple AF scenarios.

# 2	Proposal

It is proposed to update TR 23.700-91 as follows.

*** Start Change – all new text ***

## 6.X	Solution to Key Issue 2: Multiple AFs

### 6.X.1	Description

This solution addresses scenarios where there are Edge or local Application Function located in the Edge Data Network, and also one or more Edge Application Server (EAS).

In such scenarios: as part of initial PDU session establishment, an AF located in the core network is selected, however, due to UE mobility and corresponding relocation to Edge DNAI, a new local AF serving Edge Applications are selected. There are a few additional details that needs to be addressed in such multiple AF deployments, such as: how to relocate AF, how SMF provides to new AF required notification of user plane management events such as PSA relocation, etc.

### 6.X.2	Procedures

Detail procedure involving multiple AF scenarios are provided in the below figure 6.X.2-1.

Figure 6.X.2-1: Procedures to AF Relocation

Step 0: AF1 is interacting with SMF/5GC as part of initial PDU Session establishment procedure.

Step 1. AF1 decides to relocate to local AF i.e. to AF2 which serves local EAS located in the target DNAI. There could be multiple triggers for AF1 to relocate AF e.g. decision to relocate to local /another Edge Application Server due to change in DNAI, EAS load balancing, etc..

Step 2. AF1 initaites context transfer procedure with AF2, this is to provide to AF2 information such as: application status, UE ID, PDU session Information, etc.

Step 3: AF2, based on the information received from AF1, sends to NEF, Nnef_TrafficInfluence_update request, and includes Notification Correlation ID, UE ID, PDU session information, as received from AF1.

Step 4: NEF stores the AF2 requested information in the UDR (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

Step 5:  NEF acknowledges to AF2 by sending Nnef_TrafficInfluence_update response message

Step 6. UDR notifies to PCF with Nudr_DM_Notify of the changed subscription.

Step 7: PCF determines that existing PDU Sessions are potentially impacted by the AF2 request, and so it updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service operation

Step 8: Based on the updated PCC rules received from PCF, SMF may decide to reconfigure user plane e.g. change in DNAI, PSA relocation and so on.

Step 9-10. SMF sends Nsmf_EventExposure_Notify indicating the changes in the user plane management information to AF2 either directly or via NEF.

### 6.X.5	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

The proposed solution requires enhancement to AF behaviour by requiring new messages between one or more AFs

*** End Change ***