SA WG2 Meeting #141E e-meeting	S2-2007027

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI#2: Conclusion on NWDAF re-selection due to UE mobility

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/ Rel-17

Abstract of the contribution: This contribution proposes conclusion for KI#2 regarding NWDAF re-selection due to UE mobility.

1. Discussion

There are four solutions in the TR 23.700-91 address the re-selection aspect for mobility of KI #2 in the multiple NWDAF deployment scenario:

- Solution 10: Handling of mixed and distributed NWDAF deployments

- Solution 26: Reselection of NWDAF

- Solution 57: Reselection of NWDAF due to mobility change

- Solution 59: Analytics handover

This contribution paper proposes conclusion for the mobility/ reselection aspect of KI#2

Sol#10, 26, 57 and 59 propose the different procedures of re-selection for NWDAF,Sol#26, 57 and 59 share the same principle that NWDAF should trigger re-selection, compared with the procedure that AMF should trigger the re-selection in Sol#10.

Proposal:

It is proposed to use Sol#57 as baseline to merge Sol#26 and 59 as discussed in S2-200xxxx by share the same comments that NWDAF should trigger the re-selection procedures for UE mobility or other scenarios.

Also, Sol#57 proposes the NWDAF to subscribe the UE mobility event from AMF, it will guarantee the re-selection of NWDAF to be triggered when NWDAF is aware of the UE mobility change and it can’t continue the analytics service for certain users in certain area. Meanwhile Sol#59 suggests the re-selection may be triggered based the prediction of UE mobility analytics, which can be merged into Sol#57 as a complementary method.

Besides, Sol #57 is focusing on a re-selection based on UE mobility, while Sol # 26 and 59 is covering also other use cases, e.g. for load-balancing or graceful shutdown.

2. Text Proposal

It is proposed the following changes to TR 23.700-91 [2].

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

8.x	Key Issue #2: Mobility/Reselection of NWDAF

For NWDAF re-selection due to UE mobility, it is concluded that the merged Solution#57 is selected as basis for normative work.

Sol#26 and Sul#59 could cover other scenarios like load-balancing or graceful shutdown.

# * * * * End of changes * * * *