SA WG2 Meeting #141E e-meeting	S2-200xxxx

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, CATT , China Mobile, Samsung? Lenovo?

Title:	KI #16, Update Sol #48 merging with Sol #49 and 50

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update solution 48 merging with 49 and 50 for Key Issue 16. In this merged solution, it is proposed to define a new analytics of performance analytics per UPF&DNAI path for MEC optimization.

# 1. Discussion

There are six solutions to address the KI#16-UP optimization for edge computing:

- Solution 31(HW ): NWDAF Assisted UP Optimization for edge computing

- Solution 47(LG ): UE Presence Pattern analytics to support edge computing

- Solution 48(CATT ): NWDAF assisted UP optimization for EC

- Solution 49( Lenovo): Selecting an Edge Application Server instance based on NWDAF analytics

- Solution 50( Samsung): Network Assisted DNAI selection

- Solution 51( CMCC) : Optimization for edge computing

Sol#48, 49 (alternative 1), 50 also propose the NWDAF to derive a new analytics of performance analytics per UPF&DNAI path to provide Bandwidth/RTT per DNAI (as defined in solution 50) and Performance of Application Server per DNAI (as defined in solution 49) and EC service performance analytics (as defined in solution 48).

Proposal:

It is proposed to use sol#48 as baseline to merge Sol#48, 49 (alternative 1) and 50, by defining a new analytics of performance analytics per UPF&DNAI path.

## 2. Text Proposal

It is proposed the following changes to TR 23.700-91 [2].

- * * * * First change * * * *

## 6.48	Solution #48(merging Sol# 49(alternative 1), 50): NWDAF assisted UP optimization for EC by defining a new DN performance analytics

### 6.48.1	Description

The solution addresses Key Issue #16: UP optimization for edge computing.

In order to promise user experience, when a UE is visiting EC server, the 5GS needs to select an appropriate anchor UPF to serve the UE. The appropriate anchor UPF here means that, the UP path via this anchor UPF can satisfy the QoS requirement well, and the number of anchor UPF relocation can be minimized.

In this solution, it is proposed to rely on NWDAF analytics to select anchor UPF and DNAI to promise user experience. Besides UE mobility analytics defined in Rel-16, a new analytics named " DN performance analytics" is introduced, which contains the performance information for an EC application per specific area per time period.

The UE mobility analytics and "DN performance analytics" will be notified to the SMF and EC server. Based on such NWDAF analytics, the SMF can select the anchor UPF which can provide best user experience to the UE along the UE moving trajectory and determine the most appropriate target DNAI; the EC server can determine the target DNAI and/or target EC server if EC server relocation is required.

The input data for generating the "DN performance analytics" could be collected from 5GC and EC server. The input data collected from 5GC is defined in the Table 6.4.2-2 in TS 23.288 [5] with some additions, and the input data collected from EC server is defined in the Table 6.4.2-1 in TS 23.288 [5]. The additions to the Table 6.4.2-2 in TS 23.288 [5] is shown in the table 6.48.1-1.

Table 6.48.1-1 Additions to the Table 6.4.2-2 in TS 23.288 [5]

| Information | Source | Description |
| --- | --- | --- |
| Content defined in the Table 6.4.2-1 in TS 23.288 [5] | Content defined in the Table 6.4.2-1 in TS 23.288 [5] | Content defined in the Table 6.4.2-1 in TS 23.288 [5] |
| Anchor UPF ID | SMF | The anchor UPF of the PDU session containing the QoS flow |

Table 6.48.1.1-2: Application Server Performance Data from EDN network as depicted in solution#49

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | The application identity associated with the communication session of the UE when the measurement was made |
| UE identifier | AF | IP address of the UE at the time the measurements was made |
| UE location | AF | The location of the UE when the performance measurement was made |
| Application Server Instance | AF | The IP address of the Application Server that the UE had a communication session when the measurement was made |
| Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: Average Packet Delay, Average Loss Rate and Throughput. |
| Timestamp | AF | A time stamp associated to the communication session of the UE |

(statistic/prediction)

The EC service performance analytics is shown in the Table 6.48.1-3.

Table 6.48.1-3 DN service performance analytics (statistic/prediction)

| Information | Description |
| --- | --- |
| Application ID |  |
| S-NSSAI |  |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| DNAI | Identifies the access to DN for which analytics information is provided |
| DN performance (0-x) | List of DN performance for the EC application |
| >APP location | Application location, could be identified by DNAI |
| >Serving anchor UPF | The involved anchor UPF |
| >Performance | Performance indicators |
| >> Average Traffic rate | Average traffic rate for visiting the EC application |
| >> Maximum Traffic rate | Maximum traffic rate for visiting the EC application |
| >>Average Packet Delay | Average packet delay for visiting the EC application |
| >>Maximum Packet Delay | Maximum packet delay for visiting the EC application |
| >> Average Packet Loss Rate | Average packet loss for visiting the EC application |
| >Spatial Validity Condition | Indicates the specific area where the service experience can be promised |
| >Temporal Validity Condition | Indicates the specific time period in which the service experience can be promised |
| >Confidence |  |

### 6.48.2	Procedures

Figure 6.48.2-1: Procedure for NWDAF providing data analytics for an EC Application

1.	Consumer NF (e.g. SMF, AF) sends an Analytics request/subscribe (Analytics ID = UE mobility and DN performance) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2-4.The NWDAF performs data collection if it hasn't subscribe the input data for the requested analytics. The NWDAF has to collect data from AMF, SMF and AF.

5.	the NWDAF derives the requested analytics.

6.	The NWDAF provide the analytics result to the consumer NF, including UE mobility analytics and DN performance analytics.

7.	During the PDU session establishment or anchor UPF change (as specified in clause 4.3.5 of TS 23.502 [3]) procedure, the SMF, as an analytics consumer, can use UE mobility analytics to predict UE location, and then check DN performance analytics, by taking UE location into account, to find which UPF and DNAI can serve the UE with the best user experience or service performance.

The AF as an analytics consumer can also use UE mobility analytics and DN performance analytics to determine which DNAI is more suitable if EC server relocation is required.

### 6.48.3	Impacts on services, entities and interfaces

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

NWDAF:

-	Need to provide a new analytics for DN performance for a specific application.

-	Need to collect data from multiple UPFs (DNAIs ).

-	Collecting performance data of Application Server Instances from each EDN

NEF:

-	Need to expose DN performance analytics to AF.

- * * * * End of changes * * * *