3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-200xxxx

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, evaluations and conclusions

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces evaluations on solutions of KI#1 and proposes a way forward for conclusions.

1. Introduction/Discussion

This part provides a comparison for solutions of KI#1.

For solutions that address the scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server:

| Principle | What it solves | System impacts | Pros/Cons | Solution |
| --- | --- | --- | --- | --- |
| Trigger Primary authentication and authorization towards AAA server using UDM indication | Determine the primary authentication and authorization towards AAA server based on UDM indication Obtain the external UE ID, which is used during primary authentication and authorization towards AAA server Select the AUSF that supports interacting with AAA server | Impacts on AUSF, UDM and AMF | Pros: well support of the scenario that UDM is involved in primary authentication and authorization Cons: | Sol#4, #8 |
| Trigger Primary authentication and authorization towards AAA server using local policy | Determine the primary authentication and authorization towards AAA server based on local policy Obtain the external UE ID, which is used during primary authentication and authorization towards AAA server Select the AUSF that supports interacting with AAA server | Impacts on AUSF and AMF | Pros: well support of the scenario that UDM is not involved in primary authentication and authorization Cons: | Sol#4, #8, #10 |
| UDM provides the subscription data for mobility management and session management | AMF or SMF obtain the subscription data from UDM using SUPI | No | Pros: reuse current mechanism Cons: | Sol#4, #8, #44 |
| UDM provides the subscription data for mobility management and session management | AMF or SMF obtain the subscription data from UDM using SI-SUPI | Impacts on UDM/AMF/SMF | Pros: Cons: | Sol#10 |
| AAA-P/AUSF provides the subscription data for mobility management and session management | AMF or SMF obtain the subscription data from AAA-P/AUSF using SUPI | Impacts on AAA-P/AMF/SMF | Pros: applicable in case no UDM is deployed at SNPN Cons: | Sol#4, #8 |
| AAA server provides the subscription data for mobility management and session management to UDM (via AUSF) | AAA server provides the subscription data for mobility management and session management to UDM (via AUSF) | Impacts on AUSF, UDM | Pros: Cons: | Sol#10, #44 |

For solutions that address the mobility scenario:

| Principle | What it solves | System impacts | Pros/Cons | Solution |
| --- | --- | --- | --- | --- |
| Consider Inter-PLMN like interworking | Considering the Inter-PLMN like interworking, use the PDU Session Establishment procedure with Existing PDU Session indication to address the session continuity for “Home Routed” PDU sessions | No | Pros: Cons: no support of session continuity for “LBO” PDU session, only consider the Inter-PLMN like interworking | Sol#1, #2, #41 |
| Consider both the Inter-PLMN like interworking and N3IWF based interworking | Considering all mobility cases to address the session continuity for “Home Routed” or “Local Breakout” the PDU sessions, use the PDU Session Establishment procedure with Existing PDU Session indication or use the Handover of a PDU Session procedure between 3GPP and untrusted non-3GPP access Procedure, or use the PDU Session Establishment procedure with Initial Request indication | Target network needs to determine a mobility indication considering the Inter-PLMN like interworking or N3IWF based interworking | Pros: comprehensive consideration for different mobility cases and support session continuity for “Home Routed” or “Local Breakout” the PDU sessions Cons: needs a mobility indication | Sol#41 |

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-07 V0.5.0.

# * * * * First change * * * *

7.1	Key Issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN

Editor's note:	This clause may provide a general evaluation and comparison of the solutions per Key Issue #1.

7.1.X	Evaluations for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server

This clause provides evaluations for the solutions that address the scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server (AAA-S), among these solutions:

-	Sol#4 proposes that

-	In the case that SNPN UDM does not provide any subscription data the primary authentication and authorization towards the AAA-S is triggered by AMF based on local policy, the AMF selects the AAA-P that supports the primary authentication and authorization towards the AAA-S. The AMF derives the external UE ID from the SUCI/SUPI or receives the external UE ID from UE. The AMF selects the AAA-P that supports the primary authentication and authorization towards the AAA-S. The AMF obtains the access and mobility subscription data from AAA-P/AAA-S to complete the registration procedures, the SMF obtains the session management subscription data from AAA-P/AAA-S to complete the session management procedures. This is applicable when UDM is not involved in the primary authentication and authorization towards the AAA-S, e.g., the UDM is not deploy in the SNPN.

-	In the case that SNPN UDM provides access and mobility subscription data, session management subscription data, the primary authentication and authorization towards the AAA-S is triggered by the UDM indication in subscription data. The AMF may re-select a AAA-P that supports the primary authentication and authorization towards the AAA-S based on the UDM indication. The AAA-P derives the external UE ID from the SUPI or receives the external UE ID from UE via AMF. The AMF obtains the access and mobility subscription data from UDM to complete the registration procedures, the SMF obtains the session management subscription data from UDM to complete the session management procedures. This is applicable when UDM is involved in the primary authentication and authorization towards the AAA-S.

-	Sol#8 proposes the similar mechanism as the Sol#4’s case that SNPN UDM provides subscription data for the UE, where CdP (Credentials Provider) replaces the AAA-S, and AAA-P is implemented by enhanced AUSF that supports delegation of authentication server role to the CdP via new NF supporting protocol interworking with CdP.

-	Sol#10 proposes the similar mechanism as the Sol#4’s case that the primary authentication and authorization towards the AAA-S is triggered by AMF based on local policy where the AMF derives the external UE ID from the SUCI/SUPI and uses this for the security procedures. In addition, the subscription data is pre-configured at UDM or dynamically provisioned at UDM on-demand. The AMF or SMF can obtain the UE subscription data from UDM using a UE Subscription Identifier (SI-SUPI), which can be generated by UDM/UDR or be provided by the AAA server. The SI-SUPI is new key for subscription data retrieval and has impacts on the procedural level.

-	Sol#44 proposes that the similar mechanism as the Sol#4’s case that the primary authentication and authorization towards the AAA-S is triggered by AMF based on local policy. In addition, the AAA Server can interact with SNPN UDM, supporting to provision the UE subscription data e.g., session management subscription data, access and mobility subscription data at UDM during registration procedure once the primary authentication and authorization at AAA-S succeeds. The direct interaction between AAA server and UDM has impacts on the system level.

7.1.Y	Evaluations for mobility scenarios

This clause provides evaluations for the solutions that address the mobility scenario, among the solutions below described:

-	Sol#1 and Sol#2 supports mobility scenario based on roaming architecture (i.e., Inter-PLMN like interworking) between the source/target network and the Home SP, making use the PDU Session Establishment procedure with Existing PDU Session indication as defined in TS 23.502 [6] clause 4.3.2.2 to address the session continuity for the PDU sessions anchored in the home SP (SNPN or PLMN). This only addresses the session continuity for “Home Routed” PDU sessions and only considers the Inter-PLMN like interworking for mobility cases.

-	Sol#41 supports mobility scenario based on roaming architecture, usage of N3IWF located in the network where PDU session is anchored (i.e. Home SP for HR and source SNPN for LBO) and no interconnection between source and target network. A new mobility indication is provided by network to UE during registration at the target network in order to indicate to UE which solution to be used for session mobility. This addresses session continuity for PDU sessions anchored at Home SP network and PDU sessions anchored at source considering all possible mobility cases.

# * * * * Second change * * * *

8.1	Key Issue #1: Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN

Editor's note:	This clause will capture conclusions for Key Issue #1.

8.1.X	Conclusions for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server

The scenario where the SNPN (identified with PLMN ID + NID) offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server (AAA-S) is supported. The primary authentication and authorization towards the AAA-S is used during the initial registration with the SNPN:

-	The AMF triggers this primary authentication and authorization based on local policy or UDM triggers this primary authentication and authorization based on UDM indication

-	The AMF selects the NF (new NF or enhanced AUSF) that supports the primary authentication and authorization towards the AAA-S based on local policy or UDM indication

NOTE 1:	whether to use a new NF or enhanced AUSF will be determined in normative stage.

-	The external UE ID used for this primary authentication and authorization is derived from the SUCI/SUPI or provided by the UE using a dedicated procedure

After completing the primary authentication and authorization successfully, the AMF or SMF can retrieve the individual subscription data (using SUPI) or common subscription data (using external UE ID) from the UDM or the selected NF to complete the registration and session management procedures respectively.

The subscription data in UDM may be pre-configured or dynamically provisioned by AAA-S on demand.

NOTE 2:	An architecture to support AAA interfaces to service provider (SNPN) will be confirmed by SA3.

8.1.Y	Conclusions for mobility scenarios

Mobility scenarios should support the following cases

1) 	a UE moving from SNPN#1 to SNPN#2 with PDU session anchored in the Home SP#1 and vice versa;

2)	a UE moving from SNPN#1 to SNPN#2 with PDU session anchored in the SNPN#1 and vice versa

3)	a UE moving between SNPN#1 and Home SP#1 with PDU session anchored in the Home SP#1 and vice versa.

4) 	a UE moving between SNPN#1 and Home SP#1 with PDU session anchored in the SNPN#1 and vice versa.

The mobility procedures are based on:

-	In the case that there are neither common AMF (i.e., Xn interface) nor N14 interface between the source network and target network, in order to support handover for PDU sessions anchored at Home SP#:

-	the UE performs the handover of the Home Routed PDU session using PDU Session Establishment procedure with Existing PDU Session indication as defined in TS 23.502 [6] clause 4.3.2.2.2 or Handover of a PDU Session procedure between 3GPP and untrusted non-3GPP access Procedure as defined in TS 23.502 [6] clause 4.9.2.3 or 4.9.2.4 that follows the PDU Session Establishment procedure at the target network as defined TS 23.502 [6] clause 4.3.2.2.1.

-	In the case that there are neither common AMF nor N14 interface between the source network and target network, in order to support handover for PDU sessions anchored at SNPN#1

-	the UE performs the handover of the Local Breakout PDU session using PDU Session Establishment procedure with Existing PDU Session indication as defined in TS 23.502 [6] clause 4.3.2.2.2 or Handover of a PDU Session procedure between 3GPP and untrusted non-3GPP access Procedure as defined in TS 23.502 [6] clause 4.9.2.1 or 4.9.2.2 that follows the PDU Session Establishment procedure at the target network as defined TS 23.502 [6] clause 4.3.2.2.1 or PDU Session Establishment procedure with Initial Request indication as defined in TS 23.502 [6] clause 4.3.2.2.1.

-	In the case that there are common AMF and/or N14 interface between the source network and target network, mechanism defined in TS 23.502 [6] clause 4.9.1 is re-used to address UE mobility.

The registration procedure is enhanced adding a mobility indication provided by target network to UE to indicate which is the mobility procedure to be used for session mobility, i.e. those for roaming architecture (inter-PLMN like architecture), usage of N3IWF located in the network where PDU session is anchored and no interconnection between source and target network.

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *