SA WG2 Meeting #141E (e-meeting)	S2-2006833r01

Oct 12 – Oct 23, 2020, Elbonia	(merger of S2-2006833 and S2-2006893)

Source:	CATT

Title:	KI #4 evaluation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the evaluation for Key Issue #4.

- Discussion

For Key Issue #4 "QoS level support for Multicast and Broadcast communication services", there are the following solutions in the TR:

-	Solution #3, #4, #17, #35, #36 and #37.

Evaluations are made on these solutions.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change (all new text) *****

7.x	Key Issue #4: QoS level support for Multicast and Broadcast communication services

Solution #3, #4, #17, #35, #36 and #37 are proposed to address Key Issue #4.

Solution #3 and #4 mainly address KI#1 "MBS session management", but also address the following aspects for KI#4:

- Multicast QoS flow(s) management within a multicast session context, and mapping to radio bearer.

- Association between multicast QoS flows and unicast QoS flows.

Solution #17 and #36 propose the QoS model and general handling (incl. policy provision, QoS enforcement, etc.) for 5G MBS services based on the QoS framework of TS 23.501 [2] clause 5.7. Both solutions propose that the PCF performs policy control for MBS (e.g. QoS control per MBS session) if dynamic PCC is deployed. Solution #36 also introduces new QoS parameter (i.e. MBS Session-AMBR) for MBS.

Solution #35 proposes that the AF can set specific QoS requirements (e.g. priority) for the MBS service data flows of a specific UE, which is sent to the 5GC and then forwarded to the NG-RAN.

Solution #37 proposes that 5GC and NG-RAN enforce MBS QoS by performing UEs and MBS QoS flow association based on UE interests in MBS services.

Following list illustrates the detail technical aspects from candidate solutions addressing KI#4 QoS level support.

Consideration on QoS control for MBS:

Proposal:	Support QoS control for MBS in PCF if dynamic PCC is deployed

Support QoS control per MB session

Support GBR and non-GBR

Alternatives:	Alt 1: Support AF specific priority for a specific member in a MB session (Sol#35)

Editor's note: Requirement of distinguish QoS for specific user of a multicast group needs to be confirmed.

Alt 2: Do not distinguish policy for members (Sol#3, 4, 17, 36)

Consideration on QoS feature:

Proposal:	Compare to QoS feature for PDU session, do not support QoS feature apply to UL traffic and single user and do not support QoS feature related to non-3GPP access

Reflective QoS is not applicable

Alternative QoS Profile is not applicable

Notification Control is not applicable

No UL Packet filter sets

Wireline access network specific 5G QoS parameters do not apply

Consideration on granularity of QoS control:

Proposal:	Support QoS control for MBS on QoS flow granularity

Alternatives:	Alt 1: Support one QoS flow for a MB session (Sol#3, 4)

Alt 2: Support multiple QoS flows for a MB session (Sol#3, 4)

Consideration on QoS control on multicast flow in a PDU session:

Proposal:	Support QoS control on multicast flow in a PDU session

Alternatives:	Alt 1: Uses different QoS flow for multicast IP flows and unicast IP flows (Sol#3, 4)

Alt 2: May use same QoS flow for multicast IP flows and unicast IP flows (Sol#4)

***** End of Changes *****