3GPP TSG-WG SA2 Meeting #141E e-meeting	S2-2007215

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3, Sol#24: Update on the configured QoS mapping

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This paper proposes to clarify the QoS handling on L3 U2N Relay.

# 1. Introduction/Discussion

This paper proposes to update Solution #24 on the configured QoS mapping.

In the current solution, the SMF supports modifying the PDB in the QoS Profile based on either PCC rules or pre-configuration.

If the SMF does not know the configured mapping on Relay UE, the enforced end-to-end delay may not align with end-to-end delay requirement. For example, the configured mapping on a Relay UE can be 80ms in 5QI to 40ms in PQI, and 70ms in 5QI to 30ms in PQI. For a specific service, its end-to-end delay requirement is 100ms. Without the knowledge of the Relay UE’s configured mapping of 5QIs and PQIs, SMF may modify the PDB in the QoS Profile to 80ms, and then Relay UE deduces the PC5 PDB to be 40ms based on the configured mapping. At last the enforced end-to-end delay is 120ms rather than 100ms. With the knowledge of the Relay UE’s configured mapping, the SMF may modify the PDB in the QoS Profile to 70ms, and then the enforced end-to-end delay can be 100ms.

Proposal 1: SMF should know the configured mapping on Relay UE.

In case the PC5 QoS Flows are initiated by the Remote UE, the Remote UE decides the PC5 part QoS parameters, the UE-to-Network Relay deduces the Uu part QoS parameters based on the 5QIs and PQIs mapping. If the Remote UE does not know the configured mapping on Relay UE, the enforced end-to-end delay may not align with end-to-end delay requirement of Remote UE. The reason is same as that in SMF part.

Proposal 2: The Remote UE should know the configured mapping on Relay UE.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

# * * * * First change * * * *

## 6.24	Solution #24: End-to-End QoS support for Layer-3 UE-to-Network Relay

### 6.24.1	General Description

This solution addresses Key Issue#3 " Support of UE-to-Network Relay ". Specifically, this solution addresses the aspects on " How to support end-to-end requirements between Remote UE and the network via a UE-to-Network Relay, including QoS (such as data rate, reliability, latency)" and " How the network allows and controls the QoS requirement for 5G ProSe UE-to-NW relay."

In Layer 3 UE-to-NW relay solution (Solution #6), the Remote UE's data flow is served by the Relay UE's PDU Session. As the UE-to-Network relay path comprises of two legs (PC5 and Uu) as shown in figure 6.24.1-1 below, the end-to-end QoS can be met only when the QoS requirements are properly split and satisfied over the two legs respectively.

Figure 6.24.1-1: End-to-End QoS split for Layer 3 UE-to-Network Relay solution

The QoS requirements on the PC5 link are controlled with PC5 QoS rules and PC5 QoS parameters (PQI, GFBR, MFBR, PC5 LINK-AMBR, Range, etc) as specified in clause 5.4 of TS 23.287 [5]. The QoS requirements on the Uu link are controlled via with 5G QoS rules and 5G QoS parameters (5QI, GFBR, MFBR, etc) as specified in clause 5.7 of TS 23.501 [6].

The Uu leg's QoS is associated with the PDU Session established by the UE-to-Network Relay, and therefore the procedure as defined in TS 23.502 [8] clause 4.3.2 and 4.3.3 applies. The SMF of the UE-to-Network Relay would provide the corresponding QoS rules and flow level QoS parameters to the UE-to-Network Relay.

As explained above, the UE-to-Network Relay needs to translate the Uu QoS information into the corresponding PC5 QoS parameters in order to achieve the proper end-to-end QoS. Since the Remote UE and the UE-to-Network Relay uses PC5 unicast communication mode, most of the flow level QoS parameters can be directly reused. The only parameter that requires assistance in the translation is the mapping of 5QIs and PQIs. It is therefore necessary that the UE-to-Network Relay to be configured with the proper mapping information. The mapping of 5QIs and PQIs are configured at UE-to-Network Relay for a specific service or for a group of services. The 5QI for Uu and PQI for PC5 are used together to support the end-to-end QoS requirement.

NOTE 1:	The service or the group of services can be identified by Relay Service Code, IP 3-tuple(s) etc.

In case the QoS Flows setup are initiated by network, PCF or SMF decides the Uu part QoS parameters. Based on this information received form SMF, the UE-to-Network Relay deduces the PC5 part QoS parameters and establishes corresponding PC5 QoS Flows, using the procedure defined in TS 23.287 [5] clause 6.3.3.4. For example, after receiving the QoS rules and flow level parameters, the Relay UE determines the corresponding PC5 QoS Flows to establish and the mapping between the Uu QoS Flows and the PC5 QoS Flows.

In case that the Remote UE requested dedicated PC5 QoS Flows when establishing the L2 Link over PC5, the Remote UE gets the configured mapping on the Relay UE via PC5 and decides the PC5 part QoS parameters based on the configured mapping, the UE-to-Network can map the PC5 QoS requirements into Uu QoS requirements and perform the UE requested PDU session Modification as defined in TS 23.502 [8] clause 4.3.3.

### 6.24.2	Enhancements to support dynamic QoS handling

As shown in figure 6.24.1-1, the end-to-end connection from the Remote UE to the AS involves two over-the-air links, i.e. Uu and PC5. Therefore, to meet the PDB for a particular service, the AN PDB utilized by the NG-RAN needs to be reduced, in order to give some budgets for the PC5 link. Note that this is independent of whether L2 or L3 Relay architecture is used.

One way to achieve this without affecting the NG-RAN is for the SMF to modify the PDB signalled to the NG-RAN in the QoS Profile for the QoS Flows of the Remote UE's traffic. SMF follows the PCC rules (if it is PCF determined) or based on local configuration to deduct the PDB.

When dynamic PCC control is supported, the SMF can base on the PCC rules to determine the PDB to use. Otherwise, SMF can base on pre-configuration, e.g. using DNN and/or S-NSSAI, to determine if and how to modify the PDB.

When dynamic PCC control is supported, it is possible that the AF may be able to request certain QoS handling of the traffic when the Remote UE initiated a session. This can be achieved by using the feature as defined in TS 23.503 [18] clause 6.1.3.22. The AF is able to locate the UE-to-Network Relay's PCF using the procedure as defined in TS 23.503 [18] clause 6.1.1.2, since the Remote UE uses an address belonging to the UE-to-Network Relay's PDU session.

The PCF can decide the Uu part QoS parameters and generate corresponding PCC rules, and the SMF in turn generate the QoS rules and flow level QoS parameters and signal to the UE-to-Network Relay using PDU Session Modification procedure. The UE-to-Network Relay then deduces the PC5 part QoS parameters based on the configured mapping of 5QIs and PQIs and uses the L2 Link Modification procedure defined in TS 23.287 [5] clause 6.3.3.4 to set up the related PC5 QoS flows.

NOTE:	As UE-to-Network Relay uses the configured mapping of 5QIs and PQIs to deduce the PC5 part QoS parameters, the end-to-end QoS requirements provided by AF which can't align with the configured mapping of 5QIs and PQIs is not supported in this solution.

In case of network scheduled operation mode for NR PC5 is used, procedures defined in TS 23.287 [5] clause 5.4.1.4 is used to authorize the PC5 QoS requests related to the relay operation.

Editor's note:	How to determine QoS parameters for PDU Session is FFS, such as which UE's subscription is used.

Alternatively, reflective QoS control over Uu as defined in TS 23.502 [8] clause 5.7.3.5 can be leveraged for dynamic QoS handling of remote UE. In particular, it does not require any explicit intervention from SMF. This can potentially save on signalling between SMF and UE-to-Network Relay UE to frequently modify the relaying PDU session over Uu.

Upon reception of a DL packet with RQI on the Uu for the remote UE, based on the QFI indicated, the UE-to-Network Relay may optionally derive a QoS rule, corresponding to the remote UE. The derived rule is for UL packets from the remote UE for which the new QFI (based on RQoS) is to be used.

To do so, the UE-to-Network Relay may determine the PQI based on the indicated 5QI on the DL Uu (due to Reflective QoS). The UE-to-Network Relay may locally associate the remote UE (i.e. the PC5 QoS flow with the remote UE) to the derived QoS rule.

The UE-to-Network Relay may then modify either the associated PC5 QoS flow or mapping between Uu and PC5 QoS flows towards the remote UE in order to match the PQI of the PC5 flow to the indicated 5QI over the DL Uu. The UE-to-Network Relay then uses e.g. the L2 Link Modification procedures as defined in TS 23.287 [5] clause 6.3.3.4 to modify the related PC5 QoS flows.

When the UE-to-Network relay deletes the derived QoS rule e.g. after the RQ Timer expires, the UE-to-Network Relay resumes back to use the signalled QoS rule and performs L2 Link Modification procedures defined in TS 23.287 [5] clause 6.3.3.4 accordingly to use the PQI that corresponds to the 5QI of the existing signalled QoS rule.

Editor's note:	It is FFS how to activate the reflective QoS control for UE-to-Network Relay.

Editor's note:	Whether the UE-to-Network Relay needs to modify the mapping between Uu and PC5 QoS flows based on the DL packet with RQI is FFS.

### 6.24.2	Procedures

Existing procedures defined in TS 23.502 [8] and TS 23.287 [5] can be used to manage the QoS flows and PC5 QoS flows to serve the Remote UE.

### 6.24.3	Impacts on services, entities and interfaces

The solution has impacts in the following entities:

SMF:

-	SMF optionally supports modifying the PDB for a QoS Flow serving the Remote UE based on either PCC rules or pre-configuration.

UE:

-	5G ProSe UE-to-Network Relay supports the mapping of Uu flow level QoS parameters to PC5 QoS parameters, including the mapping of 5QIs to PQIs, based on configuration.

-	5G ProSe UE-to-Network Relay modifies the PQI of the PC5 link to match the QFI of the derived QoS rule.

-	Remote UE supports to decide the PC5 part QoS parameters based on the E2E QoS parameters.

-	Remote UE gets the configured mapping on UE-to-Network Relay.

PCF:

-	supports to decide the Uu part QoS parameters based on the E2E QoS parameters.

# * * * * End of changes * * * *