SA WG2 Meeting #141E (e-meeting)	S2-2007716r06

Elbonia, October 12 – 23, 2020

Source:	vivo Mobile Communications Ltd, LG Electronics

Title:	Interim conclusions

Document for:	Discussion/Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2/Rel.17

Abstract of the contribution: This contribution adds some interim highlevel conclusions.

- Background

Solution#1 to Solution#6 in TR 23.776 are about NR PC5 DRX operations. It is proposed to capture some interim conclusions based on these solutions.

.

* * * * Start of Change * * * *

7	Conclusions

For Key Issue #1 (Support of QoS aware NR PC5 power efficiency for pedestrian UEs), regarding NR PC5 DRX operations the following principles are taken as the interim conclusion:

- 	The V2X application layer provides inputs for V2X layer to determines PC5 DRX assistance information

Editor's note:	It is FFS whether the inputs described in TS 23.287 [5] (e.g. V2X Application Requirements) are sufficient or additional inputs (e.g. traffic pattern) are needed from the V2X application layer.

-	The V2X layer may provide the PC5 DRX assistance information to the AS layer.

Editor's note:	It is whether PC5 DRX assistance information, which is provided to the AS layer by the V2X layer, is defined in 3GPP and what is the content of the PC5 DRX assistance information.

Editor's note:	It is FFS if the PC5 DRX assistance information is per mode of communication (i.e. unicast/broadcast/groupcast).

-	The Access Stratum (AS) layer determines the PC5 DRX for V2X communication over PC5 reference point to enable pedestrian UE power saving.

-	The AS layer provides the applied PC5 DRX information to the V2X layer.

-	For unicast, the PC5 DRX is negotiated between the UEs in AS layer, pending on the feedback from RAN2.

Editor's note: 	The design principles need to be evaluated by RAN WGs.

Editor’s Note:	It is FFS whether DRX related parameters are needed and what is the contents, which is (pre-)configured on the UE to assist the V2X layer determine the PC5 DRX assistance information.

* * * * End of Change * * * *