SA WG2 Meeting #143E	S2-2100304r19

Feb 24 – Mar 9, 2021, Electronic

Source:	Futurewei, Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Cisco

Title:	UE Considerations for EAS rediscovery

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution introduces considerations on OS/user for EAS (re-)discovery.

# 1	Discussion

The case that OS/user overrides operator DNS setting is part of the considerations in TR clause 9.1.1. The new text proposed below captures the recommendations and limitations in Annex X.

# 2	Proposal

It is proposed to adopt the following changes into TS23.548.]

* * * * 1st Change * * * * all text is new

# Annex X (informative): UE considerations for EAS (re)discovery

## X.1	General

DNS records obtained from a network resolver contains a time-to-live (TTL) value. This is a hint provided by the network resolver and can be used to determine the length of time that the record is cached. DNS records can be cached in the UE by a system wide stub resolver and by application layer name resolution caches. The application (L7) cache is managed on a per application basis while the OS/system DNS cache is common to applications. Name resolution caches in various applications also have different policies and behaviour. Some applications cache the name records for the length of the application session while others have a time limit.

Editor’s Note: Whether coordination across the 5GC and application domain for name resolver caching behaviour is required or not is FFS.

The following clauses describe the appropriate DNS configuration for the EAS (re)-discovery to work in the UE.

## X.2	Impact of IP Addresses for DNS resolver

The UE can be configured by the 5GC with an IP address for the DNS resolver using ePCO or IPv6 Router Advertisement (RA). 5GC can reconfigure the DNS resolver IP address. In case of anycast IP address of the DNS resolver, the 5GC can use UL-CL/BP to branch out and the DN is responsible to route to the closest instance of the MNO DNS resolver without having to reconfigure the DNS resolver IP address in the UE.

Applications in the UE can request the DNS resolver configured on the UE to resolve an FQDN. However, applications can also be configured with their own DNS resolver address and can use encrypted messaging based e.g., on DNS over HTTPS (DoH) or, DNS over TLS (DoT). Configuration of application DNS resolvers is out of scope of 5GC. DNS messages delivered over DoT, or DoH might be forwarded transparently to the destination address of DNS resolver in the DNS query. The application DNS resolver can be operated by the 5GC operator or by a third party.

A network interface change or NAS SM EAS rediscovery indication can result in the UE OS clearing name/IP address translations in its DNS cache. If network interface change or NAS SM EAS rediscovery indication does not result in the UE OS clearing name/IP address translations in its DNS cache, an application can continue the L4 connection with the old EAS IP address until DNS cache entry times-out and subsequent DNS EAS address resolution request.

## X.3	UE considerations for EAS re-discovery

An application in the UE that complies with EAS (re-)discovery described in this specification is not recommended to override operator-provided DNS settings.

The OS DNS server configuration does not override the operator provided DNS in a UE compliant to the EAS (re-)discovery procedure. This is necessary for the “closest” EAS server to be selected.

NOTE 1:	If an OS, user or applications override the operator-provided DNS settings, the DNS resolvers or servers in the third party can take the source IP address of the DNS request as the location information of UE, which can correspond to the remote PSA UPF or other entities (e.g., a NAT server) on the remote/central N6 interface which can lead to a non optimal choice of the EAS server address.

NOTE 2:	If the DNS server configuration in a OS overrides the operator provided DNS, the DNS queries continue to be sent over the correct PDU session for the application.

NOTE 3:	The UE modem transparently forwards DNS messages for tethered devices that are loosely coupled.

NOTE 4: 	If the UE (OS or application) uses a DNS resolver that is different than the one provided by the 5GC, then:

-	the session breakout connectivity mode, option A and B in 6.2.3.2 will not work in case the EASDF is NOT in the DNS resolver chain for recursive DNS resolution.

## X.4	UE procedures for session breakout

In the session breakout connectivity model, the selection of a new session breakout path does not result in a new network interface indication at the UE.

NOTE 1:	In case of multiple sessions or distributed anchor point connectivity models, when there is a change of network interface, indication of network interface change can be used to flush the UE OS DNS cache.

Session breakout results in a NAS SM message indicating the need to redo DNS lookup sent by the SMF to the UE modem. Thus, in order to support some solutions of this TS, it is necessary for the operating system to receive information of EAS rediscovery from the modem when such signalling has been received and clear the DNS cache in UE OS.

* * * * End of Changes * * * *