SA WG2 Meeting #143E e-meeting	S2-2100947r02

Feb 24 – Mar 09, 2021, Elbonia	(revision of S2-21xxxxx)

Source:	Qualcomm Incorporated

Title:	TS 23.304: Clause 7.1 5G DDNMF Services

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution provides adds the 5G DDNMF services definition into TS 23.304.

1.	Introduction

Based on the KI#1 conclusion of TR 23.752, this contribution added 5G DDNMF Services definition into TS 23.304 as clause 7.1.

2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

7.1	5G DDNMF Services

7.1.1	General

The following table illustrates the 5G DDNMF Services.

Table 7.1.1-1: Services provided by 5G DDNMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| N5g-ddnmf_Discovery | AnnounceAuthorize | Request/Response | 5G DDNMF |
|  | AnnounceUpdate | Request/Response | 5G DDNMF |
|  | MonitorAuthorize | Request/Response | 5G DDNMF |
|  | MonitorUpdate | Request/Response | 5G DDNMF |
|  | MonitorUpdateResult | Notify | 5G DDNMF |
|  | DiscoveryAuthorize | Request/Response | 5G DDNMF |
|  | MatchReport | Request/Response | 5G DDNMF |
|  | MatchInformation | Notify | 5G DDNMF |

7.1.2	N5g-ddnmf_Discovery service

7.1.2.1	General

Service description: This service enables a 5G DDNMF to manage inter-PLMN ProSe Direct Discovery operations.

7.1.2.2	N5g-ddnmf_Discovery_AnnounceAuthorize service operation

Service operation name: N5g-ddnmf_Discovery_AnnounceAuthorize

Description: The consumer NF obtains the authorization from the 5G DDNMF for announcing in the PLMN.

Input, Required: Discovery type ("open" or " restricted ") and

-	(for "open" discovery type:) ProSe Application ID, ProSe Application Code, UE Identity, validity timer, Discovery Entry ID,

-	(for "restricted" discovery type:) RPAUID, Application ID, ProSe Restricted Code/Prefix, UE Identity, Discovery Entry ID,

Input, Optional: metadata, Restricted Code Suffix pool.

Output, Required: authorization result.

Output, Optional: None.

7.1.2.3	N5g-ddnmf_Discovery_AnnounceUpdate service operation

Service operation name: N5g-ddnmf_Discovery_AnnounceUpdate

Description: The consumer NF updates or revoke the authorization from the 5G DDNMF for announcing in the PLMN.

Input, Required: Discovery type = "open", UE Identity, validity timer, Discovery Entry ID

Input, Optional: ProSe Application Code

Output, Required: result.

Output, Optional: None.

7.1.2.4	N5g-ddnmf_Discovery_MonitorAuthorize service operation

Service operation name: N5g-ddnmf_Discovery_MonitorAuthorize

Description: The consumer NF obtains the authorization from the 5G DDNMF for monitoring in the PLMN.

Input, Required: Discovery type ("open" or " restricted ") and

-	(for "open" discovery type:) ProSe Application ID Name(s), UE Identity, Discovery Entry ID;

-	(for "restricted" discovery type:) RPAUID, UE Identity, Target PDUID, Application ID, Target RPAUID, Discovery Entry ID,

Input, Optional: None,

Output, Required: (for "open" discovery) ProSe Application Code(s)/Prefix, ProSe Application Mask(s), TTL; or (for "restricted" discovery) ProSe Restricted Code, validity timer

Output, Optional: None.

7.1.2.5	N5g-ddnmf_Discovery_MonitorUpdate service operation

Service operation name: N5g-ddnmf_Discovery_MonitorUpdate

Description: The consumer NF updates or revoke the authorization for the indicated UE to monitor in the PLMN.

Input, Required: Discovery type ("open" or " restricted ") and,

-	(for "open" discovery type:) ProSe Application ID name, UE Identity, TTL, Discovery Entry ID

-	(for "restricted" discovery type:) ProSe Restricted Code, Application ID, Banned RPAUID, Banned PDUID

Input, Optional: None

Output, Required: result

Output, Optional: None

7.1.2.6	N5g-ddnmf_Discovery_MonitorUpdateResult service operation

Service operation name: N5g-ddnmf_Discovery_MonitorUpdateResult

Description: The consumer NF informs the 5G DDNMF of the monitoring revocation results.

Input, Required: Discovery type = "restricted", ProSe Restricted Code, Application ID, Banned RPAUID, Banned  PDUID, Result

Input, Optional: None

Output, Required: None

Output, Optional: None

7.1.2.7	N5g-ddnmf_Discovery_DiscoveryAuthorize service operation

Service operation name: N5g-ddnmf_Discovery_DiscoveryAuthorize

Description: The consumer NF obtains the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery.

Input, Required: Discovery type = "restricted", Restricted ProSe App User ID, UE Identity, Target PDUID, Application ID, Target RPAUID, Discovery Entry ID,

Input, Optional: None

Output, Required: ProSe Query Code(s), ProSe Response Code, validity timer,

Output, Optional: None

7.1.2.8	N5g-ddnmf_Discovery_MatchReport service operation

Service operation name: N5g-ddnmf_Discovery_MatchReport

Description: The consumer NF obtains the information about the indicated discovery code from the 5G DDNMF.

Input, Required: Discovery type = "open", ProSe Application Code(s), UE identity, Monitored PLMN ID

Input, Optional: None

Output, Required: ProSe Application ID Name(s), validity timer(s).

Output, Optional: Metadata, Metadata Index Mask(s)

7.1.2.9	N5g-ddnmf_Discovery_MatchInformation service operation

Service operation name: N5g-ddnmf_Discovery_MatchInformation

Description: The consumer NF receives from the 5G DDNMF of a matching result, and the information can be used for charging purpose.

Input, Required: Discovery type ("open" or " restricted ") and,

-	(for "open" type:) ProSe Application ID(s), UE Identity

-	(for "restricted" type:) RPAUID, Target RPAUID, UE Identity, ProSe Restricted Code

Input, Optional: None

Output, Required: None

Output, Optional: None

# * * * * End of changes * * * *