3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102467r06

Elbonia, Apr 12 – Apr 16, 2021	(revision of S2-210XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0289 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of EN related to network slice association information in TS 23.288. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Remove FFS on the need to work on Cell granularity: Editor’s Note: whether the need for cell level granularity is FFS. Remove the following FFS on how NWDAF can retrieve the network slice association information: Editor’s Note: It is FFS to verify with SA5 how NWDAF can retrieve from OAM the network slice association information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes proposed in this CR represent the implementation of Alternative #2 from the discussion paper S2-2102466. Changes to solve first FFS: The cell level granularity is needed because the following analytics IDs defined in TS 23.288 allow the subscription to analytics ID to have as analytics filter per Cell ID for an area of interest and S-NSSAIs. These two information are used to restrict the focus of the analytics output and therefore NWDAF needs to consider data from the data sources that is matching only such criteria. The following analytics ID are the ones that require input data from AMF and/or OAM for the AoI per Cell granularity and for S-NSSAI specified in the analytics filter information: - Clause 6.8.1 – User Data Congestion Analytics - Clause 6.9.1 – QoS Sustainability - Clause 6.10.1 – Dispersion Analytics The proposed change is to insert back the reference to cell as being part of the network slice association information. Changes to solve second FFS: NWDAF derives the network slice association information by retrieving information AMF and using the Cellx TA mapping configured by OAM. The AMF has the mapping of its associated TAs, and per TA the access type and the list of restricted/unrestricted S-NSSAIs per PLMN. As per analysis of TS 23.502 Clause 4.10 (NG-RAN Location Reporting Procedure), it has been identified that AMF may not have the **updated mapping of TA x Cells**. AMF has the mapping of TA x Cells for the UEs it subscribe to receive the precise location information from NG-RAN. Therefore, we propose that the mapping of all Cells x TA is configured at NWDAF. Furthermore, we propose that the is no need to perform further checks with SA5 because: - As per TS 28.541 the OAM has the information of cellx TA. - OAM can simply configure this information in NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definitions for retrieval of network slice association information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 23.501 CR2758, TS 23.502 CR2651 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.2.1	General

The Data Collection from NFs is used by NWDAF to subscribe/unsubscribe at any 5GC NF to be notified for data on a set of events.

The Data Collection from NFs is based on the services of AMF, SMF, UDM, PCF, NRF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in TS 23.502 [3] clause 4.17)

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability), and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Table 6.2.2.1-1: NF Services consumed by NWDAF for data collection

| Service producer | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure (NOTE 3) | 5.2.2.3 5.2.3.5 |
| SMF | Nsmf_EventExposure (NOTE 3) | 5.2.8.3 5.2.3.5 |
| PCF | Npcf_EventExposure (for a group of UEs or any UE) Npcf_PolicyAuthorization_Subscribe (for a specific UE) | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| AF | Naf_EventExposure | 5.2.19.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |

NOTE 1:	The present document specifies that NWDAF can collect some UPF input data for deriving analytics, but how NWDAF collects these UPF input data is not defined in this Release of the specification.

NOTE 2:	There is no data collected from the PCF by the NWDAF defined in this Release of the specification.

NOTE 3:	The Nudm_EventExposure can be used when NWDAF uses the procedures specified in TS 23.502 [3] clause 4.15.4.3 to subscribe to AMF or SMF via UDM.

To retrieve data related to a specific UE, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

The UDM instance should be determined using NRF as described in clause 4.17.4 of TS 23.502 [3] and factors to determine as described in clause 6.3.8 of TS 23.501 [2].

The AMF, SMF instances should be determined using a request to UDM providing the SUPI or the group identity. To determine the SMF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple SMFs (e.g. one per PDU session).

The BSF instance should be discovered using NRF thanks to optional request parameters (e.g. DNN list, IP domain list, IPv4 address range, IPv6 prefix range) as stated in clause 4.17.4 of TS 23.502 [3], or based on local configuration at the NWDAF.

The PCF instance serving UE PDU Session(s) should be determined using a request to BSF with the allocated UE address, DNN and S-NSSAI.

When NWDAF receives a request addressed to an Internal Group ID from a consumer, NWDAF may need to initiate data collection from several 5GC NFs, such as AMF, SMF, UDM, PCF, NEF/AF, etc. NWDAF may first discover the instances of the required 5GC NFs deployed in the network, e.g. by querying NRF.

For discovering the UDM, NWDAF can query the NRF with the Internal Group ID as the target of the query. For discovering AMF, SMF, PCF, NEF, and AF, NWDAF may need to discover all the instances in the network by using the Nnrf_NFDiscovery service.

NOTE 4:	It is assumed that all members of an Internal Group ID belong to the same UDM Group ID. NWDAF can select a UDM instance supporting the UDM Group ID of the Internal Group ID.

Then, if data needs to be collected from AMF, SMF, UDM, and PCF, NWDAF may initiate the data collection with the Internal Group ID as the target, e.g. subscribing to the event exposure in all the instances of a given type of network function. This subscription to all the instances of required source of event exposure handles, e.g. mobility of UEs across AMFs, or initiation of new PDU sessions with different allocated SMFs.

For collecting data from AMF and SMF, NWDAF may collect the data directly from AMF and/or SMF, or indirectly via UDM, according to TS 23.502 [3] clause 4.15.4.3. The indirect method may be required if the event exposure subscription from NWDAF, for a UE or group of UEs, needs to survive the removal of UE context in the AMF including event exposure subscriptions, or upon the creation of new UE context in AMF or SMF serving the UE or group of UEs. In this case the UDM is responsible for (re)creating event exposure subscriptions in AMF and SMF, as specified in TS 23.502 [3], clause 4.15.4.3.

The NWDAF determines to collect data from a trusted AF supporting specific Event ID(s) and serving specific application(s) based on internal configuration.

The NEF instance that is serving a specific network slices and/or applications of a UE should be determined using NRF using optional request parameters as defined in clause 6.3.14 of TS 23.501 [2]

If NWDAF needs to collect data from an AF deployed outside the operator's domain, the NWDAF shall contact NEF with a SUPI or Internal Group ID as the target of the data collection. NEF is responsible for translation of SUPI to GPSI, or internal to external group identifiers, by querying UDM, prior to contacting the AF.

To retrieve required data for any UE, the NWDAF may subscribe to events from the AMF and/or SMF instances it has determined, setting the target of event reporting to "any UE" and the event filter(s) according to the Analytics Filter Information. Alternatively, if the required data is communication related and for any UE within an Area of interest, the NWDAF can obtain from the AMF instances it has determined a list of UEs located within the Area of Interest. Based on the obtained UE list, for each UE in the list, the NWDAF retrieves the SMF serving the UE and the NWDAF subscribes to data from the relevant SMF per each specific UE. The indirect event exposure subscription to AMF or SMF via UDM is not available for "any UE" or "any UE within an Area of interest".

NOTE 5:	If NWDAF requires collecting data from either AMF or SMF for "any UE" or "any UE within an Area of Interest", NWDAF can use the direct Event Exposure subscription to AMF or SMF, since subscriptions to "any UE" or "any UE within an Area of Interest" are persistent by nature in AMF or SMF, due to not being linked to a UE context.

To retrieve data related to "any UE" based on analytics filter information, the NWDAF shall first determine which NF instances are matching the analytics filter information (see clause 6.7.5.1) as stated in table 6.2.2.1-3 unless the NWDAF has already obtained this information due to recent operations related to this analytics filter information.

Table 6.2.2.1-3: NF Services consumed by NWDAF to determine which NF instances are matching analytics filters

| Type of NF instance (matching analytics filters) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| AMF, SMF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

To retrieve data related to analytics IDs for "any UE" with analytics filter information defining an area of interest in terms of TA or Cells and/or with specific S-NSSAIs, NWDAF requires the network slice association information to properly determine the AMFs to collect data from as well as the proper queries to OAM for data collection.

NOTE 6:	Examples of analytics ID requiring NWDAF to use network slice association information for data retrieval are: network performance clause 6.6.1; user data congestion clause 6.8.1, QoS Sustainability clause 6.9, Dispersion Analytics Clause 6.10.1; observed service experience for a Network Slice 6.4.1, and slice load analytics 6.3.

The network slice association information comprises the TAs associated with each AMF, and for each TA its associated access type, cells, and list of restricted or unrestricted S-NSSAIs per PLMN. Additionally, the mapping of cells per TA and restricted or unrestricted S-NSSAIs per PLMN for TA for each AMF can change and NWDAF shall obtain this accurate information in order to properly retrieve data for analytics generation.

In order to derive the network slice association information, NWDAF may be configured with the mapping of cells per TAI and subscribes to the "S-NSSAIs per TA mapping" event exposed by AMF. The NWDAF may use the configured information (when the subscription or request is per Cell granularity) and the area of interest in the subscription to retrieve from AMF the list of restricted or unrestricted S-NSSAIs per PLMN for TA, and access types per TA for each AMF in the required area of interest. NWDAF consumes the "S-NSSAIs per TA mapping" event exposed by AMF using as target of event reporting the list of TAIs based on the TA received in the analytics filter information or identified by the mapping of the Cells per TA matching to the Cell granularity included in the analytics filter information. The AMF "S-NSSAIs per TA mapping" event output contains each TAI associated with AMF, and for each TAI its associated access type and list of restricted or unrestricted S-NSSAIs per PLMN.

To retrieve data from SMFs for analytics IDs subscription or requests for "any UE" including analytics filter information with specific Applications, DNNs, DNAIs and area of interest per TA granularity, NWDAF shall first discover the SMF serving the area of interest via NRF.

NWDAF may directly consume events from the discovered SMF using the event target set to "any PDU session", and event filters with the same parameters of the analytics filter information, i.e. list of Application IDs, and/or DNNs, and/or DNAI, and the area of interest related to the requested analytics ID.

1.	SMF may support the exchange of UE Location parameter when SMF interacts with AMF via Nsmf_PDUSession_Create / Update / CreateSMContext / UpdateSMContext due to session establishment, modification, or release, service request, or handover procedures

If there are any changes in PDU sessions in the area of interest, for the Application ID, and/or DNN, and/or DNAI subscribed by NWDAF, SMF notifies the detected changes to NWDAF via Nmsf_EventExposure_Notify service operation, enabling NWDAF to keep an updated map of SMF and PDU sessions associated with the analytics filter information in an area of interest.

Editor's note:	Whether the need for an SMF mapping the PDU Session association with TA or NWDAF performs such mapping is FFS.

2.	SMF may also subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 from TS 23.502 [3] to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. Based on the retrieved list of SUPIs in the area of interest, SMF identifies the PDU sessions in the area of interest.

Editor's note:	Whether the same discovery mechanism shall be supported by DCCF and whether further alignments are required is FFS.

# * * * * End of changes * * * *