SA WG2 Meeting #S2-144E	S2-2102783r06

12 April - 16 April 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0565 | rev | - | Current version: | 17.0.0. |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4-T3, Enabling restricted PDU Session for remote provisioning of UE using User Plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the user plane remote provisioning for SNPN credentials, it has been agreed, - A restricted PDU session is supported to be dedicated for the remote provisioning. Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address via any of the methods listed below, the device uses a well-known FQDN to perform PS discovery. - When Onboarding network is O-SNPN, the information required to restrict the usage is locally configured in the SMF, and the SMF restricts the usage for the UE registered for onboarding. - When Onboarding network is a PLMN, the functionality to restrict usage is activated for the UE by SMF based on received existing subscription profile from the UDM and on existing policies from PCF, or local configuration. This CR implement this to 23.503 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding SNPN abbreviation, clarify PCC rule for SNPN UE remote provisioning. And adding a new clause on SNPN UE Remote Provisionning support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SNPN UE Remote Provisionning support is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.1.3.2.3, 6.1.3.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.316 [27] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AMBR	Aggregated Maximum Bitrate

ANDSP	Access Network Discovery & Selection Policy

ARP	Allocation and Retention Priority

ASP	Application Service Provider

BDT	Background Data Transfer

BSF	Binding Support Function

CHF	CHarging Function

H-PCF	A PCF in the HPLMN

H-UDR	A UDR in the HPLMN

MPS	Multimedia Priority Service

NBIFOM	Network-based IP flow mobility

NSWO	Non-Seamless WLAN Offload

NWDAF	Network Data Analytics Function

OAM	Operation Administration and Maintenance

OCS	Online Charging System

PCC	Policy and Charging Control

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

PRA	Presence Reporting Area

ProSe	Proximity based Services

ProSeP	ProSe Policy

RAN	Radio Access Network

SNPN	Stand-alone Non-Public Network

URSP	UE Route Selection Policy

V2XP	V2X Policy

V-PCF	A PCF in the VPLMN

V-UDR	A UDR in the VPLMN

vSRVCC	video Single Radio Voice Call Continuity

WLANSP	WLAN Selection Policy

Next CHANGE

##### 6.1.3.2.3	PCC rule authorization

PCC Rule authorization is the selection of the 5G QoS parameters, described in TS 23.501 [2] clause 5.7.2, for the PCC rules.

The PCF shall perform the PCC rule authorization for dynamic PCC rules belonging to AF sessions that have been selected in step 1, as described in clause 6.1.3.2.2, as well as for PCC rules without corresponding AF sessions.

For the authorization of a PCC rule the PCF shall consider any 5GC specific restrictions, subscription information and other information available to the PCF. Each PCC rule receives a set of QoS parameters that are supported by the specific Access Network.

The authorization of a PCC rule associated with an emergency service shall be supported without subscription information. The PCF shall apply local policies configured for the emergency service.

The authorization of a PCC rule used for SNPN UE remote provisioning via user plane shall be supported without subscription information. The PCF shall apply policies for support of SNPN UE remote provisioning as described in clause 6.1.3.X.

Next CHANGE

#### 6.1.3.X	SNPN UE Remote Provisioning support via User Plane

SNPN UE remote provisioning support via user plane is specified in 3GPP TS 23.501 [2] clause 5.30.

SNPN UE remote provisioning is a network service provided through a DNN and S-NSSAI used for UE onboarding.

For a PDU Session with a DNN and S-NSSAI used for UE onboarding, the PCF may make authorization and policy decisions that restrict the use of the PDU Session, e.g., by restricting the traffic to/from Provisioning Server address(es) and DNS server address(es) only.

For a PDU Session established to the DNN and S-NSSAI used for onboarding, the PCF does not perform a subscription check. Instead, the PCF uses the Onboarding Configuration Data for this DNN and S-NSSAI combination to make authorization and policy decisions.

The PCF sets the SDF template of the PCC rule(s) according to the Onboarding Configuration Data for this DNN and S-NSSAI combination that may include the Provisioning Server address(es) or DNS server address(es). The PCC Rule Authorization function selects QoS parameters applicable to the SNPN UE remote provisioning service. Then, the PCF installs these PCC Rules at the SMF to enable traffic to/from the dedicated IP addresses (i.e. Provisioning Server address(es) and DNS server address(es)) with the associated QoS.

The PCC Rules provided by the PCF take precedence over the locally stored policy used for the restricted PDU session at the SMF.

Editor’s Note: 	It is FFS if PCF obtains the Onboarding configuration data from other NF, e.g. from UDR and how the procedure would work,

END OF CHANGES