SA WG2 Meeting #147E	S2-2107942

October 18 – 22, 2021, E-Meeting

# Title:	Reply LS on NSAC procedure

# Response to:	(S2-2107062/ S3-213036)

# Release:	Rel-17

# Work Item:	Study on enhanced security for network slicing Phase 2 (FS_eNS2_SEC)

Source:	SA2

To:	SA3

Cc:	-

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks SA3 for the LS on NSAC procedure.

SA3 asked following questions:

1.	Once a UE is registered to a slice, it is always counted in NSACF quota even if the UE never uses the slice. This may cause the slice to be underutilized especially when many legitimate/malicious UEs register to many slices.

-	One potential solution is to consider the UE usage of a slice, e.g. whether there is any PDU session established. Is it feasible?

SA2 answer:

SA2 would like to indicate that the UE usage of a user services is not considered when counting the number of UEs registered to a network slice,

2.	SA3 has identified some unclear aspects when the NSSAA, the NSAC, with the EAC feature enabled, and the EAC procedures interact with each other. SA3 would appreciate if these aspects are clarified:

-	Assuming NSSAA is executed before NSAC, then if NSSAA is successful but NSACF quota has been reached, UE will be rejected, is this assumption correct? When UE registers again, does NSSAA need to be performed again?

SA2 answer:

The NSSAA status is kept in the UE context in AMF (also transferred between AMFs), i.e. the AMF may avoid NSSAA when NSSAA has been successful for an S-NSSAI.

If none of the S-NSSAIs of Requested can be registered due to NSAC, then AMF may decide to include Default Subscribed S-NSSAIs in the Allowed NSSAI, if they are not subject for NSAC.

-	A burst in the number of UEs attempting to register under EAC mode inactive may cause potentially circumvention of access/admission control in race condition. Is there an issue with the EAC procedure upon a burst of UE registrations?

SA2 answer:

If needed, the operator may activate EAC mode when the current count is reaching certain operator defined threshold.

2. Actions:

To SA3:

ACTION: 	SA2 asks SA3 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021		Elbonia

TSG-SA2 Meeting #149E	February 14 – 25, 2022		Elbonia