SA WG2 Meeting #S2-152E	S2-2205640

17 August – 26 August 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	Futurewei

Title:	KI #4, 5 Criteria and Principles for Evaluation

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM /Rel-18

Abstract of the contribution: This contribution provides some general criteria for evaluating KI #4, 5.

# 1	Discussion

This contribution considers some general evaluation criteria for KI 4, 5 that are captured in several solutions.

# 2	Proposal

It is proposed to adopt the following changes into TS23.700-60.

* * * * 1st Change (all new text) * * * *

# 7	Overall Evaluation

Editor's note:	This clause provides evaluations of different solutions.

## 7.x	Evaluation of solutions for Key Issues 4, 5

### 7.x.1	General Evaluation Criteria

The following criteria are used for evaluation of KI#4, #5 solutions:

- Application packets/ QoS flows that originate or terminate across multiple end-devices (i.e., UEs, servers) are supported. Media packet handling rules between AF – 5GC may be provisioned on a per-application /domain basis (e.g., a range of IP addresses representing a set of AS) or on a per-flow basis when the application domain selects handling preferences after PDU session establishment.

- Application domain (AF) may provision rules in 5GC for specific preferences to classify media packets from that domain (e.g., size of media packets, preference for high-speed motion for some applications, spatial quality for some others). If no preference is provided, default media handling rules provisioned by SMF in the UPF are used.

- Classification of media transported over RTP, SRTP and HTTP are supported as these protocols are used extensively in current and emerging low latency media /application transports. Media packets may be unencrypted, encrypted with visible media header, or fully encrypted based on various application development and deployment factors including privacy considerations.