3GPP TSG-WG SA2 Meeting #153e	S2-2208999

E-meeting, October 10-17, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI #3, conclusion update for clarification.

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This contribution proposes to update KI#3 conclusion.

1. Background

L4S is one of promising end-to-end congestion control mechanism and getting more supported. However, there are still chances to fail to connect due to non-capable interims routers. One statistics suggested the possibility of negotiation failures [1].

Figure 1. Scenario on interim routers resetting ECN field

[1] C. -X. Chen, "Measuring the state of ECN on the Internet," 2017 International Conference on Information and Communication Technology Convergence (ICTC), 2017, pp. 172-177, doi: 10.1109/ICTC.2017.8190967.

However, it is feasible that the operators are able to guarantee the certain data network’s deployment (e.g. edge computing) to avoid such a scenario.

The author of this paper believes that Rel-18 congestion notification with control plane may be enhanced to help operators handle such a scenario.

2. Proposal

It is proposed to update KI#3 conclusion as clause 8.1 of TR 23.700-60 v1.1.0.

# * * * * 1st change * * * *

8.1	Conclusions for Key issue#3: 5GS information exposure for XR/media Enhancements

The following bullet points summarize the principles for the way forward to support current congestion level information exposure:

-	5G System may use ECN marking for the purpose of Low Latency, Low Loss and Scalable Throughput services L4S according to [37] and [62] for uplink or and downlink QoS Flows via one of the following two methods.

-	To support L4S, NG-RAN performs ECN marking according to [37] and [62] for uplink and downlink in IP layer of the received packets.

NOTE 1:	The criteria for RAN to determine (e.g. its congestion level) when to perform the marking is up to RAN implementation.

-	PSA UPF performs ECN marking according to [37] and [62] for uplink and downlink QoS Flows based on current congestion level information reported from NG-RAN via GTP-U header

NOTE 2:	If the network operator want to apply the ECN marking for L4S, it shall guarantee that any sender (UE or Server) requesting classic ECN congestion control will not tag its packets with the ECT(1) in order to avoid conflicted usage of ECT(1) in L4S and that the interim routers over the path between the PSA-UPF to AS will not reset ECN field. Otherwise, L4S is not supported in network.

Editor's note:	Supports for L4S and for exposure of congestion level, Data rate, delay difference and round-trip delay, are pending RAN WG's feedback on the feasibility of RAN judgment and/or exposure of the corresponding info (e.g. per QoS flow congestion level).

-	5G System also may support API based exposure of congestion level information towards AF as following:

-	The following information may be exposed:

-	QNC for GBR QoS Flow: data rate cannot be guaranteed;

-	AF uses Nnef_AFSessionWithQoS to subscribe the above exposure to NEF/PCF, same as local exposure mechanism defined in TS 23.548 [61].

-	Exposure path of Network Exposure defined in clause 6.4 of TS 23.548 [61] is reused with extensions of GTP-U header and UPF/L-NEF services to exposure the above information.

-	Exposure path of RAN/UPF reporting congestion level information via SMF/PCF/NEF is also supported.

The following bullet points summarize the principles for the way forward to support exposure for other network information:

-	Data rate, delay difference and round trip delay may be exposed to AF.

-	The UPF may support obtaining and exposing the above information. Exposure path defined in clause 6.4 of TS 23.548 [61] is reused to expose the above information.

-	The RAN may support exposing the above information via SMF/PCF/NEF.

Editor's note:	It is FFS whether to expose the Normal data transmission interruption event to AF.

Editor's note:	It is FFS whether to support estimated QoS information to be exposed to AF is FFS.

# * * * * End of changes * * * *