3GPP TSG-WG SA2 Meeting #154 	S2-2210912

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Apple

Title:	KI#3: Conclusions update

Document for:	Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: Selection of final conclusions for key issue #3.

# 1. Introduction/Discussion

It is proposed select the following options:

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-41:

# * * * * First change * * * *

## 8.3	Conclusions for KI#3

For support of limited AoS slices not matching deployed TAs:

-	Reconfiguration of TAs to keep the end-to-end significance of slice unchanged, but an operator configures the cells of a TA that are outside AoS to have no or limited resources using existing NG-RAN OAM configuration. The solution can be combined with a mechanism that limited the SM signaling for slices when the UE is outside the AoS.

NOTE: 	Reconfiguration of TAs is supported by current specifications.

-	AMF is configured with S-NSSAI availability policies by NSSF that the AMF sends to the UE in the Configured NSSAI. Availability validity can be time and location. The UE uses the policies and when the availability is not valid, the UE considers the S-NSSAI to be 1) not registered or 2) registered while no UP resources are allowed to be activated based on information in the received policy.

For improved support of temporary network slices:

- 	AMF is configured with S-NSSAI availability policies by NSSF that the AMF sends to the UE in the Configured NSSAI.

Availability validity can be time and location. The UE uses the policies and when the availability is not valid, the UE considers the S-NSSAI to be 1) not registered or 2) registered while no UP resources are allowed to be activated based on information in the received policy.

NOTE:	Temporary network slices does not mean that the network slices are decommissioned and created as per the timing information, but the network slices are not meant to be available for use by the UE.

For the graceful and gradual termination aspect:

-	When a slice is not available for use for the UE, the PDU Sessions of the slice should be gracefully (for supporting UEs of the timing information) and gradually released (for no supporting UEs of the timing information):

-	If the UE supports the S-NSSAI availability policies, the network may provide the timing information to the UEs so the UE knows in advance when a network slice ceases to be supported. In this case, the UE can take the necessary actions to prepare for the slice not becoming available.

-	In addition, the AMF, for non-supporting UEs and for the case of UE not performing any actions despite of the timing information provided by the network, may be triggered by the OAM to start gradually terminating PDU Session(s) associated with S-NSSAI subject to be terminated. The AMF releases PDU Session(s), associated with the S-NSSAI subject to be terminated, based on operator’s policy available at the AMF.

# * * * * End of changes * * * *