3GPP SA WG2 Meeting #155	S2-2302926

# 20-24 February 2023, Athens, Greece	(was S2-2300161)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0147 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with security functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA3 in LS S3-223919 indicated that it has agreed to update TS 33.501 in accordance with the SA2 recommendation to document the Multicast/Broadcast Service Security Function (MBSSF) used for user plane security as a new logical network function. This CR is performing alignment with this SA3 decision and the related updates in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Documents the Multicast/Broadcast Service Security Function (MBSSF) in clause 6.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with security specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 6.13	MBS Security function

Security function may be used to protect MBS related signalling/data. Detailed descriptions of security requirements, procedures and handling for 5G Multicast/Broadcast Service (MBS) are provided in TS 33.501 [20].

|For MBS multicast security control plane procedures, MBS security function is implemented in the MBSF/MBSTF so that it can be applied only when MBSF/MBSTF are used (i.e. Configuration option 2 and 3). For configuration option 1 how to support MBS security is out of 3GPP scope.

The following additions for MBS multicast control plane procedures in the present specification apply if the MBS security function for multicast as defined in TS 33.501 [20] is used:

-	The multicast session security context, as defined in TS 33.501 [20], is used to protect MBS traffic of an MBS session. During the session establishment and when a UE joins, the multicast session security context contains MSK and MTK.

-	The UEs in the MBS session use the received multicast session security context to process the protected MBS traffic.

-	MBSF distributes the multicast session security context to the MB-SMF via the Nmbsmf_MBSSession_Create Request or Nmbsmf_MBSSession_Update Request message.

-	The SMF interacts with the MB-SMF to obtain the multicast session security context. The MB-SMF provides the security context in the Nmbsmf_MBSSession_ContextStatusSubscribe response message and in the Nmbsmf_MBSSession_ContextStatusNotify request message.

-	If the UE is authorized to join the Multicast MBS session, the SMF shall provide the multicast session security context to the UE in N1 SM container if it received the multicast session security context from the MB-SMF.

-	When the MSK needs to be updated, MBSF shall send the updated multicast session security context to the MB-SMF, and then the MB-SMF shall trigger the session update as specified in clause 7.2.6 to provide the updated multicast session security context to the UEs in the related MBS session. The updated multicast session security context shall contain an updated MSK and may contain an updated MTK in addition.

NOTE 1:	If no MSK but only the MTK is to be updated, the session update described in the previous bullet is not triggered and the MTK is updated as defined in TS 33.501 [20].

NOTE 2:	Interaction between MBSF and MBSTF will be defined in TS 33.501 [20] and TS 26.502 [18].

For MBS multicast security user plane procedures,  MBS logical security function is implemented in Multicast/Broadcast Service Security Function (MBSSF) defined in TS 33.501 [20].

NOTE 3:	Clause 9.1 defines the services for supporting security function for control plane procedure. The additions to the user plane procedure to support the security function for multicast and broadcast can be used as defined in TS 33.501 [20].

# >>>> NEXT CHANGES<<<<

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [5] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CDN	Content Delivery Network

FEC	Forward Error Correction

FSA	Frequency Selection Area

LL SSM	Lower Layer SSM

MBMS	Multimedia Broadcast/Multicast Service

MBS	Multicast/Broadcast Service.

MBSF	Multicast/Broadcast Service Function.

MBSSF	Multicast/Broadcast Service Security Function

MBSTF	Multicast/Broadcast Service Transport Function.

MB-SMF	Multicast/Broadcast Session Management Function.

MB-UPF	Multicast/Broadcast User Plane Function

PTM	Point To Multipoint

PTP	Point To Point

SSM	Source Specific IP Multicast address.

TMGI	Temporary Mobile Group Identity

# >>>>END OF CHANGES<<<<