3GPP TSG-WG SA2 Meeting #156E 	S2-2304014r01

# April 17-24, 2023	(revision of S2-230XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1284 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UE centric AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-03-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS network, which has been concluded in TR 23.700-87. This CR reflects the TR conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new Annex for both the UE centric AR Architecture and network centric AR architecture based on the DC architecture. This CR includes the UE centric architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

# Annex XX (normative):

# Support of AR Communication

# XX.1 General

This annex describes the enhancements to the IMS architecture supporting data channel services to additionally support  AR services. Two modes of AR services are supported; UE centric and network centric.

Editors Note: The definition of ‘AR media’ is for FFS.

## XX.2 Architecture

## XX.2.1 UE Centric AR

For the UE centric case, AR traffic is transparent to the IMS network and is exchanged between the two peer UEs via the deployed media functions (e.g IMS AGW/TrGW).

Figure XX.2.1-1 shows the architecture to support UE Centric AR communication.

Figure XX.2.1-1: Architecture to support UE Centric AR communication

# XX.3 Procedures

## XX.3.1 UE Centric Procedure

Figure XX.2.1-1 depicts a typical call flow procedure to establish a UE centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel connections.

2.	The user of UE-A upgrades the IMS session with AR experience. 
UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels.

In this scenario, the UE performs AR media rendering based on the AR media from its local application.

3.	UE-A captures and encodes AR media.

4.	UE-A sends the AR media to the peer through the established media connection(s).

5.	UE-A receives AR media from the peer through established media connection(s).

6.	The UE-A decodes and displays the received  AR media on its screen.

Figure XX.3.1-1: Establishment of UE Centric AR communication session

# * * * * End of changes * * * *