3GPP TSG-SA2 Meeting #156-e	S2-2305107

April 17-21, 2023, Electronic					                                      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4486 | Rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set Importance spanning across QoS Flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is the following Editor’s note in clause 5.37.5.2 Editor's note: Whether and how PDU Set Importance can span across QoS Flows is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the discussion in S2-2305101 it is proposed to add the following NOTE in clause 5.37.5.2 and delete the related Editor’s note: In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the PDU Set Importance across For QoS Flows (of the same or different UEs) of the same Priority Level, NG-RAN could consider PDU Set Importance when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved Editor’s note remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# * * * * 1st change * * * *

#### 5.37.5.2	PDU Set Information and Identification

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The NG-RAN may use the Priority Level across QoS Flows  and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE: In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the PDU Set Importance across QoS Flows  of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.

Editor's note:	The PDU Set Size is pending SA WG4 progress on SA WG4 5G_RTP WI. It is up to SA4 to decide whether PDU Set Size in an SA4 defined RTP header, extension header and/or payload is in bytes and whether the PDU Set Size in an SA WG4 defined RTP header, extension header and/or payload includes the RTP and/or IP overhead.

NOTE:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

The SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the Protocol Description indicating the header, extension header (e.g. RTP/SRTP) and payload type (e.g. H.264) used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

PSA UPF can identify the PDU Set information using the Protocol Description and the received RTP/SRTP headers or using implementation specific means.

Editor's note:	PDU Set Information in RTP/SRTP header, extension header and/or payload is pending SA WG4 progress on SA WG4 5G_RTP WI.

For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

END CHANGE