SA WG2 Meeting #157	S2-2307238

# Berlin, Germany, May 22nd – 26th, 2023           	  			  			       		 (revision of S2-230)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0155 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Re-discovery in HR-SBO context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | inter V-SMF intra VPLMN and inter PLMN Hand-over in HR-SBO case is not fully specified EAS Re-discovery in HR-SBO context is not fully specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fully Specify inter V-SMF intra VPLMN and inter PLMN Hand-over in HR-SBO case EAS Re-discovery in HR-SBO context is not fully specified |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete EDGE_Ph2 KI1 related specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

fIrst  CHANGE

#### 6.7.3.2	Network triggered EAS Re-discovery in HR-SBO context

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery when HR-SBO is supported and allowed in the target serving PLMN.

Editor’s Note: the case where HR-SBO is not possible for the target serving PLMN is FFS

Figure 6.7.3.2-1: Network triggered EAS rediscovery HR-SBO context procedure

1	The procedure described in clause 6.7.2.X is performed: inter V-SMF intra VPLMN or inter PLMN N2 Hand-over in HR-SBO case

As part of this procedure, if the AF had subscribed to the corresponding event and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF, indicating the new serving PLMN ID. This may take place as soon as the H-SMF has received an indication of Handover Complete (see clause 6.7.2.X).

NOTE:	via this mechanism the AF is aware of the PLMN to contact to issue traffic influence requests. The AF is assumed to check whether it has a SLA with the new serving PLMN. If the AF has no SLA with the new serving VPLMN the AF interacts with H-NEF to have EAS used that are reached via HPLMN UPF(s)  .

This may trigger the AF triggered EAS relocation / rediscovery as defined in step 1b of Figure 6.2.3.3-1.

2a.	(V-SMF removal case) For AF triggered EAS rediscovery via interacting with HPLMN, the AF may indicate to the H-NEF the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3].

2b.	(V-SMF insertion or V-SMF change case) For AF triggered EAS rediscovery via interacting with target VPLMN, the AF may indicate to the V-NEF the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3]. This may trigger step 2 of of Figure 6.2.3.3-1 where the SMF that initiates PDU Session modification is the V-SMF that initiates Nsmf_PDUSession_Update request with the requested PCO

3a	(V-SMF removal case) This may trigger step 2 of of Figure 6.2.3.3-1 where the SMF that initiates PDU Session modification is the H-SMF that issues a Nsmf_PDUSession_Update request.

3b.	(V-SMF insertion or change case) The H-SMF initiates Nsmf_PDUSession_Update Response towards the (target) V-SMF including the PCO.

4.	The V-SMF initiates PDU Session Modification command including the PCO to the UE.

The PCO includes EAS rediscovery indication (optional) and the impact field (optional).

NEXT CHANGE All text is NEW

#### 6.7.2.X	inter V-SMF intra VPLMN and inter PLMN N2 Hand-over in HR-SBO case

This clause defines the procedure for inter V-SMF intra VPLMN and inter PLMN N2 based Hand-over for HR-SBO PDU sessions. This procedure is based on Inter NG-RAN node N2 based handover with I-SMF insertion/change/removal defined in TS 23.502 [3] clause 4.23.7.3.

The flow assumes the UE has an HR-SBO PDU session established using a source serving PLMN (e.g. involving a source V-SMF) and there is a HO to a target serving PLMN (e.g. involving a target V-SMF). The flow is also meant to apply when source or target serving network (before or after an inter PLMN HO) is the HPLMN;

Editor’s Note: the case where HR-SBO is not possible for the target serving PLMN is FFS

Figure 6.7.3.2-1: inter V-SMF intra VPLMN and inter PLMN N2 based Hand-over in HR-SBO case

NOTE: 	the flow above does not show all steps in clause 4.23.7.3 of TS 23.502 [3] but only those that are impacted by the support of HR-SBO

The procedure described in clause 4.23.7.3 (N2 based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF with following modifications.

0.	The AF may subscribe onto H-SMF for the event related with a mobility of the PDU Session towards a serving PLMN where local traffic offload is possible, i.e. mobility of the PDU Session either towards HPLMN or towards a VPLMN where HR-SBO is possible i.e. supported and allowed

- Step 1 of TS 23.502 [3] Figure 4.23.7.3.2-1 (initiation of the N2 based handover). For an intra PLMN HO, the source AMF provides the target AMF with whether HR-SBO is allowed for each PDU session of the UE

Steps 3 to 5 are skipped if the UE moves with V-SMF removal

For an inter PLMN HO, the source AMF does not provide the target AMF with whether for a PDU session of the UE HR-SBO is allowed. The target T-AMF uses local policies related to the SLA with the HPLMN of the UE to determine whether HR-SBO allowed indication is sent for the PDU Sessions of the UE,

NOTE:	This means that if HR-SBO is allowed only for some users of the HPLMN and is not allowed for the UE being handed over, this will only be detected at a later step of the procedure i.e. at step 8.

2.	2a (V-SMF insertion or V-SMF change) Step 2 and 3 of TS 23.502 [3] Figure 4.23.7.3.2-1 take place. If the T-AMF consider that HR SBO is possible, the T-AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the target V-SMF in Nsmf_PDUSession_CreateSMContext Request as part of the step 3 of the procedure in figure 4.23.7.3.2 of TS 23.502 [3].

2b (V-SMF removal) Step 9 to 12b of TS 23.502 [3] Figure 4.23.7.3.2-1 take place: the target AMF sends to H-SMF a Nsmf_PDUSession_CreateSMContext request  as in Step 9 of TS 23.502 [3] Figure 4.23.7.3.2-1. The H-SMF selects an EASDF in HPLMN and configures the DNS context in this EASDF (Neasdf_DNSContext_Create) and may select and configure UPF(s) in HPLMN;

3.	steps 4 or 5 of TS 23.502 [3] Figure 4.23.7.3.2-1.

3a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request/Response, followed by the Nsmf_PDUSession_Context Request/Response.

3b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response indicating HR-SBO support in the Request in the case of an intra PLMN V-SMF case.

In step 3b, the SM context includes reference information on the V-EASDF context it was controlling for the PDU Session, Authorization Result for HR-SBO and VPLMN Specific Offloading Policy if source and target V-SMFs belong to same VPLMN.

4.	The (target) V-SMF may (re-)select a (new) V-EASDF e.g., based on the target UE location. The (target) V-SMF may invoke Neasdf_DNSContext_Create including the DNN, S-NSSAI and PLMN ID to obtain the new V-EASDF address. In case of V-SMF insertion, the UE IP address is set to the unspecified address in Neasdf_DNSContext_Create as specified in clause 7.1.2.2. If this is an Inter PLMN mobility and the target V-SMF is configured to use a V-EASDF, the target V-SMF shall select a (new) V-EASDF

5	The (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in step 2 of Figure 6.7.2.2-1.

NOTE:	When there are other V-UPF(s) between UL CL/BP and H-UPF, the (target) V-SMF sets up user plane between this ULCL/BP and the V-UPF.

6	The H-SMF (V-SMF removal) or the target V-SMF (V-SMF insertion or V-SMF change) answers to the target AMF with a Nsmf_PDUSession_CreateSMContext Response as in Step 8 (V-SMF insertion or V-SMF change)  or step 13 (V-SMF removal) of TS 23.502 [3] Figure 4.23.7.3.2-1

The hand-over further continues as defined in steps 15 to last of Figure 4.23.7.3.2-1 of TS 23.502 [3] and then per clause 4.23.7.3.3 of TS 23.502 [3], with clause 4.23.7.3.3 of TS 23.502 [3] modified as follows,

7	(V-SMF insertion or V-SMF change) when the T-AMF sends to the Target V-SMF an indication of Handover Complete within Nsmf_PDUSession_UpdateSMContext Request (at step 2 of TS 23.502 [3] Figure 4.23.7.3.3-1) the target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF. The V-SMF indicates HR-SBO support (even in intra PLMN V-SMF change). If an EASDF has been selected in step 4, the selected V-EASDF address is also provided in the request; if this is an inter PLMN change, and the UE indicated support of the feature, the V-SMF may provide an EAS rediscovery indication, to the H-SMF. This triggers step 8 to 11

8.	(V-SMF insertion or V-SMF change) If the UE has changed of serving VPLMN or has moved from HPLMN to a VPLMN the H-SMF authorizes the request for HR-SBO based on the SM subscription data (i.e., HR-SBO authorization indication) and once authorized retrieves the optional VPLMN Specific Offloading Policy from H-PCF.

9.	(V-SMF insertion or V-SMF change) As defined in step 8 of clause 4.23.7.3.3 of TS 23.502 [3], the H-SMF invokes Nsmf_PDUSession_Update Response, providing the same HR-SBO related information as sent by the H-SMF in step 2 of Figure 6.7.2.2-1 including the DNS information for the UE (the V-EASDF address (optional)) to be sent in PCO. The H-SMF furthermore provides V-SMF with EAS rediscovery indication to be sent to the UE if received at step7  The target V-SMF stores the DNS information for the UE until it receives an indication of HO completion. This information is also provided for intra VPLMN V-SMF change.

If the request for HR-SBO is not authorized and a V-EASDF context had been created, the V-SMF deletes the DNS context from the selected V-EASDF, and the subsequent steps related to the HR-SBO in this procedure are skipped.

10	(V-SMF insertion or V-SMF change) The (target) V-SMF configures the V-EASDF with the DNS handling rules using the received VPLMN specific offloading policy as defined in step 3 of Figure 6.7.2.2-1.

If the V-SMF has interacted with the V-EASDF in step 4, the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The V-SMF may configure the V-UPF selected in the step 5 to forward DNS messages to V-EASDF.

11	(V-SMF insertion or V-SMF change) the target V-SMF sends the DNS information for the UE (received at step 9) and EAS rediscovery indication via PCO in a PDU session Modification Command sent to the UE

(In the case of V-SMF removal) At step 10 of 4.23.7.3.3-1 of TS 23.502 [3], when receiving Nsmf_PDUSession_UpdateSMContext Request (Handover Complete indication), the H-SMF sends the DNS information for the UE (determined at step 9b) via PCO in a PDU session Modification Command sent to the UE

12 In case of inter PLMN V-SMF change or in case of V-SMF removal, when the S-AMF invokes Nsmf_PDUSession_ReleaseSMContext Request to inform the Source V-SMF to release the SM context of the PDU Session, e.g. at step3a or 11a of TS 23.502 [3] Figure 4.23.7.3.3-1, the DNS context in the old (V-)EASDF is removed by the (source) V-SMF using Neasdf_DNSContext_Delete service.

In case of V-SMF insertion, when the target V-SMF sends Nsmf_PDUSession_Update Request with a Handover Complete Indication at step 6 of TS 23.502 [3] Figure 4.23.7.3.3-1, the DNS context in the old EASDF is removed by the H-SMF using Neasdf_DNSContext_Delete service

If the serving PLMN has not changed the new V-SMF is responsible of determining whether and when to remove the context in the old V-EASDF.

13.	if the AF had subscribed in step 0 and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF. This may take place as soon as the H-SMF has received an indication of Handover Complete

14.	EAS discovery procedure by UE may take place: EAS Discovery procedure with V-EASDF for HR-SBO (§ 6.7.2.3) / with Local DNS for HR-SBO (§6.7.2.4) / with V-EASDF using IP replacement mechanism (§6.7.2.5)

NEXT CHANGE

END OF CHANGES