SA WG2 Meeting #156E (e-meeting)	S2-2306336

# Berlin, Germany, May 22 – 26, 2023				(revision of S2-2305154)

Source:	Nokia, Nokia Shanghai-Bell

Title:	Proposals for open issues for key issue 3 (roaming)

Document for:	Approval

Agenda Item:	9.23.2

Work Item / Release:	eNA_Ph3 / Rel-18

Abstract: This contribution discusses some open points for key issue 3. A related CR is in S2-2306335.

# 1. Introduction

TS 23.288 contains the following:

Editor's note: It is FFS whether an analytics profile is required in addition to analytics.

Editor's note:	Whether and how the V-NWDAF aggregates the analytics from HPLMN and the analytics derived in VPLMN is FFS.

Editor's note:	Whether and how the H-NWDAF aggregates the analytics from VPLMN and the analytics derived in HPLMN is FFS.

# 2. Discussion

- Analytics Profile.

Editor's note: It is FFS whether an analytics profile is required in addition to analytics.

Operators may desire to limit the amount of exposed analytics information (both in terms of analytics IDs and parameters) about outbound roaming users to a small amount of what is available in analytics and may also want to know the analytics information that can be obtained from peer operators to prepare to use this information in their network. Legal constraints and/or regional regulatory requirements may further limit the amount of information to be exchanged. It is expected that GSMA will work on recommendations for roaming agreements that detail the information that can be exchanged

The intention of the analytics profile is to define a simple format with essential analytical information in the hope that this can ease the interoperability and simplify the provisioning and use of analytical information. Having a fixed set of parameters can ease interoperability and can also simplify a possible filtering based on different agreements between operators.

It is suggested to support an analytics profile for an outbound roaming user in addition to analytics.

2. Aggregating analytics derived by peer PLMN and own analytics

Editor's note:	Whether and how the V-NWDAF aggregates the analytics from HPLMN and the analytics derived in VPLMN is FFS.

Editor's note:	Whether and how the H-NWDAF aggregates the analytics from VPLMN and the analytics derived in HPLMN is FFS.

Some analytics rely on input data from VPLMN and HPLMN and seem more suited to this.

In addition, if several analytics IDs are requested, some could be derived in own PLMN and some in the peer PLMN

It can be left to implementation and the nature of the requested analytics whether analytics from peer PLMN and own input data are combined.

3. Does NF consumer in HPLMN know that analytics target is roaming?

The current procedures suggest that an analytics consumer need to select an NWDAF with roaming entry capability.

However, an analytics consumer may not know at all whether a UE is roaming when requesting analytics or statistics. It may also have no information about NSSAI and ID of peer PLMN.

An NWDAF at the originating side can determine the PLMN ID where the UE is located, and perform mapping of slice information.

It should not be up to an NF consumer but an NWDAF at originating side to decide whether an analytics request requires input from another PLMN. No related input from the NF consumer is required.