3GPP TSG-WG2 Meeting #158	S2-2309120

# Goteborg, Sweden, August 21 – 25, 2023                                          (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0173 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on EAS change procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To align the terms by replacing the “EAS re-discovery” with “EAS change”. Since the procedure does not only cover EAS re-discovery, but also provide EAS IP replacement mechanism for edge relocation case, it is not suitable to use “EAS re-discovery” as title. - To resolve the Editor’s Note: Editor's note: Whether this can trigger an intra-VPLMN V-SMF Change (induced by target DNAI) and if yes how, needs to be checked. When target DNAI provided from H-SMF to V-SMF. If the V-SMF cannot serve the target DNAI, it could send the target DNAI to AMF, and the AMF selects a target V-SMF based on the target DNAI as described in clause 4.23.5.4 of TS 23.502 [3] by relacing I-SMF with V-SMF. Then the target V-SMF retrieves SM context from the source V-SMF, containing HR-SBO allowed indication, EAS IP replacement information and target DNAI in the Request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The main changes are as follows: 1) Replacing the “EAS re-discovery” with “EAS change” in title and procedures. 2) Clarifyling the intra-VPLMN V-SMF change based on target DNAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

6.7.3	EAS Change Procedure

6.7.3.1	General

The EAS change in VPLMN can be triggered due to UE mobility, AF interacting with HPLMN, or AF interacting with VPLMN.

6.7.3.2	Network triggered EAS change in HR-SBO context

Figure 6.7.3.2-1 shows the procedure of EAS change when HR-SBO is supported and allowed in the target serving PLMN.

Figure 6.7.3.2-1: Network triggered EAS changein HR-SBO context procedure

1	The procedure described in clause 6.7.2.6 is performed: inter V-SMF intra-VPLMN or inter-PLMN N2 handover in HR-SBO case.

As part of this procedure, if the AF had subscribed to the corresponding event and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF, indicating the new serving PLMN ID. This may take place as soon as the H-SMF has received an indication of Handover Complete (see clause 6.7.2.6).

NOTE:	Via this mechanism, the AF is aware of the PLMN to contact to issue traffic influence requests. The AF is assumed to check whether it has an SLA with the new serving PLMN. If the AF has no SLA with the new serving VPLMN, the AF interacts with H-NEF to have EAS used that are reached via HPLMN UPF(s).

This may trigger the AF triggered EAS relocation / rediscovery as defined in step 1b of Figure 6.2.3.3-1 and in step 4a of Figure 6.3.3.1.1-1.

2a.	For AF triggered EAS change via interacting with HPLMN, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3]. The AF may also provide EAS IP replacement information and target DNAI together with an indication of the PLMN associated with this target DNAI, i.e. the serving PLMN ID.

2b.	For AF triggered EAS change via interacting with target VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3]. This may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates PDU Session modification is the V-SMF that initiates Nsmf_PDUSession_Update request with the requested PCO. The AF may also provide EAS IP replacement information and target DNAI.

3a.	(For AF triggered EAS change via interacting with HPLMN case) The AF request relayed via a PCC rule to H-SMF may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates the PDU Session modification is the H-SMF: the H-SMF issues a Nsmf_PDUSession_Update request. This Nsmf_PDUSession_Update request may contain EAS IP replacement information and target DNAI provided by AF in step 2. If the V-SMF cannot serve the target DNAI, it invokes a Nsmf_PDUSession_SMContextStatusNotify service operation to send the target DNAI to AMF, and the AMF selects a target V-SMF based on the target DNAI as described in clause 4.23.5.4 of TS 23.502 [3] by relacing I-SMF with V-SMF. The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response, containing HR-SBO allowed indication, EAS IP replacement information and target DNAI in the Request.

3b.	(For AF triggered EAS change via interacting with VPLMN case) The H-SMF initiates Nsmf_PDUSession_Update Response towards the (target) V-SMF including the PCO information to be sent to the UE. In intra-PLMN V-SMF change, the target V-SMF may use the source and target DNAI to determine the Impact field to be sent to the UE. In inter-PLMN mobility, the target V-SMF provides EAS rediscovery information without an Impact field.

4.	The V-SMF initiates PDU Session Modification command including the PCO to the UE.

The PCO may include EAS rediscovery indication (optional) and the impact field (optional).

5	The V-SMF may configure the V-UPF (UL CL and L-PSA) with EAS IP replacement information provided by AF in step 2.

>>>> End of Changes <<<<