3GPP TSG-SA WG2 Meeting #158	S2-2309152

# Goteborg, Sweden, 21 – 25 Aug, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0177 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Inter V-SMF (intra-VPLMN and inter-PLMN) Idle mode mobility procedure supporting HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Inter V-SMF (intra-VPLMN and inter-PLMN) Idle mode mobility procedure supporting HR-SBO is described in clause 6.7.2.7. - The following Editor's note remains for precise description on steps with procedure figure to be added. Editor's note: This procedure needs a figure and a more precise description needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolving the remaining Editor's note with following change: - Descriptions added in steps for clarification. - Procedure figure is added resolving the remaining Editor's note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502 CR 4401 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.7.2.7	Inter V-SMF (intra-VPLMN and inter-PLMN) Idle mode mobility in HR-SBO case

This clause defines the procedure for inter V-SMF intra-VPLMN and inter-PLMN Idle mode mobility for HR-SBO PDU Sessions. This procedure is based on Mobility Registration Update procedure defined in clause 4.2.2.2.2 of TS 23.502 [3] with V-SMF insertion/change/removal defined in clause 4.23.3 of TS 23.502 [3] and steps refer to descriptions on Inter V-SMF (intra/inter-PLMN) N2 Handover procedure defined in clause 6.7.2.6.

Figure 6.7.2.7-1: Inter V-SMF (intra-VPLMN and inter-PLMN) Idle mode mobility procedure in HR-SBO

1.	The UE establishes a HR-SBO PDU Session described in clause 6.7.2.2.

2.	The UE initiates Mobility Registration Update procedure to the Target AMF as described in clause 4.2.2.2.2 of TS 23.502 [3]. During the registration procedure, the target AMF (in roaming case) receives the SMF selection data including HR-SBO allowed indication per DNN/S-NSSAI from the UDM in the step 14b of the procedure in clause 4.2.2.2.2 of TS 23.502 [3].

3.	Step 2-16 in clause 4.2.2.2.2 of TS 23.502 [3] is performed for Mobility Registration Update. During the registration procedure, the target AMF (in roaming case) receives the SMF selection data including HR-SBO allowed indication per DNN/S-NSSAI from the UDM in the step 14b of the procedure in clause 4.2.2.2.2 of TS 23.502 [3].

The target AMF determines the V-SMF insertion or change based on the subscription data to support HR-SBO and invokes the Namf_Communication_RegistrationStatusUpdate to release the old SM context as in the step 10 of the procedure in clause 4.2.2.2.2 of TS 23.502 [3].

4.	(V-SMF insertion or V-SMF change) the Target AMF selects a V-SMF supporting HR-SBO for the established session with the corresponding DNN and/or S-NSSAI and sends the received HR-SBO allowed indication in the PDUSession_CreateSMContext Request to the target V-SMF.

5.	The target V-SMF selects the V-EASDF or the local DNS server as described in step 4 of Figure 6.7.2.6 -1.

6.	The target V-SMF performs UL CL/BP and local PSA insertion/change/removal as described in step 5 of Figure 6.7.2.6 -1.

7.	The target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF with HR-SBO request indication as described in step 7 of Figure 6.7.2.6-1.

8.	The H-SMF authorizes the request for HR-SBO based on the SM subscription data and as described in step 8 of Figure 6.7.2.6-1.

Once the HR-SBO is authorized, the H-SMF retrieves the VPLMN Specific Offloading Policy from H-PCF and sends the VPLMN Specific Offloading Information in the Nsmf_PDUSession_Update Response message to the target V-SMF.

9.

The target V-SMF configures the V-EASDF and the V-UPF based on the received VPLMN Specific Offloading Information as described in step 9 of Figure 6.7.2.6 -1 and  responds to the target AMF with PDUSession_CreateSMContext Response.

10.	The Mobility Registration Update procedure continues after step 16 in clause 4.2.2.2.2 of TS 23.502 [3].

11.	After successful registration, the EAS discovery procedure by UE may take place as described in step 14 of Figure 6.7.2.6-1.

* * * * End of Changes * * * *