3GPP SA WG2 Meeting #158	S2-2309478

21-25 Aug 2023, Goteborg, Sweden                  	(revision of S2-2308802)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0313 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on 5G ProSe UE-to-UE Relay Discovery with Model A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, vivo, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the S2-2308314/R2-2306697 LS on announcement of neighbor UEs from RAN2, it describes as following: RAN2 discussed the Model A discovery procedure for U2U relay, and made the following agreement: For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message.. Here the neighbour UEs denote the UEs in proximity of relay UE. RAN2 would like to ask SA2 to take this RAN2 agreement into account. Based on the LS, it is proposed to add the PC5 signal strength condition for 5G ProSe UE-to-UE Relay Discovery with Model A. |
| Summary of change: |  | Add a NOTE regarding the PC5 signal strength condition for 5G ProSe UE-to-UE Relay Discovery with Model A: the 5G ProSe UE-to-UE Relay shall only announce the neighbour UEs for which the PC5 signal strength between them is above a configured threshold. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In 5G ProSe UE-to-UE Relay Discovery with Model A procedure, the condition of sending a UE-to-UE Relay Discovery Announcement message is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.3.2.4.2	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

Depicted in Figure 6.3.2.4.2-1 is the procedure for 5G ProSe UE-to-UE Discovery with Model A.

Figure 6.3.2.4.2-1: 5G ProSe UE-to-UE Relay Discovery with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered other UEs in proximity (e.g. via a previous 5G ProSe UE-to-UE Relay Discovery or 5G ProSe UE-to-UE Relay Communication procedures). The 5G ProSe UE-to-UE Relay obtains the User Info ID of other UEs in proximity per RSC.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, RSC and list of User Info ID of the 5G ProSe End UEs and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

The 5G ProSe UE-to-UE Relay shall only announce User Info IDs of other UEs in proximity which provided relay_indication when they were previously discovered.

NOTE:	5G ProSe UE-to-UE Relay announces User Info IDs of other UEs in proximity only if their PC5 signal strength measured by the 5G ProSe UE-to-UE Relay is above configured signal strength threshold as specified in TS 38.331 [16].

A 5G ProSe End UE monitors announcement messages from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

END OF CHANGES