3GPP SA WG2-Meeting #158	S2-2309632

Gothenburg, Sweden, August 21-25, 2023	(revision of S2-230xxxx)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New SID on 5GS XR Metaverse enhancements

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Architecture enhancement for XR and Metaverse services

Acronym:	FS_XRMeta_enh

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950005 | Study on Localized Mobile Metaverse Services | Stage 1 study item |

# 3	Justification

SA2 Rel-18 work item on XR introduced support for PDU Set based handling for QoS framework however some aspects of PDU Set framework could not be addressed properly (e.g., support for layered media, relationship between different layers, tailoring PDU set error handling based on application needs, support for PDU Sets containing forward error correction). The ability for the AF to influence PDU set forming or importance determination is also limited especially for application traffic flows containing multiple media streams and protocols. Most fundamentally, in Rel-18 network control of PDU Set based QoS focus is mainly for the downlink direction and we have an open Editor’s note for uplink direction (pending RAN2 update) in SA2 specification.

Rel-18 also introduced the support for power saving optimizations based on dynamic End of Data Burst indication in addition to more static periodicity and burst size information. This, as well as the PDU Set based handling, could further be enhanced to support adaptive applications.

SA1 is also working on specific use cases and service requirements for 5GS support of enhanced XR-based services as well as potentially other functionality, to offer shared and interactive user experience of local content and services, accessed either by users in the proximity or remotely. In particular, the following areas are studied:

-	Support of interactive XR media shared among multiple users in a single location, including:

-	performance (KPI) aspects; e.g. latency, throughput, connection density

-	efficiency and scalability aspects, for large numbers of users in a single location.

-	the combination of haptics type of XR media and other non-haptics types of XR media.

-	Identification of users and other digital representations of entities interacting within the Metaverse service.

-	Acquisition, use and exposure of local (physical and digital) information to enable Metaverse services, including:

-	acquiring local spatial/environmental information and user/UE(s) information (including viewing angle, position and direction);

-	exposing local acquired spatial, environmental and user/UE information to 3rd parties to enable Metaverse services.

-	Other aspects, such as privacy, charging, public safety and security requirements.

The study also investigates gaps between the identified new potential requirements and the requirements already specified for the 5G system.

A difference between this study and TR 22.847 " Study on supporting tactile and multi-modality communication services" is that Metaverse services would involve coordination of input data from different devices/sensors from different users and coordination of output data to different devices at different destinations to support the same task or application.

# 4	Objective

The following aspects will be studied:

WT1 Enhancement for PDU Set based QoS handling.

1.1 Study whether and how to enhance PDU Set related and PDU Set information (including Control Plane and/or User plane information provided by the AF/AS) and the corresponding PDU Set QoS handling enhancement.

NOTE: this will require close coordination between SA4 and SA2.

1.2 Support QoS control and PDU Set identification for XR stream with e2e encryption (e.g. fully encrypted header, partially encrypted header). This is applicable for PDUs received at N6 for DL.

1.3 Enhancements to support PDU Set based QoS handling in uplink direction.

NOTE: need for this work task depends on Rel-18 progress.

1.4 Study whether and how to leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network (i.e. to enable differentiated handling of PDU Sets within QoS Flow).

WT2 QoS handling enhancement for XRM services.

2.1 Study whether and how enhancements can be done for traffic detection and QoS Flow mapping for multiplexed data flows (different media types multiplexed within the same 5-tuple traffic flow).

2.2 Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by AF/AS.

NOTE: this will require close coordination between SA4 and SA2.

WT4 Further enhancement to support XR based on non-3GPP access.

4.1 Study how to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.

-	Support L4S in untrusted/trusted access (N3IWF, TNGF).

-	Support L4S in wireline access (W-AGF).

4.2 Study how PDU Set QoS Control mechanisms can be extended to non-3GPP access networks.

-	Support PDU Set QoS in untrusted/trusted access (N3IWF, TNGF)

-  Support PDU Set QoS in wireline access (W-AGF).

NOTE X: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

WT7 Network exposure Study whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer.

NOTE:  alignment and coordination with RAN work will be needed for the study.

## TU estimates and dependencies

| Work Task ID |  | TU Estimate (Study) |  | TU Estimate (Normative) |  | RAN Dependency (Yes/No/Maybe) |  | Inter Work Tasks Dependency |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WT#1 |  | 2.0 |  | 2.0 |  |  |  | WT#1 is self-contained |  |
| WT#1.1 |  |  |  |  |  | Yes |  |  |  |
| WT#1.2 |  |  |  |  |  | No |  |  |  |
| WT#1.3 |  |  |  |  |  | No |  |  |  |
| WT#1.4 |  |  |  |  |  | No |  |  |  |
| WT#2 |  | 2.0 |  | 2.0 |  |  |  | WT#2 is self-contained |  |
| WT#2.1 |  |  |  |  |  | Yes |  |  |  |
| WT#2.2 |  |  |  |  |  | Yes |  |  |  |
| WT#4 |  | 1.5 |  | 1.5 |  |  |  | WT#4 is self-contained |  |
| WT#4.1 |  |  |  |  |  | No |  |  |  |
| WT#4.2 |  |  |  |  |  | No |  |  |  |
| WT#7 |  | 1 |  | 1 |  |  |  | WT#7 is self-contained |  |

Total TU estimates for the study phase: 6.5

Total TU estimates for the normative phase: 6.5

Total TU estimates: 6.5 + 6.5 = 13

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | TR 23.xxx | Study on 5GS XR enh | SA#103 (March 2024) | SA#104 (June 2024) |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN impact to be covered by RAN WGs.

Potential security impact to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell