SA WG2 Meeting #S2-159							                 S2-2310359

09 - 13 October, 2023, Xiamen, P.R. China

Title:	[Draft]LS reply on Data rate monitoring

Response to:	LS on Data rate monitoring (C4-233573/S2-2310108)

Release:	Release 18

Work Item:	XRM

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR4736

1. Overall Description:

SA2 thanks CT4’s LS on Data rate monitoring (C4-233573/S2-2310108) and would like to provide the following replies to CT1’s questions:

CT4 Question1:

Request SA2 to clarify the data rate information that the UPF shall expose. For instance, if the (PFCP) Measurement Period IE (indicating the period to generate periodic measurement reports) is set to 10 seconds and the monitoring averaging window is 2 seconds, should the UPF measure the data rate during 2 seconds only once within the 10 seconds and report this measurement, or should the UPF measure the data rate up to 5 times within the 10 seconds and report the minimum, maximum and average data rate over the measurement period?

SA2 answer:

The data rate report is based on the definition in clause 5.8.2.18 in TS 23.501. With regard to the instance, the latter one seems more precise but SA2 treats this detail of the data rate as stage3’s definition and it is up to CT4’s decision.

CT4 Question2:

what is the standardized value of the monitoring averaging window for non-GBR QoS flows?

SA2 answer:

2000ms as the Default Averaging Window of the QoS flow as described in clause 5.7.4 of TS 23.501.

# 2	Actions

To CT4

ACTION: 	SA2 respectfully asks CT4 to take the above information into account.

# 3	Dates of next TSG SA WG 2 meetings

SA2#160	13th - 17th November 2023, 			Chicago, USA

SA2#161	26th February-1st March, 2024                            	Athens, GR