3GPP TSG-SA WG2 Meeting #159	S2-2310410

# Xiamen, China, 09 – 13 October 2023								  	    (Revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0364 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Issue on U2U relay reselection triggered by PC5 RLF or PC5 link release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the agreed running CR for TS38.300 (R2-2309339), the U2U remote UE can trigger the U2U relay reselection based on the cases as quoted below. The U2U Remote UE may triggers U2U Relay reselection in the following cases: - The PC5 signal strength of the current U2U Relay UE is below a (pre)configured signal strength threshold; (Case 1) - When U2U Remote UE detects PC5 RLF; (Case 2) - When U2U Remote UE receives the PC5 RLF indication from the U2U Relay UE; (Case 3) - When U2U Remote UE receives a PC5-S link release message from U2U Relay UE; (Case 4) - Indicated by the upper layer of the U2U Remote UE. (Case 5) From lower layer perspective, negotiated 5G ProSe UE-to-UE Relay reselection procedure is not applicable to Case (2) – (4) since the current negotiated 5G ProSe UE-to-UE Relay reselection procedure relies on available links on both hops but in above cases of (2)-(4) one of the links is questionable. In this situation, the U2U remote UE can initiate the link establishment procedure with with integrated discovery instead of initiating the negotiated 5G ProSe UE-to-UE Relay reselection procedure. Thus, this CR is proposed to take the cases of unavailable direct link into account for 5G ProSe UE-to-UE Relay reselection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.7.4.1, the statements “The 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection according to the cases defined in TS 38.300 [12]” and “5G ProSe UE-to-UE Relay Communication with integrated Discovery defined in clause 6.7.3” are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The negotiated 5G ProSe UE-to-UE Relay reselection procedure triggered by PC5 RLF or PC5-S link release does not work and thus leading to failure on the U2U relay communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs keep performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay for relay reselection. The 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection according to the cases defined in TS 38.300 [12].

For 5G ProSe UE-to-UE Relay reselection, a 5G ProSe UE-to-UE Relay may be discovered by either the discovery procedures defined in clause 6.3.2.4, 5G ProSe UE-to-UE Relay Communication with integrated Discovery defined in clause 6.7.3, or by the negotiated 5G ProSe UE-to-UE Relay reselection procedure defined in clause 6.7.4.2 or clause 6.7.4.3.

In the negotiated UE-to-UE Relay reselection defined in clause 6.7.4.2 or clause 6.7.4.3, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection and to establish the communication via the reselected 5G ProSe UE-to-UE Relay prior to releasing the communication via the current 5G ProSe UE-to-UE Relay.

END OF CHANGES