3GPP TSG-WG SA2 Meeting #159	S2-2310543

# Xiamen, China, Oct 9 – 13, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0950 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the velocity estimation to the input data for relative proximity analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The OAM may collect UE information through MDT. However, there are two ways to collect UE information using MDT, one is based on signaling to collect UE information for a specific UE and the consumer should provide UE ID (e.g. SUPI) of the UE, and the other is based on administration to collect UE information for UEs located in an area and the output is indicated by an administration identifier that is not recognized by NWDAF. Therefore, the Note1 need to be modified to clarify how NWDAF obtains the UE information from OAM. The velocity of UE may be collected from GMLC as specified in clause 5.5 of TS23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the Note1 how NWDAF obtains the UE information from OAM. - Add the velocity estimation to the input data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The input data of analytics ID is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.19.2	Input data

The detailed input data collected by the NWDAF for relative proximity analytics is listed in Table 6.19.2-1, Table 6.19.2-2 and Table 6.19.2-3.

Table 6.19.2-1: OAM input data for relative proximity analytics

| Information | Source | Description |
| --- | --- | --- |
| Per UE information | OAM |  |
| > Speed |  | UE Speed |
| > Orientation |  | UE Orientation |

NOTE 1:	To collect the input data in Table 6.19.2-1, NWDAF can invoke TraceJob with the UE ID (e.g. SUPI) of the UE as described in clause 4.3.30 of TS 28.622 [41] using Generic Provisioning Management Service as described in clause 11.1 of TS 28.532 [6] to get Sensor Information as part of MDT data collection mechanism. The UE sensor information includes Barometric pressure, UE speed and UE orientation as described in clause 5.10.29 of TS 32.422 [42].

Table 6.19.2-2: Proximity related input data collected via DCAF/NEF

| Information | Source | Description |
| --- | --- | --- |
| Proximity Attribute | DCAF / NEF | Characterise a set of UEs in relative proximity. |
| > Number of UEs |  | Total number of UEs in proximity of target UE. |
| > Timestamp |  | A time stamp of time that the proximity attribute derived. |
| > Application ID(s) |  | Identifying the applications(s) providing this information. |
| > List of UE IDs |  | UE IDs in proximity of target UE if available. |
| > Other attribute(s) |  | Other attributes for the set of UEs i.e. destination, route, average speed, time of arrival. |
| > Confidence |  | Confidence on relative proximity data. |

NOTE 2:	The list of UE IDs that fulfil a proximity criterion can be determined by NWDAF based on the input information collected from the UE application via DCAF. The UE does not need to determine proximity information related to other UEs.

NOTE 3:	When DCAF is deployed in untrusted domain, NEF is employed to mediate the interactions between NWDAF and DCAF via the Naf_EventExposure_Subscribe service specified in clause 5.2.19.2 of TS 23.502 [3], as described in clause 6.2.8.2.3.

Table 6.19.2-3: Proximity related input data from 5GC/AF

| Information | Source | Description |
| --- | --- | --- |
| UE ID(s) | AMF | SUPI(s) of individual UE(s). |
| UE location | AMF, GMLC | Absolute location of an UE. |
| UE location accuracy | GMLC | Accuracy of the location estimation. |
| Velocity estimate | GMLC | Speed and direction information. |
| Collision risk distance | AF | Derived from the UE connected physical object dimensions e.g. length, width, height. |
| UE heading | AF | UE moving direction. |
| > Absolute heading |  | Heading of the UE movement with respect to the true north. |
| > Relative heading |  | Heading of the UE movement with respect to another UE. |
| UE trajectory | AF | Timestamped UE positions. |
| >UE location |  | Geographical area where the UE is located. |
| >Timestamp |  | Time stamp for the UE location. |

# * * * * End of changes * * * *