3GPP TSG-SA WG2 Meeting #159	 S2-2310664

# Xiamen, CN, Oct 09 – 13, 2023															             (rev of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0055 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction/clarification to AF provisioning service parameters for Ranging/SL Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.3.10 specifies the AF provisioning of mapping between Application Layer ID and GPSI: ### 4.3.10 AF In addition to the functions defined in TS 23.501 [2], the AF supports the following: - provision mapping between Application Layer ID and GPSI to UDR in application data. However, the above functionality of mapping is not reflected in clause 6.2.5 (see below): ### 6.2.5AF-based service parameter provisioning for Ranging/SL Positioning over control plane For Ranging/SL Positioning service parameter provisioning (i.e. creating, updating and deleting), the procedure defined in clause 4.15.6.7 of TS 23.502 [3] is performed with the following considerations: - The AF in TS 23.502 [3] is considered as Ranging/SL Positioning Application Server in this specification. - Service Description indicates Ranging/SL Positioning service domain information. - Service Parameters include parameters for Ranging/SL Positioning service. The detailed information on the parameters is described in clause 5.1.2. Besides, in clause 6.2.5, the reference to clause 5.1.2 (Authorisation and policy/parameter provisioning to NG-RAN) should be corrected to clause 5.1.1.2 (Policy/Parameter provisioned to UE). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description that AF provisions also the mapping between Application Layer ID and GPSI. Correct the reference from 5.1.2 to 5.1.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.2.5	AF-based service parameter provisioning for Ranging/SL Positioning over control plane

For Ranging/SL Positioning service parameter provisioning (i.e. creating, updating and deleting), the procedure defined in clause 4.15.6.7 of TS 23.502 [3] is performed with the following considerations:

-	The AF in TS 23.502 [3] is considered as Ranging/SL Positioning Application Server in this specification.

-	Service Description indicates Ranging/SL Positioning service domain information.

-	Service Parameters include parameters for Ranging/SL Positioning service, which consist of the Policy/Parameters that are to be provisioned to the UE as described in clause 5.1.1.2 and the mapping between Application Layer ID and GPSI.

NOTE:	It is assumed that the Ranging/SL Positioning service domain information is set based on the Service Level Agreement with the operator.

* * * End of Changes * * *