3GPP TSG-SA WG2 Meeting #159	S2-2310830

# Xiamen, China, 9 – 13 October 2023

Title:	Reply LS on 5G ProSe UE-to-UE relay discovery

Response to:	LS on 5G ProSe UE-to-UE relay discovery (C1-236448/S2-2310091)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Fei Lu

Tel. Number:

E-mail Address:	lufei2 at OPPO.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2310158

1. Overall Description:

SA2 would like to thank CT1 for the LS on 5G ProSe UE-to-UE relay discovery (C1-236448/S2-2310091). SA2 has discussed the questions raised by CT1 and provides the answers as following:

Question 1: How can this issue be resolved?

Answer to Question 1: SA2 has agreed to remove the relay_indication from the 5G ProSe UE-to-UE Discovery (see attached CR S2-2310158)

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly?

Answer to Question 2: The procedure defined in clause 6.3.2.4.4 of TS 23.304 is only used if the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay (e.g. during a previous 5G ProSe UE-to-UE Relay Discovery procedure) or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC (see step 4 of clause 6.7.4.2 of TS 23.304 and step 7 of clause 6.7.4.3 of TS 23.304)

Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message?

Answer to Question 3: The 5G ProSe UE-to-UE relay UE is not aware which service the discoverer end UE wants to use. And the 5G ProSe UE-to-UE relay UE will accept the 5G ProSe UE-to-UE Relay Discovery Solicitation message if a 5G ProSe UE-to-UE relay UE matches the User Info ID of a candidate 5G ProSe UE-to-UE Relay received in the 5G ProSe UE-to-UE Relay Discovery Solicitation (see step 4 of clause 6.3.2.4.4 of TS 23.304)

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE in the above procedures?

Answer to Question 4: In the Direct Communication Request message for 5G ProSe UE-to-UE relay communication with integrated discovery, it is possible that the ProSe application layer does not provide the user info of the target 5G ProSe end UE.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above information into account.

3. Date of Next SA2 Meetings:

SA2#160	13th   – 17th November 2023 		Chicago, US