3GPP TSG-WG SA2 Meeting #159	S2-2310909

Xiamen, P.R. China, 09 - 13 October, 2023	(revision of S2-230xxxx)

Title:	[DRAFT] Response LS on EAC Mode Subscription Optimization

Response to:	S2-2310107/C4-233550

Release:	Rel-18

Work Item:	TEI18

Source:	3GPP SA2

To:	3GPP CT4

Cc:

Contact Person:

Name:	Fenqin Zhu

E-mail Address:	zhufenqin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

SA2 would like to thank CT4 for their LS on EAC Mode Subscription Optimization in C4-233550 and would like to provide the following feedback:

Q1: Whether the AMF should be allowed to create and/or modify the EAC mode subscription independently, instead of always relying on a service operation for UE NSAC?

A1: Currently, it is not allowed the AMF to create and/or modify the EAC mode subscription independently, i.e. not rely on the service operation for UE NSAC. From the incoming LS if the AMF want to stop receiving the EAC notification but no UE NSAC for interaction with NSACF, the explicit subscription can help the AMF update the subscription with NSACF directly without waiting the incoming UE NSAC. As an implementation mechanism it is possible that AMF sends a UE NSAC operation for one of the registered UE, i.e. no maximum number update at the NACF, with the updated subscription information. By doing this way it is also possible for AMF to update the subscription directly and not need wait the UE NSAC.

On the other hand, if another new explicit subscription mechanism is introduced at Rel-18, it adds the complexity to the system. For example, due to two mechanisms coexist at the network, whether AMF or NSACF need support two mechanisms, and how the AMF determines the mechanism supported by the peer side NSACF? All of these needs be considered additionally.

Per above consideration, SA2 does not see any benefit to introduce this new mechanism at this stage.

Q2: Will it be considered to have EAC Mode per access type, when per access type NSAC is configured in the NSACF?

A2:  One of the NSAC principle is that the AMF/SMF is not required to be aware the NASC is executed on which access type. The NSAC executed on which access type is only be aware by the NASCF. Per that consideration, SA2 consider that the EAC mode shall be still kept as access type agnostic, i.e. not need introduce the EAC mode per Access Type.

2. Actions:

To CT4:

ACTION: 	SA2 kindly requests CT4 group to take the above responses into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160		November 13 – 17, 2023			Chicago, USA

TSG-SA2 Meeting #160 Ad Hoc	January 22 – 26, 2024			TBD