3GPP TSG-SA2 Meeting #159	S2-2311427

# 9 – 13 October 2023, Xiamen, China										(revision of S2-2310900)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0870 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on obtaining analytics information from MDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been agreed that the NWDAF may collect from MDAS fault prediction analytics which are used by the NWDAF to determine the data quality of Data sources. However, clause 6.2.14.2 which describes the analytics collection from MDAF does not describe the procedure for the NWDAF to collect fault prediction analytics from MDAF. In addition it is possible the MDA request for analytics information to be filtered on per NF type. This procedure is currently missing from clause 6.2.14.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding details in clause 6.2.14.2 on how the NWDAF collects fault prediction analytics from MDAF and how information can be requested on per NF type basis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on how NWDAF obtains analytic information from MDAF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

#### 6.2.14.2	Procedure for analytics collection from MDAF

Figure 6.2.14.2-1: Procedure for analytics collection from MDAF

Precondition: Initially MDAF(s) or MDA MnS producers register their capabilities, i.e. MnS information or MnS profile as described in clause 5 of TS 28.537 [46] to a MnS discovery service producer. The MnS discovery service producer may contain all or partial information related to the capabilities of MDA MnS producer.

1.	An analytics consumer issues a request or subscription towards the selected NWDAF as described in clause 6.1.

2.	NWDAF discovers the MDA Management Function from the MnS discovery service producer by sending a MnS producer discovery service operation. The service operation may include the following parameters:

-	requestedMDAType: indicates a specific MDA capability such as Slice coverage analysis, Mobility performance analysis defined in clause 7.2 of TS 28.104 [45];

-	Area of Interest;

-	Network Slice information (i.e. NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [22]).

3.	The MnS discovery service producer feedback the relevant Mns information of the MDA Management Function.

NOTE 1:	MnS information refer to the information used by the consumer to discover the procedure of specific Management Services and to derive the addresses of the Management Services.

4.	If more than one MDAF information is received, NWDAF selects the highest suitable MDAF and gets the address of the MDAF from Mns information. Then NWDAF sends analytics request to the MDA Management Function by triggering a MDARequest service operation as defined in clause 9.3.2 of TS 28.104 [45]. The service operation may include the following parameters:

-	requestedMDAOutputs: contains one or multiple MDAOutputPerMDAType, each of which consists of mDAType and optional MDAOutputIEFilters. The mDAType indicates type of analytics such as Slice coverage analysis, Mobility performance analysis, fault prediction analysis as defined in TS 28.104 [45].

-	reportingMethod: method for analytics output (it can only be notification-based reporting since file-based, and streaming are not supported in NWDAF).

-	reportingTarget: indicates the reporting target of the MDA outputs (in case the MDA MnS consumer is different from the one that issued the request).

-	analyticsScope: Area of Interest, Network Slice information, NF type information etc.

-	startTime: indicates the start time of the analytics requested by the MnS consumer.

-	stopTime: indicates the stop time of the analytics requested by the MnS consumer.

NOTE 2:	The selection of the most suitable MDAF can be based on MnS information, i.e., MDA type capability, received in combination with other information related to the consumer request, e.g. area of interest and time scheduling.

Editor's note:	Where and how this information coming from the consumer request would be obtained needs to be coordinated with SA WG5.

5.	The MDA Management Function provides the analytics to the NWDAF by triggering an MDAReporting service operation.

6.	NWDAF provides the analytics response back to the analytics consumer after processing the MDA Function analytics together with other data receives from NF sources according to the Analytics ID defined in clause 6.

************************************ END OF CHANGE ****************************************