3GPP TSG-SA WG2#161	S2-2402123

Athens, Greece, 26 February-1 March 2024	(was S2-2400725)

Source:	Qualcomm Incorporated

Title:	23.700-66：KI#2: New solution to support subscription enhancement for NG-RAN node PDU session Energy consumption related information report

Document for:	Discussion/Approval

Agenda Item:	19.4

Work Item / Release:	Discussion/Approval

Abstract of the contribution: This contribution proposed a solution to support subscription enhancement for NG-RAN node PDU session Energy consumption related information report.

# 1.	Discussion

This contribution proposed a solution to address the KI#2 to support the subscription enhancement for QoS flow or PDU session level Energy consumption related information report.

# 2.	Text proposal

It is proposed to agree the following changes to solve KI#2 in TR 23.700-66.

# >>>>BEGINNING OF CHANGES (new solution)<<<<

## 6.x	Solution #X: Subscription enhancement to support PDU session energy consumption information report

### 6.x.1	Key Issue mapping

This is a solution for key issue #2.

### 6.x.2	Functional Description

The NG-RAN energy consumption info configuration is stored as SM related subscription information for specific PDU session in UDM. The NG-RAN energy consumption info configuration includes the following:

- Per PDU session and/or QoS flow level NG-RAN Energy consumption info,

- NG-RAN Energy consumption info, e.g., PDU session level NG-RAN Energy Consumption, QoS flow level NG-RAN Energy Consumption.

- Report time period,

- Periodic reporting,

- Event trigger, etc.

### 6.x.3	Procedures

During PDU session establishment procedure, SMF requests the SM related UE subscription data from UDM, the NG-RAN Energy consumption info configuration is stores as PDU session level SM subscription data sent from UDM to SMF.

SMF may activate the NG-RAN Energy consumption info report procedure for specific PDU session or QoS flow by taking into account the NG-RAN Energy consumption info configuration received from UDM.

### 6.x.4	Impacts on existing services, entities and interfaces

UDM:

- The NG-RAN Energy consumption info configuration is stored in UDM as SM subscription data for specific PDU session.

# >>>>END OF CHANGES<<<<